3GPP TSG-CT WG1 Meeting #157	C1-256032

Sophia-Antipolis, France 13-17 October 2025

Source:	CEWiT

Title:	Pseudo-CR on Abnormal cases in the AIOT device and AIOTF

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Approval

1. Reason for Change

Addition of abnormal case handling in the AIOT device (inventory procedure) and AIOTF (command procedure).

- A command message cannot be sent to a device while inventory procedure is still in progress. Hence it becomes a security imperative for the device to ignore the message.

- If command procedure timer expires at the AIOTF, while waiting for device response, it needs to be treated as a procedure failure.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

*******

* * * First Change * * *

### 5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Failure of authentication.

If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message.

b)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

c)	Collision of AIoT paging and command message.

If a command message is received while an inventory procedure is in progress, the AIoT device shall ignore the command message.

* * * Next Change * * *

### 5.3.5	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

a)	Expiry of timer Tx

In case timer Tx has expired while the AIOTF is waiting for response from the AIoT device, the AIOTF shall abort the procedure and consider it as a procedure failure.

* * * End of Change(s) * * *