3GPP TSG-CT WG1 Meeting #157	C1-256046

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Abnormal case of integrity verification failure for AIoT command

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

For the delayed AIoT command response issue, RAN2 agrees to go with the AS based solution (i.e. AIoT MAC layer will schedule more resource for transmission). However, whether to schedule more resource for transmission also depends on whether the delayed AIoT command response is the only case for "0 SDU". The following is agreement of RAN2#131:

If the security of command fails (e.g. integrity verification failure), the AIoT device should discard the command message. However, if the NAS layer silently discards the command message, it will create another case for "0 SDU". AIoT MAC layer hence is not able to distinguish whether it is a case for delayed AIoT command or security failure. The motivation to distinguish is that for integrity verification failure case it is a waste of resource for scheduling more transmission occasion.

Hence, it is proposed that the AIoT NAS layer can indicate to the lower layer in case of security failure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The READ COMMAND message is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

c)	The integrity verification failure for the READ COMMAND message

The AIoT device shall discard the READ COMMAND message and provide an indication to lower layers.

Editor's note:	It is FFS the command and inventory procedure collision case.

* * * Next Change * * * *

5.3.3.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The WRITE COMMAND message is unsuccessfully implemented (e.g. user memory operation error)

The AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

b)	The integrity verification failure for the WRITE COMMAND message

The AIoT device shall discard the WRITE COMMAND message and provide an indication to lower layers.

* * * Next Change * * * *

5.3.4.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The integrity verification failure for the PERMANENT DISABLE COMMAND message

The AIoT device shall discard the PERMANENT DISABLE COMMAND message and provide an indication to lower layers.

* * * End of Changes * * * *