3GPP TSG-CT WG1 Meeting #157	C1-256048

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Discussion on collision of AIoT procedures

Agenda item:	19.70

Document for:	DISCUSSION

1. Introduction

In R2-2504935 (LS on parallel service request), RAN2 gives agreements on support of parallel service requests for overlapping reader scenario:

Agreements on parallel service request

1.	Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

2.	The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

Besides, SA3 also points out in clause 5.2.2 of TS 33.369 that:

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

Both RAN2 and SA3 have agreed that if a new paging is received as an ongoing procedure in progress, the AIoT device will abort the ongoing procedure and respond to the new paging.

However, in clause 5.2.2 of TS 24.369, CT1 has agreed that if a new paging is received as an ongoing procedure in progress, the AIoT device will proceed with the ongoing procedure and then respond to the new paging later. Therefore, the conclusion of CT1 is completely opposite to that of RAN2/SA3.

### 5.2.5 Abnormal cases in the AIoT device The following abnormal cases can be identified: a) Failure of authentication. If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message. b) Collision of AIoT paging. If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

Therefore, CT1 needs to carefully discuss how to handle the collision cases, including collision between inventory procedures and collision between inventory procedure and command procedure.

2. Discussion

2.1 Collision of inventory procedures

When receiving a new paging while an inventory procedure is ongoing, the AIoT NAS layer can progress the two inventory procedures:

a)	Option-1: process new after ongoing

AIoT device NAS layer buffers the new paging request (i.e. new AIoT identification information), processes the ongoing inventory procedure (e.g. performing matching), and indicates matching result of the ongoing procedure to lower layers. After completion the ongoing inventory procedure, the NAS layer processes the buffered new paging request (i.e. new AIoT identification information) and indicates the matching result of new procedure to lower layers.

b)	Option-2: abort ongoing and process new

AIoT device NAS layer aborts the ongoing inventory procedure, processes the new inventory procedure (e.g. performing matching), and indicates matching result of the new inventory procedure to lower layers.

According to RAN2 agreement "The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.", when receiving a new paging request, the AIoT MAC entity always responds to the new paging. In addition, according to TS 38.391 as quoted below, when responding to a new paging request, AIoT MAC entity will abort the ongoing procedure, release the stored AS ID, and replace the Transaction ID associated with the ongoing procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall: 1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID, if any; 3> store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID; 3> if the Paging ID Presence Indication field indicates Paging ID field is absent: 4> consider the device is selected and indicate it to the upper layers; 3> else: 4> forward the content of the Paging ID field to the upper layers; 4> if the upper layers indicate that the Paging ID is matched: 5> consider the device is selected; 3> if the device is selected: 4> initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

In this case, if the NAS layer still processes the ongoing procedure and indicates the matching result to lower layers (i.e. Option-1), the AIoT device cannot deliver the old inventory report to NG-RAN correctly because the AS ID and transaction ID associated with the old paging have been released. In opposite, AIoT MAC will use the occasion of new paging to transmit the old report. That is, if the NAS layer of AIoT device proceeds with the new paging request (i.e. new AIoT identification information) after completion of the ongoing procedure, AIoT MAC entity cannot support to send the new inventory report because the D2R occasion has been used for the transmission of the old inventory report.

Therefore, Option-1 (process new after ongoing) for NAS layer is actually not feasible under the current RAN2 agreement.

Observation 1: For the collision of inventory procedures, Option-1 (process new after ongoing) is not feasible under current RAN2 agreement.

If NAS layer chooses Option-2, the D2R message (i.e. the INVENTORY REPORT message) of the new procedure can be delivered correctly. However, it may bring the security issues as indicated by SA3.

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

Specifically, it is OK if the paging request from attacker is earlier than the paging request from the network, as the AIoT device will handle the latest inventory. However, if the paging request from attacker is later than the paging request from the network, or if the attacker continuously sends paging requests (the attacker may target either at a single AIoT device or multiple AIoT devices), the AIoT device cannot be discovered by the network because the AIoT device always responds to the latest inventory. However, according to the NOTE from SA3, this denial-of-service attack is not specified and it can depend on implementation-specific method to handle this situation.

Observation 2: For the collision of inventory procedures, Option-2 (abort ongoing and process new) is feasible under current RAN2 agreement.

In summary, considering the conclusions from RAN2 and SA3, and the feasibility of both options without RAN2 impact, we think that going with Option-2 (abort ongoing and process new) is the best choice for CT1.

Proposal 1: Adopt Option-2 (i.e. abort ongoing and process new) for handling the inventory collision case.

2.2 Collision of inventory procedure and command procedure

When receiving a new paging while a command procedure is ongoing, similarly, the AIoT NAS layer can progress the two procedures (new paging and ongoing command) as follows:

- Option-1: process new after ongoing

AIoT device NAS layer buffers the new paging request (i.e. new AIoT identification information), processes the ongoing command procedure (i.e. performing writing, reading or permanent disabling), and delivers NAS message of the ongoing command procedure to lower layers. After completion the ongoing command procedure, the NAS layer processes the buffered new paging request (i.e. new AIoT identification information) and indicates the matching result of new procedure to lower layers.

b)	Option-2: abort ongoing and process new

AIoT device NAS layer aborts the ongoing command procedure, processes the new inventory procedure (e.g. performing matching), and indicates matching result of the new inventory procedure to lower layers.

According to RAN2 agreement “The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.”, when receiving a new paging request, the AIoT MAC entity always start the procedure related to the new paging. In addition, when responding to a new paging request, AIoT MAC will abort the ongoing command procedure, release the stored AS ID, and replace or release the Transaction ID associated with the ongoing command procedure.

In this case, if the AIoT NAS layer still processes the ongoing command procedure and delivers the NAS message to lower layers (i.e. Option-1), the AIoT device cannot deliver the command response to AIOTF correctly because the AS ID and transaction ID associated with the command procedure have been released, and AIoT MAC considers the newly given SDUs is for the new inventory procedure. In addition, even if the NAS layer of AIoT device proceeds with the new paging request (i.e. new AIoT identification information) after completion of the ongoing command procedure, AIoT MAC entity cannot support to send the new inventory report because the D2R occasion has been used for the transmission of the old command NAS message.

Therefore, Option-1 (process new after ongoing) for NAS layer is actually not feasible under current RAN2 agreement.

Observation 3: For the collision of inventory procedure and command procedure, Option-1 (process new after ongoing) is not feasible under current RAN2 agreement.

If the AIoT NAS layer chooses Option-2, the D2R message (i.e. the INVENTORY REPORT message) of the new procedure can be delivered correctly according to RAN2. However, it may bring the following issues:

a)	Device cannot execute command

If an attacker sends a paging request and the request is later than the network command, or if the attacker continuously sends paging requests (the attacker may target either at a single AIoT device or multiple AIoT devices), the AIoT device may not be able to execute a real command from the network because the AIoT device always responds to the latest inventory.

b)	Error from the aborted command

If the aborted command is a write command, handling of a "partially written data" is needed (though it can be treated as an implementation-specific scope). For example, an AIoT device with rollback capability may revert the data. Otherwise, an AIoT device may terminate the write operation immediately. However, if the written data is a temporary ID (see clause 5.4 of TS 33.369), partially writing the temporary ID may result in a security issue.

- How to handle the disable command case

Since the corresponding MAC resources have already been released, it is impossible for the device to send the response of disable command. We are open for CT1 to discuss how to handle this case.

Our understanding is, when the AIOTF sends a disable command, the AIOTF should not send a follow-on inventory. Hence the new inventory is from either an attacker, an unexpected error from AF, or an unwise decision from the AF.

No matter which case, we suggest that, if this case happens, the AIoT device can locally disable the communication capability, and ignore the new received inventory. With that, the AIOTF should not diagnose an error if no disable response is received from the AIoT device.

Observation 4: For the collision of inventory procedure and command procedure, Option-2 (abort ongoing and process new) is feasible under current RAN2 agreement.

Proposal 2: Adopt Option-2 (abort ongoing and process new) for handling the inventory and command collision case.

Proposal 3: For collision between disable command and inventory procedure, the AIoT device can locally disable the communication capability, and ignore the new received inventory.

3 Summary

For collision between inventory procedures:

Observation 1: For the collision of inventory procedures, Option-1 (process new after ongoing) is not feasible under current RAN2 agreement.

Observation 2: For the collision of inventory procedures, Option-2 (abort ongoing and process new) is feasible under current RAN2 agreement.

Proposal 1: Adopt Option-2 (i.e. abort ongoing and process new) for handling the inventory collision case.

A corresponding CR is submitted as C1-256049.

For collision between inventory procedure and command procedure:

Observation 3: For the collision of inventory procedure and command procedure, Option-1 (process new after ongoing) is not feasible under current RAN2 agreement.

Observation 4: For the collision of inventory procedure and command procedure, Option-2 (abort ongoing and process new) is feasible under current RAN2 agreement.

Proposal 2: Adopt Option-2 (abort ongoing and process new) for handling the inventory and command collision case.

Proposal 3: For collision between disable command and inventory procedure, the AIoT device can locally disable the communication capability, and ignore the new received inventory.

A corresponding CR is submitted as C1-256050