3GPP TSG-CT WG1 Meeting #157	C1-256054

# Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Discussion on principles for 6G L3 NAS protocols

Agenda item:	20.2

Document for:	DISCUSSION

1 Introduction

The 3GPP TS 24.007 describes the principles applied by the layer 3 (L3) Non-Access Stratum (NAS) protocols, including principal architecture, sublayers, basic message format, and error handling. It gives a guidance when introducing a specific standardized L3 NAS protocol and around 32 NAS protocols follow its guidance since 2G (see 3GPP TS 24.007).

With the evolution of mobile communication from 5G to 6G, it is also a good opportunity for CT1 to evaluate whether the existing principles applied by the L3 NAS protocols is efficient and sufficient.

2 Discussion

2.1 General

The Pain points learned from 5G NAS protocol are due to the following reasons: architecture related reasons, protocol related reasons, service requirement related reasons, or deployment related reasons. When evaluating the principles applied by the L3 NAS protocols in 6G, it should be considered how to avoid or eliminate the pain points learned from 5G NAS protocol.

Besides, 6G will serve a wide range of devices, from smartphones and AR glasses to robots and IoT sensors. These devices have different capabilities and requirements. Therefore, 6G protocols needs to evolve to support the 6G architecture and new 6G services requirements, including not only enhanced communication services but also beyond communication services. The 6G architecture and new service requirements could bring impact on principles of the L3 NAS protocols.

Hence, to evaluate the principles applied by the L3 NAS protocols, the following factors should be considered:

a)	pain point(s) learned from 5G NAS protocol;

b)	6G architecture; and

c)	new service requirements.

The following sections are some considerations on L3 architecture enhancement for 6G.

2.2 Pain points learned from 5G NAS protocol

2.2.1 IEI space runs out

According to 3GPP TS 24.007, the IEI space of the type 6 IE is 16 (i.e. from 70 to 7F). Table 2.2-1 summarises the IEI space of each IE type.

Table 1. IEI space of each IE type

| IE type | IEI Value Range | Number of IEIs |
| --- | --- | --- |
| Type 1 | 8- to F- | 8 |
| Type 2 | 80 to FF | 128 |
| Type 4 | 00 to 6F | 112 |
| Type 6 | 70 to 7F | 16 |

In R18, CT1 has ran out of type 6 IEIs in REGISTRATION ACCEPT message. After discussion for serval meetings, CT1 finally agreed to introduce a type 6 IE container IE. The type 6 IE container IE introduces an additional IEI and length field, and within the content field it still uses the IEI to identify each type 6 IE. Although this solution cannot fundamentally solve the IEI space runs out issue, it is the best solution under the NAS protocol of 5G for that specific release.

From our perspective, this pain point occurs due to

a)	the inappropriate IE type frequency evaluation;

b)	lots of NAS functionalities (e.g. registration, NAS capability negotiation, and etc.) is carried out via 5GMM protocol and many parameters need to be exchanged via MM procedure.

Hence, an appropriate IEI space of each IE type can be re-evaluated in 6G.

2.2.2 Comprehension required IEI

Some IE may be defined as comprehension required IEs. Generally, the UE ignores the IE with unknown IEI. However, the UE takes actions if it is a unknow comprehension required IE, as specified in 3GPP TS 24.007 and 3GPP TS 24.501.

According to 3GPP TS 24.007, the IEI space of the type 6 IE is 4, and the IEI space for type 4 IE is 16. It is summarized in Table 2.

Table 2. Comprehension required IEI

| IE type | IEI Value Range | Number of IEIs | Number of Comprehension Required IEI |
| --- | --- | --- | --- |
| Type 1 | 8- to F- | 8 | / |
| Type 2 | 80 to FF | 128 | / |
| Type 4 | 00 to 6F | 112 | 16 |
| Type 6 | 70 to 7F | 16 | 4 |

Though the comprehension required IE is defined, this IE is rarely used. What’s more, there is a clean-up for the IEI allocation issue because some specifications wrongly assign a "comprehension required" IEI to a "normal" IE. For example, the IEI of Remote UE context list IE is allocated as "7D" and then be changed to "70" due to "7D" is reserved as a "comprehension required" IEI (see version 17.6.1 and its later version of 3GPP TS 24.501).

Hence, the IEI allocation of each IE type can be re-evaluated in 6G.

2.2.3 Simplified L3 NAS protocol

During the discussion of AmbientIoT-CT (Unique identifier:1080021) in CT1, there were some discussions on whether a simplified L3 NAS protocol can be supported for an AIoT device by:

a)	support of a fixed-structure NAS message. This is similar to the messages defined in OSI 11784/11785. In this case, a NAS message can be a bit sequence with a fixed information component. No error handling or IEI detection is needed then, as the IE order and the content are fixed for a specific message.

b)	support of a dynamic structure of NAS message. Some part of the NAS message header can be used to indicate what is included next, to enable a same message type can apply different structures.

c)	support of a dynamic format of IE. A dynamic format of IE enables a flexible design of IE coding, especially for the IE with unknown length.

d)	simplified the error handlings as defined in 3GPP TS 24.007.

During the discussion of NORDAT_CP (Unique identifier:1060018) in CT1, to improve transmission efficiency of NAS messages for the NB-IoT NTN scenario, the overhead reduction for NAS messages carrying small data has been discussed. This could also affect the general principles of the L3 NAS protocol in 6G era.

From our perspective, this pain point occurs due to:

a)	different types of UEs are supported in the network; and

b)	requirement differs for different types of UEs.

Besides, SA1 requirements on diversity of UE types in TR 22.870 can be found as below:

It is envisioned that a next generation system will continue to support a population of UEs with varying capabilities that would support different use cases. For example, simple UEs with limited capabilities would be supported by the 6G network, alongside more sophisticated UEs that support more advanced features and capabilities.

Hence, different types of UEs should be considered when evaluating the principles applied by the L3 NAS protocols in 6G.

2.3 NAS modularization

Modular NAS (or so-called NAS modularization) is a modularized way of defining NAS protocol, wherein each NAS module is self-contained functional Module designed to support certain NAS functionality or set of NAS functionalities, which could impact the elementary NAS procedure and basic message and IE format.

A discussion on NAS modularization can be found on C1-256025.

Hence, it is worthy to discuss whether the NAS modularization brings impact on the principles of the L3 NAS protocols in 6G.

2.4 NAS distribution

NAS distribution is a distributed way of routing NAS messages. With NAS distribution, the NF protocol (e.g. SM protocol) may be no longer piggybacked over the MM protocol.

The NAS distribution may bring impact on the principles applied by the L3 NAS protocols in 6G. For example, as defined in 3GPP TS 24.007, for 5GS, the Extended Protocol Discriminator (EPD) is a part of the message header and used for the task to pass messages provided by lower (sub-layers) to the appropriate sub-layer entity. The EPD in 5G has two values (i.e. "5GS session management messages" and "5GS mobility management messages"). With NAS distribution, some questions may be raised for CT1, for example:

a)	Does more EPD values are required for 6G?

b)	Does EPD still a part of NAS message for 6G?

Hence, it is worthy to discuss whether the NAS distribution brings impact on the principles applied by the L3 NAS protocols in 6G.

NOTE:	The NAS distribution aspect has dependency on stage-2 requirement. The analysis of NAS distribution should be progressed only after a stable stage-2 requirement.

2.5 New service

New 6G services, such as native artificial intelligence (AI) services, computing services, 6G data framework, may also have impact on principles applied by the L3 NAS protocols.

NOTE:	The new services have dependency on stage-2 requirement. The analysis of new services should be progressed only after a stable stage-2 requirement.

3 Proposal

3.1 Suggested timeline

CT1 can independently discuss and progress the study on the L3 NAS principles for the part without stage-2 dependency. Any part with stage-2 dependency should be progressed only after a stable stage-2 requirement.

3.2 Suggested scope

3GPP TS 24.007 should be used as a start point of this study. With the start point, CT1 can discuss:

a)	Reuse: evaluating which L3 NAS principles can be directly applied to the 6G NAS;

b)	Update: identifying which L3 NAS principles should be updated and applied to the 6G NAS;

c)	New: exploring the new L3 NAS design principles necessary to overcome the 5G NAS protocol pain points or to support the new 6G service for 6G NAS.

With above, the scope of the study includes the following three aspects:

a)	study on the L3 architecture and sublayers of 6G NAS.

b)	study on the basic message format and IE format of 6G NAS message.

c)	study on the error handling of 6G NAS message.