3GPP TSG-CT WG1 Meeting #157	C1-256055

# Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	PLMN Selection for 6G UE

Agenda item:	20.2

Document for:	DISCUSSION

1 Background

As defined in 3GPP TS 23.122, PLMN selection is the key functionality for UE in idle mode which is a necessary prerequisite for terminals to access mobile network services, and the stage 2 work of PLMN selection is under CT1 remit. The PLMN selection mechanism has been continuously enhanced and evolved since 2G, and provides a fundamental support for ensuring users' communication experience.

The PLMN selection for 6G UE is work task for 6G CT1 studies.

2 Discussion

2.1 General

The PLMN selection procedure as defined in 3GPP TS 23.122 should be used as a starting point of the study. In addition to the legacy PLMN selection, CT1 needs to evaluate:

-	Whether there is any pain point for the existing PLMN selection mechanisms

-	Whether new mechanisms can be introduced for the PLMN selection

With above, the following are some considerations on 6G PLMN selection:

-	Pain points of existing PLMN selection

-	Enable AI in PLMN selection

-	Potential impact of 6G new services

2.2 Pain points of existing PLMN selection

2.2.1 Optimal PLMN selection between TN and NTN

Generally, for UE who have subscribed with both Terrestrial Network (TN) and Non-Terrestrial Network (NTN), when UE enters both NTN and TN coverage, UE should choose TN network as much as possible because TN network has better service experiences and cheaper expenses than NTN network. That is:

-	As shown in Figure 1, when a UE enters a TN coverage area from an NTN coverage area, it should search and select TN as soon as possible. Compared to NTN, TN has higher bandwidth, lower transmission latency, and cheaper tariffs. When UE enters TN coverage, it should try to camp on the TN network as soon as possible.

Figure 1: UE enters an area with both TN and NTN coverage

-	As shown in Figure 2, when a UE enter an area with only NTN coverage from a TN coverage area, it should search and select NTN as soon as possible to provide services to the user.

Figure 2: UE enters an area with only NTN coverage

Hence, an optimal PLMN selection with the consideration of NTN impact should be studied in 6G PLMN selection.

2.2.2 Consideration on PLMN with same priority

The home operator may have roaming agreements with multiple operators in the roaming country or region. There may not be a strict priority between the PLMNs of these roaming operators, that is, these PLMNs’ priorities are equal. However, in the existing "Operator Controlled PLMN Selector with Access Technology" list, there is priority between PLMNs.

The following scenario is observed:

-	PLMN1 has higher priority than PLMN2 within "Operator Controlled PLMN Selector with Access Technology"

-	in a roaming country, the UE moves to an area where PLMN1 has only 3G coverage and PLMN2 has 5G coverage

In above case, the UE can only select the 3G PLMN1 and cannot select the 5G PLMN2. However, to ensure a good user experience, the UE is expected to camp on 5G cell of PLMN2.

Hence, PLMN with same priority should be considered in 6G PLMN selection.

2.2.3 Network assisted PLMN selection

Network may be able to provide some information to assist the PLMN selection and avoid unnecessary network search. The following scenario is observed:

a)	Voice service not subscribed

Data card only supports the data service and does not support the voice service. Users can buy the data card through various sales channels from the operator.

According to clause 4.3 of TS 24.501, when the network indicates IMS voice not available during registration procedure, a "voice centric" UE will disable the current network mode capability (such as N1 mode capability) and try another RAT PLMN selection. Since the mobile phones are voice centric, the UE should continue PLMN selection.

It is observed that for both case 1) network does not support IMS voice and case 2) network supports but UE has no subscription, the network indicates the IMS voice as not available. However, for case 2), the UE does not need to periodically attempt to search the PLMNs that support the voice services. It will increase additional power consumption and also affect the data service experience.

b)	USIM arrears

USIM arrears happens when a user fails to pay telephone bills on time. When a UE with a USIM in arrears registers on the PLMNs:

-	Some PLMNs reject the registration request of the UE. This causes frequent PLMN selection in the UE in order to provide services to users as quickly as possible.

-	Some other PLMNs accept the registration request of the UE. Although the registration is accepted, those PLMN cannot provide voice and data service for UE due to the USIM arrears. Moreover, if voice service is unavailable for voice-centric UE, the UE will continue the PLMN selection.

Since UE cannot know whether the USIM is in arrears or not, the UE may make some unnecessary PLMN selections, which increases the UE power consumption.

Hence, it is worthy to explore if the network can provide some assistance for PLMN selection in 6G PLMN selection.

2.3 Enable AI in PLMN selection

With the development of AI, it becomes an interesting topic that whether it is possible to enable AI in PLMN selection to get a better user experience. The following gives an example of consideration:

a)	when a UE enters an elevator, it drops to a lower RAT’s PLMN (e.g. 4G) or loses coverage. When the UE leaves the elevator, it is expected that the UE can quickly return to a higher RAT’s PLMN (e.g. 6G). With the help of AI capability in the UE, it is possible for the UE to identify a best time to perform the PLMN selection and return to a higher RAT’s PLMN.

b)	when a UE enters a garage, it drops to a lower RAT’s PLMN (e.g. 4G) or loses coverage. When the user drives out of the garage, it is expected that the UE can quickly return to a higher RAT’s PLMN (e.g. 6G). With the help of AI capability in the UE, it is possible for the UE to identify a best time to perform the PLMN selection and return to a higher RAT’s PLMN.

Therefore, AI capability in the UE can help optimize the PLMN selection in some specific scenarios (such as elevators, garage, basements, tunnels, subways, high-speed railways, roaming boundaries, etc.) and accelerate the UE to select an optimal PLMN+RAT combination.

On the other hand, it is also a good change to evaluate if the AI can help bring new mechanisms to PLMN selection for better user experience.

2.4 Impact of 6G new services

New 6G services, such as native artificial intelligence (AI) services, computing services, 6G data framework, may bring forward unprecedented demands for the NAS protocols. The new 6G services may not be supported in traditional networks (e.g. 5G, 4G, etc.). When the UE is not camped on the 6G network, the new service cannot be initiated. Hence, based on potential stage 1 requirement, CT1 could discuss the evolution of PLMN selection to better serve the 6G new services.

3 Proposal

3.1 Suggested timeline

CT1 is responsible for stage-2 of PLMN selection. Hence it is suggested that CT1 could start the study of 6G PLMN selection after related SID is approved at CT plenary based on stage 1 requirements.

3.2 Suggested scope

The PLMN selection procedure as defined in 3GPP TS 23.122 can be used as a starting point of the study.

The following impacts and areas of work are expected to be covered in the 6G study of PLMN selection.

CT1:

-	Study on 6G PLMN selection framework

-	Study on potential enhancement of PLMN selection to resolve pain points of existing PLMN selection

-	Study on whether and how to enable AI in PLMN selection

-	Study on potential impact of 6G new services on PLMN selection