3GPP TSG-CT WG1 Meeting #157	C1-256105

Sophia Antipolis, France, 13-17 October 2025

Source:	OPPO

Title:	pCR on Integrity protection for AIoT NAS Command messages

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70 (AmbientIoT-CT)

Document for:	Decision

1. Introduction

This pCR depends on SA3 approval of the CR0021-TS33.369 (S3-253270 or its final revision).

2. Reason for Change

Clause 5.3.2 in 3GPP TS 33.369 [6] specifies the AIoT NAS Command Request and Response messages shall be integrity protected with the MAC (Message Authentication Code). Therefore, AIoT device shall run the Command message and the shared-with-AIOTF secret key through the MAC algorithm to derive the MAC and compare it with the MAC derived by the AIOTF. Same applies to the response to the AIOTF. In other words, AIoT NAS Command Request and Response messages shall contain a new IE, a MAC IE. Annex D.3.1.1 in 3GPP TS 33.369 [9] specifies the integrity algorithm NIA shall be used for computing a 32-bit message authentication code (MAC-I/NAS-MAC), which is a binary string. Therefore, the new MAC IE can be represented with 8 hex numbers, or 8 octets long hex string.

The above requires changs to the following clauses:

- Clause 3.2. Abbreviations. Adding a definition for MAC.

- Clause 4.2.4. Integrity protection of AIoT NAS signalling messages. Reference to TS 33.369.

- Clause 7.1. Message functional definitions and contents. Adding MAC IE to all Command messages.

- Clause 7.2.3. Security parameters. Adding MAC IE definition.

Besides, this pCR addresses also the following issues:

- Clause 7.1. Message functional definitions and contents. To ensure security parameters are always present before other IEs, "SP-" prefix is added to all security parameters in respective message definitions.

- Clause 7.2. Encoding of information elements. As AIoT NAS message format requires security parameters shall immediately follow the message type (Figure 7.1.1-1), clause 7.2.3 (Security parameters) should list all of these. For that purpose, Authentication response parameter (clause 7.2.7) and RAND (clause 7.1.10) are moved under clause 7.2.3. Respective renumbering is applied to clause 7.2.

c.	Clause 7.2.2. Message type. Missing text on spare values is added.

3. Conclusions

This pCR offers two major categories of changes:

- Changes required by SA3 considered integrity protection solution.

- Miscellaneous changes, including the spec restructuring.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

*******

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIoT	Ambient Internet of Things

AIOTF	AIoT Function

MAC	Message Authentication Code

NAS	Non-Access-Stratum

* * * 2nd Change * * * *

### 4.2.4	Integrity protection of AIoT NAS signalling messages

Integrity protection of AIoT NAS signalling messages is specified in clause 5.3.2 of 3GPP TS 33.369 [6].

Editor's note: Integrity protection of AIoT NAS signalling messages is FFS, pending available stage 2 requirements.

* * * 3rd Change * * * *

# 7	Encoding of AIoT NAS protocol

## 7.1	Message functional definitions and contents

### 7.1.1	General

Within the AIoT NAS protocol defined in the present document, every message is a standard L3 message as defined in 3GPP TS 24.007 [3]. This means that the message consists of the following parts:

1)	message type;

2)	zero or more security parameters;

3)	zero or more other information elements, as required.

The organization of an AIoT NAS message is illustrated in the example shown in figure 7.1.1-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message type |  |  |  |  |  |  |  | octet 1 |
| Security parameters |  |  |  |  |  |  |  | octet 2 octet a |
|  |  |  |  |  |  |  |  | octet a+1 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-1: General message organization example for an AIoT NAS message

Editor's note:	Additional content of AIoT NAS general message organization is FFS and can be added as normative requirements are available.

Editor's note:	The position of the Message type in the AIoT message as well as the position of any potential security parameters are FFS.

NOTE:	For clarity of presentation, the names of the security parameter information elements are prefixed with "SP-" string, e.g. SP-RANDAIOT_d in Table 7.1.2.1-1.

### 7.1.2	Inventory report

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | TBD 7.2.3.tbd | M | V | n |
|  | SP-RANDAIOT_d | RAND 7.2.3.3 | M | V | TBD |
|  | SP-RESAIOT | Authentication response parameter 7.2.3.1 | M | V | TBD |
|  | AIoT device identity | AIoT device identity 7.2.6 | M | V | n |

### 7.1.3	Read command

##### 7.1.3.1	Message definition

The READ COMMAND message is sent by the AIOTF to the AIoT device to read the AIoT data from the memory.

See table 7.1.3.1-1.

Message type:	READ COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.3.1-1: READ COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read command message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |
|  | Read offset | Offset 7.2.8 | M | V | 2 |
|  | Read length | AIoT data length 7.2.5 | M | V | TBD |

### 7.1.4	Read complete

#### 7.1.4.1	Message definition

The READ COMPLETE message is sent by the AIoT device to the AIOTF to provide the AIoT data that has been read from the memory.

See table 7.1.4.1-1.

Message type:	READ COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.4.1-1: READ COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read complete message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

### 7.1.5	Read command reject

#### 7.1.5.1	Message definition

The READ COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of reading the AIoT data from the memory.

See table 7.1.5.1-1.

Message type:	READ COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.5.1-1: READ COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |
|  | Read cause | Cause 7.2.7 | M | V | 1 |

### 7.1.6	Write command

##### 7.1.6.1	Message definition

The WRITE COMMAND message is sent by the AIOTF to the AIoT device to write the indicated AIoT data to the memory.

See table 7.1.6.1-1.

Message type:	WRITE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.6.1-1: WRITE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write command message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |
|  | Write offset | Offset 7.2.8 | M | V | 2 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

### 7.1.7	Write complete

#### 7.1.7.1	Message definition

The WRITE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT data has been successfully written to the memory.

See table 7.1.7.1-1.

Message type:	WRITE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.7.1-1: WRITE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write complete message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |

### 7.1.8	Write command reject

#### 7.1.8.1	Message definition

The WRITE COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of writing the AIoT data from the memory.

See table 7.1.8.1-1.

Message type:	WRITE COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.8.1-1: WRITE COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |
|  | Write cause | Cause 7.2.7 | M | V | 1 |

### 7.1.9	Permanent disable command

##### 7.1.9.1	Message definition

The PERMANENT DISABLE COMMAND message is sent by the AIOTF to the AIoT device to permanently disable the communication capability of AIoT device.

See table 7.1.9.1-1.

Message type:	PERMANENT DISABLE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.9.1-1: PERMANENT DISABLE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Permanent disable command message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |

### 7.1.10	Permanent disable complete

#### 7.1.10.1	Message definition

The PERMANENT DISABLE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT device communication capability has been disabled.

See table 7.1.10.1-1.

Message type:	PERMANENT DISABLE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.10.1-1: PERMANENT DISABLE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Permanent disable complete message identity | Message type 7.2.2 | M | V | 1 |
|  | SP-MAC | Message Authentication Code 7.2.3.2 | M | V | 4 |

* * * 4th Change * * * *

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |
| All other values are spare, for future use and if received, such value shall be ignored in this version of the specification. |  |  |  |  |  |  |  |  |  |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

* * * 5th Change * * * *

### 7.2.3	Security parameters

This clause defines security parameters, which, if present shall be encoded in "octet 2" to "octet a", as specified in Figure 7.1.1-1.

Editor's note:	This IE contains the mandatory security parameters in AIoT NAS messages and content is FFS awaiting available normative requirements in SA3. The IE can be renamed and split into multiple individual security IEs to better suit security requirements.

#### 7.2.3.1	Authentication response parameter

The purpose of the authentication response parameter information element is to provide the network with the authentication response calculated in the AIoT device.

The Authentication response parameter information element is coded as shown in figure 7.2.3.1-1 and table 7.2.3.1-1.

The Authentication response parameter is a type 3 information element with TBD octets length. In an AIoT authentication challenge, the response calculated in the AIoT device is TBD octets in length, and is placed in the Authentication response parameter information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication response parameter IEI |  |  |  |  |  |  |  | octet 1 |
| RES value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.3.1-1 Authentication response parameter information element

Table 7.2.3.1-1: Authentication response parameter information element

RES value (octet 2 to TBD)

The RES value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

#### 7.2.3.2	Message Authentication Code

The purpose of the MAC (Message Authentication Code) information element is to provide the AIoT device and the AIOTF with a non-predictable authentication code (MAC-I/NAS-MAC) for checking the message integrity, see Annex D.3.1.1 in 3GPP TS 33.369 [6].

The MAC information element is coded as shown in figure 7.2.3.2-1 and table 7.2.3.2-1.

The MAC is 4 octets long type 3 information element of V format.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAC value |  |  |  |  |  |  |  |  | octet 1 to octet 4 |  |

Figure 7.2.3.2-1: MAC information element

Table 7.2.3.2-1: MAC information element

MAC value (octets 1 to 4):

MAC value is 32-bits long binary string, which represents the message authentication code (MAC-I/NAS-MAC). MAC value is encoded with 4 hexadecimal numbers.

#### 7.2.3.3	RAND

The purpose of the RAND information element is to provide the AIoT device or the AIOTF with a non-predictable number to be used in security calculations.

The RAND information element is coded as shown in figure 7.2.3.3-1 and table 7.2.3.3-1.

The RAND is a type 3 information element with TBD octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAND IEI |  |  |  |  |  |  |  | octet 1 |
| RAND value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.3.3-1 RAND information element

Table 7.2.3.3-1: RAND information element

RAND value (octet 2 to TBD)

The RAND value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

### 7.2.4	AIoT data

The purpose of the AIoT data information element is to carry the AIoT data that has been read from the memory or that is to be written to the memory of the AIoT device.

The AIoT data information element is coded as shown in figure 7.2.4-1 and table 7.2.4-1.

The AIoT data is a type 6 information element with a minimum length of 3 octets and a maximum length of TBD octets.

Editor's note:	the maximum length of the IE contents, when included in a READ COMPLETE message or WRITE COMMAND message, is FFS and subject to RAN2 agreement.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT data IEI |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of AIoT data contents |  |  |  |  |  |  |  |  | octet 2 octet 3 |  |
| AIoT data contents |  |  |  |  |  |  |  |  | octet 4 octet n |  |

Figure 7.2.4-1: AIoT data information element

Table 7.2.4-1: AIoT data information element

AIoT data contents (octets 4 to n):

The AIoT data contents contain the contents of the AIoT data that has been read from the memory or that is to be written in the memory of the AIoT device.

### 7.2.5	AIoT data length

The AIoT data length is a field of TBD octets length that indicates the length of the AIoT data to be read.

### 7.2.6	AIoT device identity

Editor's note:	This IE contains information provided by the AIoT device that can be translated by the network to a globally unique AIoT device permanent identifier. The information and coding of the IE is FFS awaiting available normative specification.

### 7.2.7	Authentication response parameter

The purpose of the authentication response parameter information element is to provide the network with the authentication response calculated in the AIoT device.

The Authentication response parameter information element is coded as shown in figure 7.2.7-1 and table 7.2.7-1.

The Authentication response parameter is a type 3 information element with TBD octets length. In an AIoT authentication challenge, the response calculated in the AIoT device is TBD octets in length, and is placed in the Authentication response parameter information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication response parameter IEI |  |  |  |  |  |  |  | octet 1 |
| RES value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.7-1 Authentication response parameter information element

Table 7.2.7-1: Authentication response parameter information element

RES value (octet 2 to TBD)

The RES value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

### 7.2.7	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.7-1 and table 7.2.7-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.7-1: Cause information element

Table 7.2.7-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| All other values are spare and if received shall be interpreted as "error, unspecified". |  |  |  |  |  |  |  |  |  |

### 7.2.8	Offset

The offset is a field of 2 octets length that indicates the offset where to read the AIoT data from or where to write the AIoT data to.

### 7.2.10	RAND

The purpose of the RAND information element is to provide the AIoT device or the AIOTF with a non-predictable number to be used in security calculations.

The RAND information element is coded as shown in figure 7.2.10-1 and table 7.2.10-1.

The RAND is a type 3 information element with TBD octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAND IEI |  |  |  |  |  |  |  | octet 1 |
| RAND value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.10-1 RAND information element

Table 7.2.10-1: RAND information element

RAND value (octet 2 to TBD)

The RAND value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

* * * End of Changes * * * *