3GPP TSG-CT WG1 Meeting #157	C1-256106

Sophia Antipolis, France, 13-17 October 2025

Source:	OPPO

Title:	Discussion paper on LP-WUSPS assistance information

Spec:	3GPP TS 24.501 v19.4.0

Agenda item:	19.4 (TEI19, NR_LPWUS-Core)

Document for:	Discussion

1. Introduction

RAN2 identified a problem (see R2-2506245) and has sent an LS to CT1 (see C1-256061/R2-2506261), which reads:

1.	RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE.

2.	RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS per UE in IDLE and INACTIVE. Detail signalling is up to SA2 and CT1.

3.	RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1 and RAN3.

4.	RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE.

The wording of the above bullet point (4) maybe a little inaccurate, but RAN2 intention is to say that without amendments to NAS signalling, UE by default will be allowed to use LP-WUS in IDLE and INACTIVE states. One could argue that won't be a complete solution, as the network may not permit UEs to always use the feature. In other words, NAS signalling needs to be amended anyway, as the network shall be able to disable the feature in the UEs when the network decides so.

Hence, CT1 should focus on RAN2 requirements in the above pullet points (1) and (2), as point (3) is primarily under RAN3 remit.

2. Discussion

CT1 should look into few issues, which are described below.

2.1 Impact on Registration Request

Currently, Registration Request delivers 5GMM capability IE to the AMF. With WUSA and LP-WUSPSAI-SI flags, the UE can indicate if (a) WUS assistance information reception supported and if (b) LP-WUSPS assistance information supported or not.

If it is always the network that enabled or disables the LP-WUSPS assistance information provision, then the existing LP-WUSPSAI-SI flag would be sufficient, i.e. there is no need to amend the Registration Request.

If UE is allowed to request or not request the LP-WUSPS assistance information provision, then an explicit indication becomes necessary. Therefore, the Registration Request would need an amendment.

2.2 Impact on Registration Response

With the Registration Response, the AMF should be able to indicate to the UE that the network has decided to either enable or disable LP-WUSPS assistance information provisioning. Currently, however, this is not possible, as the LP-WUSPS assistance information IE encoding has an unjustified limitation – the LP-WUSPS assistance information types are not extendable. Table 9.11.3.111.1 reads "all other values are reserved" for no apparent reason. Therefore, currently it is not possible to meet said RAN2 requirements by making use of the LP-WUSPS assistance information IE. As this is 3GPP Rel-19 IE, CT1 could fix the shortcoming without creating backward incompatibility problem towards the earlier 3GPP releases.

This fix is simple – CT1 should lift the limitation and make the LP-WUSPS assistance information type value field extendable.

2.3 Flexible i.e. future proof protocol solution

Another consideration would be making the protocol more flexible and support the scenario when UE may need to send more than one type of information simultaneously. Such feature may become useful already in 3GPP Rel-19. For example, if the LP-WUS is disabled, and the UE decides to send e.g. LP-WUSPS paging subgroup ID, then the same NAS message needs to carry both LP-WUS enable request and also LP-WUSPS paging subgroup ID. In any case, NAS protocol should be flexible enough to support RAN2 and SA2 features by making backward compatible changes.

3. Proposal

It is proposed to agree CR7014-TS24.501 in C1-256107.