3GPP TSG-CT WG1 Meeting #157	C1-256126

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0471 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optional elements in a response to a request for ad hoc group determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In a SIP 200 (OK) response described in Section 24.2.7.2, the presence of the <preconfigured-group> element and the <comn-participants-criteria> element should be made optional |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MCData client stores the elements if available. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If these elements are not included, what is stated in the specification is not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 24.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 24.2.7.2	Sending a SIP MESSAGE request for ad hoc group determination

Upon receiving a request from the user to determine an ad hoc group with a list of MCData users to be invited or criteria for determining a list of MCData users to be invited, the MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in clause 6.2.4.1;

2)	shall insert in the SIP MESSAGE request:

a)	an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	a <request-type> element set to a value of "ad-hoc-group-determination"; and

ii)	if the MCData client needs to include an active functional alias, an <anyExt> element containing the <functional-alias-URI> element set to the URI of the used functional alias; and

3)	if the MCData user has requested to include either:

a)	the list of MCData users to be invited, shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body with the MCData IDs of the invited MCData users, according to rules and procedures of IETF RFC 5366 [18]; or

b)	the criteria for determining the list of MCData users to be invited, shall insert a <comn-participants-criteria> with one or more criteria as a comma separated list into <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP INVITE request; and

5)	shall send the SIP MESSAGE request towards the participating MCData function according to 3GPP TS 24.229 [5].

On receiving a SIP 200 (OK) response, the MCData client:

1)	shall store:

i)	the ad hoc group identity in the <mcdata-calling-group-id > element;

ii)	the preconfigured MCData group ID in the <preconfigured-group> element in the <anyExt> element, if available; and

iii)	the criteria in the <comn-participants-criteria> element in the <anyExt> element, if available;

in the <mcdata-Params> element in the <mcdatainfo> element in the application/vnd.3gpp.mcdata-info+xml MIME body; and

2)	may notify the user about success in determining an ad hoc group.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the re-SIP INVITE request, the MCPTT client:

1)	may notify the user about failure in determining an ad hoc group with an appropriate response along with the description.

* * * End of Change(s) * * *