3GPP TSG-CT WG1 Meeting #157	C1-256131

# Sophia Antipolis, France, 13-17 October 2025

Title:	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

Response to:	LS (S2-2507636) on issues related to support of IMS voice over NB-IoT NTN connected to EPC from SA2

Release:	Release 20

Work Item:	FS_5GSAT_Ph4_ARC

Source:	CT1

To:	SA2

Cc:	RAN2, SA4, SA3, SA1, RAN1

Contact Person:

Name:	Hui Wang

Tel. Number:	-

E-mail Address:	wanghui.wh@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT1 would like to thank SA2 on their LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC.

Regarding the question directed at CT1:

Question 8 (To CT1): In addition to question 7 related to security, is there any further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN?

Firstly, Rel-19 NORDAT_CP work has made good progress:

- For transport of user data over control plane in IDLE mode, the NAS message overhead has been reduced from 12 bytes to 2 bytes; and

- For transport of user data over control plane in CONNECTED mode, the NAS message overhead has been reduced from 6 bytes to 2 bytes.

CT1 considers the reduced NAS message header is fundamental for transport of user data, there is no way to ensure the user data is correctly routed if the NAS message overhead is further reduced.

Secondly, according to guidelines on Rel-20 work planning in CT WGs (CP-252252), CT1 would like to clarify that FS_5GSAT_Ph4_ARC is a Rel-20 study item in SA2. CT1 will not start any Rel-20 study item Any SID/WID until Q1’26.

Handling of Rel-20 5G Advanced:

  - Work items with dependencies on stage 2 may be created if stage 2 normative work is stable enough in Q1’26 , but no actual CRs can be agreed in Q1’26

Answer: CT1 thinks there is no further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN.

2. Actions:

To SA2 group.

ACTION: CT1 kindly asks SA2 to take the above into account.

3. Date of Next CT1 Meetings:

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2026..................India