3GPP TSG-CT WG1 Meeting #157	C1-256144

# Sophia-Antipolis, France, 13-17 October 2025

Source:	Ericsson

Title:	Discussion on command not supported by the AIoT device

Agenda item:	19.70

Document for:	DISCUSSION

1. Abstract

This document discusses stage-2 requirement for handling of a command not supported by the AIoT device, and proposes a solution.

2. Discussion

2.1 Stage-2

TS 23.369 states "If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.".

Observation-1: Stage-2 requires the AIoT device to inform network when an unsupported command is received.

2.2 Analysis

If an AIoT device does not support a procedure, implementors of the AIoT device will ignore all text of the procedure and coding of NAS messages used in the procedure.

If a message used in a procedure not supported by the AIoT device is received, the AIoT device would not perform any handling specified in the procedure (no matter what the procedure states) and instead would perform TS 24.369 clause 6 "Handling of unknown, unforeseen, and erroneous protocol data" statement "If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.".

As result, according to current version of TS 24.369, if a received message is not supported by the AIoT device, the message is ignored without further actions.

Thus, current stage-3 does not satisfy stage-2 requirement above.

Observation-2: Current stage-3 does not satisfy stage-2 requirement.

For comparison, when the same situation happens in 5GS, the following text of 24.501 applies: 'If UE receives a 5GMM message or 5GSM message with message type not defined for the extended protocol discriminator (EPD) or not implemented by the receiver, it shall return a status message (5GMM STATUS or 5GSM STATUS depending on the EPD) with cause #97 "message type non-existent or not implemented".'

Even though AIoT NAS is different from 5GS NAS, applying the 5GS NAS principle in AIoT NAS for AIoT device would enable satisfying the stage-2 requirement.

Proposal-1: If AIoT device receives unsupported message, AIoT device responds with AIOT STATUS message with cause "message type non-existent or not implemented".

If network sends a command to the AIoT device which does not support the command, and the AIoT device responds with AIOT STATUS message with cause "message type non-existent or not implemented", the procedure fails. The command, AIOT STATUS message, the preceding INVENTORY REPORT message, and paging message (if paging for individual device was used) have wasted radio resources.

Observation-3: Sending a command to the AIoT device which does not support the command, results into waste of radio resource by paging message (if paging for individual device is used), INVENTORY REPORT message, the command, and AIOT STATUS message.

To limit such waste of radio resource to one round of a paging message, an INVENTORY REPORT message, the command and the AIOT STATUS message, if the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", the AIOTF can determine that the AIoT device does not support the command, and refrans from initiating of further command procedures of the same command type towards the AIoT device.

This will ensure that further unsupported command procedures of the same command type are not performed and thus radio resource would not be wasted by them.

Proposal-2: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by further command procedures of the same command type, the AIOTF refrans from initiating of further command procedures of the same command type towards the AIoT device.

In order to avoiding waste of radio resource for command procedures of the other command types, i.e. different than the one for which command procedure fails, the AIoT device can provide its capabilities in the AIOT STATUS message. The AIOTF refrains from initiating of command procedures of the command type not indicated as supported in the AIOT STATUS message, towards the AIoT device.

This will ensure that unsupported command procedures of the command types different than the command type for which command procedure fails, are not performed and thus radio resource would not be wasted by them.

Proposal-3: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by command procedures of the command types other than the command type of the sent command, the AIoT device includes its AIOT capabilities, if any, in the AIOT STATUS message and the AIOTF refrans from initiating of command procedures of command types not supported in the AIOT capabilities, towards the AIoT device.

3. Conclusions

Observation-1: Stage-2 requires the AIoT device to inform network when an unsupported command is received.

Observation-2: Current stage-3 does not satisfy stage-2 requirement.

Proposal-1: If AIoT device receives unsupported message, AIoT device responds with AIOT STATUS message with cause "message type non-existent or not implemented".

Observation-3: Sending a command to the AIoT device which does not support the command, results into waste of radio resource by paging message (if paging for individual device is used), INVENTORY REPORT message, the command, and AIOT STATUS message.

Proposal-2: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by further command procedures of the same command type, the AIOTF refrans from initiating of further command procedures of the same command type towards the AIoT device.

Proposal-3: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by command procedures of the command types other than the command type of the sent command, the AIoT device includes its AIOT capabilities, if any, in the AIOT STATUS message and the AIOTF refrans from initiating of command procedures of command types not supported in the AIOT capabilities, towards the AIoT device.

4. Proposal

It is proposed to discuss the above.