3GPP TSG-CT WG1 Meeting #157	C1-256146

# Sophia-Antipolis, France, 13-17 October 2025

Source:	Ericsson

Title:	Discussion on 6GS NAS study work structure in CT1

Agenda item:	19.70

Document for:	DISCUSSION

1. Abstract

This document discusses how to structure 6GS NAS study work in CT1 and proposes a solution.

2. Discussion

2.1 CT1 responsibility

Based on current CT1 terms of references (CP-203231), CT1 is responsible for stage-3 of:

-	Mobility Management, Call Control, Session Management and Location services Layer 3 signalling between the User Equipment (UE) and the Core Network (CN);

and CT1 is also responsible for stage-2 of:

-	public warning systems (PWS);

-	point-to-point short message service (SMS); and

-	NAS functions related to the MS in idle mode. This includes PLMN selection, access control and steering-of-roaming.

Observation-1: CT1 is responsible for stage-3 aspects and some stage-2 aspects.

2.2 Constraints on CT1 work on 6GS

At beginning of Rel-20, SA2 and SA3 are unlikely to conclude their own stage-2 studies on 6GS. Therefore, CT1 will have no stage-2 for which CT1 could study 6GS NAS stage-3.

Thus, at beginning of Rel-20, CT1 will be able to study:

1)	pure protocol issues. This study would be limited to aspects which do not require stage-2 requirements. For 6GS NAS, given that 2G/3G/EPS/5GS NAS protocols are deployed and are working well, we should not deviate from existing principles without clear and agreed reasons. However, we should learn from past - i.e. we should identify any issues in 5GS NAS and study solutions for those issues, to be applied in 6GS NAS.

2)	stage-2 areas in CT1's responsibility, i.e.:

-	public warning systems (PWS);

-	point-to-point short message service (SMS); and

-	NAS functions related to the MS in idle mode. This includes PLMN selection, access control and steering-of-roaming.

Study on those stage-2 aspects are unrelated to SA2's studies, except for SMS where SA2 terms of references include end-to-end procedures for 3GPP services including SMS. Thus, for SMS, we should limit only to aspects related to SM-RL and SM-TL layers, as in 23.040.

Once SA2's conclusions are available, there will be some stage-2 outputs for which to study stage-3 solutions for 6GS NAS, and thus CT1 will be able to start studying stage-3 for 6GS NAS for SA2's conclusions.

NOTE: Agreed SA2's key issues, architectural requirements, or architectural assumptions are not sufficient stage-2 for which stage-3 solutions for 6GS NAS can be studied. SA2's key issues does not provide sufficient stage-2 details for stage-3 work, and SA2's architectural requirements and architectural assumptions are not stable until SA2 concludes its study.

Observation-2: Studies unrelated to stage-2 owned by SA2 can start immediately.

Observation-3: Stage-3 studies for 6GS NAS related to stage-2 in SA2 cannot start until SA2's study concludes.

CT plenary approved CP-251297 stating "Each CT WG can have multiple 6G SIDs, each one dedicated to a specific topic within the WG’s remit." and CP-252253 stating "CT WGs should limit the number of 6G SIDs per WG to not more than 5 SIDs for Rel-20.". This gives CT1 a space to split the 6GS study to several SIDs.

Observation-4: CT plenary enabled CT1 to split 6GS studies to several SIDs.

Study on pure protocol issues and study on stage-2 areas in CT1's responsibility can start immediately. If study on pure protocol issues and study on stage-2 areas in CT1's responsibility are split to two SIDs, this enables tracking of completion rate of each of SIDs. This is particularly important for stage-2 areas in CT1's responsibility, to be able to identify completion rate of entire stage-2 for 6GS, in all WGs.

Observation-5: It would be useful to split 6GS study to several SIDs, since stage-3 studies related to stage-2 in SA2 starts later than study on pure protocol issues and study on stage-2 areas in CT1's responsibility, and to enable tracking of completion rate of each of the aspects.

3. Proposal

It is proposed to structure CT1's work on 6GS as follows:

SID-1: one SID for study on identifying limitations of 5G NAS protocol and solutions for avoiding them for 6G. This study can start immediately.

SID-2: one SID for study on architecture in 6G for aspects with stage-2 in CT1's remit. This study can start immediately.

SID-3: one SID for study on 6GS NAS protocol for stage-2 in SA2 and SA3. This study can be agreed only once conclusions of SA2 study are available.