3GPP TSG-CT WG1 Meeting #157	C1-256150

# Sophia-Antipolis, France, 13-17 October 2025

# Source:	Ericsson

# Title:	New SID on identifying 5G NAS protocol limitations and solutions for avoiding them for 6G

# Document for:	Approval

# Agenda Item:	19.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on identifying 5G NAS protocol limitations and solutions for avoiding them for 6G

Acronym:	FS_6G_5NPLA

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No |  |  | x |  | x |
| Don't know | x |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1050110 | Study on 6G Use Cases and Service Requirements | Study of 6GS stage-1 requirements |
| 1080057 | Study on Architecture for 6G System | Study of 6GS stage-2 requirements in SA2's remit |
| 750025 | CT aspects of 5G System - Phase 1 | 5GS NAS |

Dependency on non-3GPP (draft) specification:

none

# 3	Justification

Between Rel-15 and Rel-19, 5GS NAS has significantly evolved. As new features are added, limitation of 5GS NAS protocol has became visible. The following limitations of 5GS NAS protocol were identified:

1)	limitations related to message coding:

-	IEIs for type 6 IEs without "comprehension required" were exhausted for some NAS messages (e.g. REGISTRATION ACCEPT).

-	IEIs for type 1 IEs are not far from being exhausted in CONFIGURATION UPDATE COMMAND (7 values out of 8 available values used), REGISTRATION REQUEST (5 values out of 8 available values assigned), and REGISTRATION ACCEPT (5 values out of 8 available values assigned).

2)	limitations related to IE coding:

-	values of the type of identity field of the 5GS mobile identity IE were exhausted. Similarly, the type of identity field of the 5GS identity type IE is nearly exhausted (7 values out of 8 available values assigned).

-	values of the payload container type field of the Payload container type IE are not far from being exhausted (13 values out of 16 available values assigned).

-	values of the request type field of the Request type IE are not far from being exhausted (6 values out of 8 available values assigned).

3)	limitations related to inconsistent design for exchange for information, when the information is expected to be generated or consumed by entity other than the MM management entity (i.e. AMF) and where the MM management entity transparent passes the information

-	for SMS, UPDS, LCS, ... a protocol is defined between the UE and the endpoint.

-	for CP-SOR and UPU, solely an IE is passed between the UE and the endpoint.

4)	limitations related to missing definitions

-	24.501 expects the UE to performs some actions when served by "satellite NG-RAN access technology" but does not specify how the UE determines an NG-RAN cell is of "satellite NG-RAN access technology" or not.

5)	limitations due to conflicts of some procedure collisions are not handled consistently

-	when UE-requested PDU session modification procedure collides with network-requested PDU session modification procedure, then network ignores PDU SESSION MODIFICATION REQUEST except the URSP rule enforcement reports IE and proceeds with the network-requested PDU session modification procedure.

6)	coordination between 5GMM and 5GSM protocols in TS 24.501 subclause 4.2, abnormal case for PDU session establishment in TS 24.501 subclause 6.4.1.6 h) and abnormal case of UL NAS TRANSPORT message handling in TS 24.501 subclause 5.4.5.2.5 a) 12), prevent completion of PDU sessione establishment procedure when the UE is moved from 5GMM-CONNECTED to 5GMM-IDLE while PDU session establishment procedure is in progress.

Further limitation of 5GS NAS protocol can be identified.

The limitation of 5GS NAS protocol should be avoided in 6G.

# 4	Objective

Objective of this SID is to:

1)	identify limitations of 5GS NAS protocol; and

2)	identify solutions which can be used to avoid those limitations in 6G.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.8xx | Study on identifying 5G NAS protocol limitations and solutions for avoiding them for 6G | CT#113 (Sep 2026) | CT#113 (Sep 2026) | Sedlacek, Ivo, Ericsson, ivo <dot> sedlacek <at> ericsson <dot> com |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Sedlacek, Ivo, Ericsson, ivo <dot> sedlacek <at> ericsson <dot> com

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

Potentially, SA3 for security issues.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Verizon