3GPP TSG-CT WG1 Meeting #157	C1-256165

# Sophia Antipolis, France 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0107 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Editor's note to align with stage 2 on changing XR to multi-modal. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are editor's notes on the alignment with stage 2 on changing XR to multi-modal is FFS. This needs to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The editor's notes on the alignment with stage 2 on changing XR to multi-modal is resolved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misallignment with stage 2 and the editor's notes will stay un-resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.24.3, 7.2.24.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 7.2.24.3	SDDM client CoAP procedure

In order to request a SEALDD multi-modal transmission connection establishment to the SDDM-S, the SDDM-C shall generate a CoAP POST request to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.4.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req"; and

c)	shall include a "XREstablishmentRequest" object:

1)	shall include a "sealClientId" attribute set of the identity of the SDDM-C;

2)	shall include a "sealddFlowsInfos" attribute set to the list of the SDDM flow used by the SDDM-C and SDDM-S;

i)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flows used by the SDDM-C and SDDM-S to identify the application traffic;

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

3)	may include a "valTgtUe" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

4)	may include a "serverId" attribute set to the information of the VAL server; and

5)	may include a "valServiceId" attribute set to the identity of the VAL service of the vertical application.

The SDDM-C shall send the CoAP POST request to the SDDM-S protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

# *** Next Change ***

#### 7.2.24.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.4.4.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req"; and

b)	a "XREstablishmentRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-res"; and

b)	shall attempt to create the SDD multi-modal transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include a "XREstablishmentResponse" object in the CoAP POST 2.01 (Created) response message:

i)	shall include a "result" attribute set to "success";

ii)	shall include a "mmSealddFlow" attribute set to the information of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal service traffic;

A)	 shall include a "sealddFlowsInfos" attribute set to the list of SDDM flows used by the SDDM-C and SDDM-S;

shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

may include a "portNumber" attribute specifying the identity of the port number of the traffic;

may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

iii)	may include a "headerExtType" attribute specifying the header extension information;

iv)	may include a "headerExtId" attribute specifying the header extension identity;

v)	may include a "packetizationIndication" attribute specifying the packet indication information;

vi)	may include a "payloadType" attribute specifying the payload type information; and

vii)	may include a "payloadFormat" attribute specifying the payload format information; and

2)	otherwise, shall include a "XR EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response as specified in clause A.4.4.2.2.3.1.

The SDDM-S shall send the CoAP POST response towards the SDDM-C.

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	The definition of multimodalSealddFlowId is FFS.

# *** End of Changes ***