3GPP TSG-CT WG1 Meeting #157	C1-256171

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4538 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of S&F wait time duration in control plane CIoT EPS optimization with overhead reduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH, NORDAT_CP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes to correct the handling of S&F wait time duration in the UE in control plane CIoT EPS optimization with overhead reduction, in order to align with the handling of S&F wait time duration in the UE in the attach procedure, service request procedure and tracking area update procedure. 1.The following text was introduced by C1-255617 to TS 24.301, which was agreed for regular Service Request procedure. ### ##### 5.6.1.4.2 UE is using EPS services with control plane CIoT EPS optimization … If the UE supporting S&F satellite operation receives the S&F satellite operation parameters IE in the SERVICE ACCEPT message: a) if the S&F wait timer duration field is present in the S&F satellite operation parameters IE and its value does not indicate zero, the UE shall stop timer T3451, if running and start timer T3451 with the S&F wait timer duration value provided in the S&F satellite operation parameters IE. ### The UE can start the T3451 with the value of the S&F wait time duration received from the network in control plane CIoT EPS optimization with overhead reduction, however, the S&F wait time duration should not indicate zero. It is therefore proposed to correct the handling of S&F wait time duration in control plane CIoT EPS optimization with overhead reduction for alignment. 2. S&F satellite operation parameters IE is mentioned both in bullet c) and bullet d). It’s proposed to delete S&F satellite operation parameters IE in bullet c). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.The UE can start the T3451 with the value of the S&F wait time duration received from the network in control plane CIoT EPS optimization with overhead reduction, however, the S&F wait timer duration should not indicate zero. 2.It’s proposed to delete S&F satellite operation parameters IE in bullet c). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of the S&F wait time duration received from the network in control plane CIoT EPS optimization with overhead reduction, is not aligned with the handling of S&F wait time duration in the UE in the attach procedure, service request procedure and tracking area update procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 5.6.1.4.3	UE is using EPS services with control plane CIoT EPS optimization with overhead reduction

For case a, b and c in clause 5.6.1.1, upon receipt of the EMM TRANSPORT message, after completion of the EMM common procedures according to clause 5.6.1.3, if any, if:

a)	the MME needs to perform an EPS bearer context status synchronization:

1)	for an EPS bearer context associated with Control plane only indication; or

2)	for an EPS bearer context not associated with Control plane only indication, and there is no downlink user data pending to be delivered via the user plane;

b)	the control plane data back-off time for the UE is stored in MME and the MME decides to deactivate congestion control for transport of user data via the control plane;

c)	the MME needs to provide the UE with Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" IE or Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" IE; or

d)	the MME needs to provide the UE with S&F satellite operation parameters IE;

then the MME shall send a SERVICE ACCEPT message.

Furthermore, the MME may:

a)	initiate the setup of the user plane radio bearer(s), if downlink user data is pending to be delivered via the user plane;

b)	send an EMM TRANSPORT message to the UE, if downlink user data is pending to be delivered via the control plane;

c)	send a NAS signalling message not related to an EMM common procedure to the UE, if downlink signalling is pending;

d)	initiate release of the NAS signalling connection if user data is included in the Data container IE and the DDX field indicates that no further uplink and no further downlink data transmission subsequent to the uplink data transmission is expected, unless the MME has additional downlink user data or signalling pending; or

e)	send a SERVICE ACCEPT message to complete the service request procedure, if no NAS security mode control procedure was initiated, the MME did not send a SERVICE ACCEPT message as specified above to perform an EPS bearer context status synchronization, and the MME did not initiate any of the procedures specified in item a to e above.

NOTE 1:	The MME can initiate the setup of the user plane radio bearer(s) if the MME decides to activate the congestion control for transport of user data via the control plane.

The MME shall delete any stored paging restriction for the UE and stop restricting paging.

In NB-S1 mode, if the MME needs to initiate the setup of user plane radio bearer(s), the MME shall check if the UE can support the establishment of additional user plane radio bearer based on the multiple DRB support indicated by UE in the UE network capability IE.

If the MME needs to initiate an EPS bearer context status synchronization, the MME shall include an EPS bearer context status IE in the SERVICE ACCEPT message.

If the MME sends a SERVICE ACCEPT message upon receipt of the EMM TRANSPORT message, and:

a)	the DDX field of the Data container IE is set to "No further uplink and no further downlink data transmission subsequent to the uplink data transmission is expected";

b)	the UE has indicated support for the control plane data back-off timer; and

c)	the MME decides to activate the congestion control for transport of user data via the control plane;

then the MME shall include the T3448 value IE in the SERVICE ACCEPT message.

If the MME sends a SERVICE ACCEPT message and decides to deactivate congestion control for transport of user data via the control plane then the MME shall delete the stored control plane data back-off time for the UE and the MME shall not include timer T3448 value IE in SERVICE ACCEPT message.

If the UE supports S&F satellite operation and the MME is operating in S&F mode, the MME may include the S&F satellite operation parameters IE in the SERVICE ACCEPT message to provide:

a)	a S&F wait time duration;

b)	an estimated uplink delivery time duration;

c)	for S&F monitoring list:

1)	a S&F monitoring list; or

2)	an indication to delete any previously received S&F monitoring list; or

d)	any combination of list items a, b and c.

If the MME needs to initiate an EPS bearer context status synchronization, the MME shall consider the service request procedure successfully completed when it sends the SERVICE ACCEPT message. If the MME does not need to initiate an EPS bearer context status synchronization, the MME shall consider the service request procedure successfully completed in the following cases:

a)	when it successfully completes a NAS security mode control procedure;

b)	when it receives an indication from the lower layer that the user plane is setup, if radio bearer establishment is required;

c)	upon receipt of the EMM TRANSPORT message and completion of the EMM common procedures, if any, if the EMM TRANSPORT message was successfully integrity checked and the Data container IE in the EMM TRANSPORT message, if applicable, was successfully deciphered, radio bearer establishment is not required, and the MME has downlink user data or signalling not related to an EMM common procedure pending; and

d)	with the transmission of a SERVICE ACCEPT message or with the decision to initiate release of the NAS signalling connection, if the EMM TRANSPORT message was successfully integrity checked and the Data container IE in the EMM TRANSPORT message, if applicable, was successfully deciphered, radio bearer establishment is not required, and the MME does not have any downlink user data or signalling pending.

If the MME considers the service request procedure successfully completed the MME shall:

a)	if user data is included in the Data container IE, forward the user data and the EPS bearer identity of the Data container IE, if any, to the ESM layer;

b)	if SMS message is included in the Data container IE, forward the SMS message of the Data container IE to upper layers; or

c)	if LCS message is included in the Data container IE, forward the LCS message and additional information, if any, of the Data container IE to upper layers.

The UE shall treat the receipt of any of the following as successful completion of the procedure:

a)	a SECURITY MODE COMMAND message;

b)	a security protected EMM message different from a SERVICE REJECT message and not related to an EMM common procedure;

c)	a security protected ESM message; and

d)	receipt of the indication from the lower layers that the user plane radio bearers are set up.

For case b in clause 5.6.1.1, the UE shall also treat the indication from the lower layers that the RRC connection has been released as successful completion of the procedure.

For cases a and c in clause 5.6.1.1, the UE shall treat the indication from the lower layers that the RRC connection has been released as an abnormal case and shall follow the procedure described in clause 5.6.1.6, item b.

For case a, b and c in clause 5.6.1.1, upon successful completion of the procedure, the UE shall reset the service request attempt counter, stop the timer T3417 and enter the state EMM-REGISTERED.

If the MME needs to initiate an EPS bearer context status synchronization or to provide the UE with Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" IE or Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" or the MME needs to provide the UE with S&F satellite operation parameters IE, the UE can receive a SERVICE ACCEPT message even after it received a SECURITY MODE COMMAND message or an indication from the lower layers that the user plane radio bearers are set up and determined successful completion of the service request procedure. Upon receipt of the SECURITY MODE COMMAND message or an indication from the lower layers that the user plane radio bearers are set up, the UE shall start timer T3449. If the UE receives a security protected ESM message or a security protected EMM message not related to an EMM common procedure, the UE shall stop the timer T3449. If the UE receives a SERVICE ACCEPT message while the timer T3449 is running, the UE shall treat the SERVICE ACCEPT message and stop the timer T3449. If the UE is not in state EMM-SERVICE-REQUEST-INITIATED and timer T3449 is not running, the receipt of the SERVICE ACCEPT message is considered as protocol error and the UE shall return EMM STATUS message as specified in clause 7.4; otherwise, the UE shall treat the SERVICE ACCEPT message.

If the UE treats the SERVICE ACCEPT message and an EPS bearer context status IE is included in the message, the UE shall deactivate all those EPS bearers contexts locally (without peer-to-peer signalling between the UE and the MME) which are active in the UE but are indicated by the MME as being inactive. If a default EPS bearer context is marked as inactive in the EPS bearer context status IE included in the SERVICE ACCEPT message, and this default bearer is not associated with the last remaining PDN connection in the UE, the UE shall locally deactivate all EPS bearer contexts associated to the PDN connection with the default EPS bearer context without peer-to-peer ESM signalling to the MME. If the default bearer is associated with the last remaining PDN connection of the UE in the MME, and EMM-REGISTERED without PDN connection is supported by the UE and the MME, the UE shall locally deactivate all EPS bearer contexts associated to the PDN connection with the default EPS bearer context without peer-to-peer ESM signalling to the MME.

If the T3448 value IE is present in the received SERVICE ACCEPT message, the UE shall:

a)	stop timer T3448 if it is running;

b)	consider the transport of user data via the control plane as successful; and

c)	start timer T3448 with the value provided in the T3448 value IE.

If the T3448 value IE is present in the SERVICE ACCEPT message and the value indicates that this timer is either zero or deactivated, the UE shall consider this case as an abnormal case and proceed as if the T3448 value IE is not present.

If the UE in EMM-IDLE mode initiated the service request procedure by sending a EMM TRANSPORT message, the SERVICE ACCEPT message does not include the T3448 value IE, and timer T3448 is running, then the UE shall stop timer T3448.

If the MME received the list of TAIs from the satellite E-UTRAN as described in 3GPP TS 23.401 [10], and determines that, by UE subscription and operator's preference, any but not all TAIs in the received list of TAIs is forbidden for roaming or for regional provision of service, the MME shall include the TAI(s) in:

a)	the Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" IE;

b)	the Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" IE; or

c)	both,

in the SERVICE ACCEPT message.

NOTE 2:	"Forbidden tracking areas for roaming" corresponds to cause values #13 and #15, and "forbidden tracking areas for regional provision of service" corresponds cause value #12.

If the UE receives the Forbidden TAI(s) for the list of "forbidden tracking areas for roaming" IE in the SERVICE ACCEPT message and the TAI(s) included in the IE which are belonging to the serving PLMN or equivalent PLMN(s) is not part of the list of "forbidden tracking areas for roaming", the UE shall store the TAI(s) included in the IE into the list of "forbidden tracking areas for roaming" and ignore the TAI(s) which do not belong to the serving PLMN or equivalent PLMN(s).

If the UE receives the Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" IE in the SERVICE ACCEPT message and the TAI(s) included in the IE which are belonging to the serving PLMN or equivalent PLMN(s) is not part of the list of "forbidden tracking areas for regional provision of service", the UE shall store the TAI(s) included in the IE into the list of "forbidden tracking areas for regional provision of service" and ignore the TAI(s) which do not belong to the serving PLMN or equivalent PLMN(s).

If the UE supporting S&F satellite operation receives the S&F satellite operation parameters IE in the SERVICE ACCEPT message:

a)	if the S&F wait timer duration field is present in the S&F satellite operation parameters IE and its value does not indicate zero, the UE shall stop timer T3451, if running and start timer T3451 with the S&F wait timer duration value provided in the S&F satellite operation parameters IE.

While timer T3451 is running, the UE shall not initiate NAS procedures over S&F satellite E-UTRA cells in the TAI list, unless the S&F satellite E-UTRA cell broadcasts the same satellite ID broadcasted by the S&F satellite E-UTRA cell from which the SERVICE ACCEPT message was received;

b)	if the Estimated S&F uplink delivery time duration field is present in the S&F satellite operation parameters IE, the UE may store the estimated S&F uplink delivery time duration provided in the S&F satellite operation parameters IE and shall erase any previous stored value;

c)	if the SFMLI field in the S&F satellite operation parameters IE is set to "S&F monitoring list present", the UE shall delete any previously stored value for the S&F monitoring list of the current PLMN and the UE may store the S&F monitoring list value for the current PLMN, provided in the S&F satellite operation parameters IE; and

d)	if the SFMLI field in the S&F satellite operation parameters IE is set to "S&F monitoring list not present and delete any previously received S&F monitoring list", the UE shall delete any previously received S&F monitoring list value stored for the current PLMN.

NOTE 3:	How the UE uses the estimated S&F uplink delivery time duration, and the S&F monitoring list is left for UE implementation.

NOTE 4:	The estimated S&F uplink delivery time duration is associated only with the satellite ID of the satellite which provides the estimated S&F uplink delivery time duration.

***** End of change *****