3GPP TSG-CT WG1 Meeting #157	C1-256202

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0843 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signal Strength Measurement using the Multi-hop L3 UE-to-UE Relay Discovery Procedure for IP Data Unit type with Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the multi-hop L3 U2U relay discovery procedure for IP data unit type with Model B, it is specified in 8c.2.1.2.3.2 that a ‘discoverer end UE’ may use the procedure to measure the signal strength of the discovery message between the UEs. Here the ‘discoverer end UE’ can be either the 5G ProSe multi-hop layer-3 end UE or the 5G ProSe multi-hop layer-3 UE-to-UE relay UE, according to 8c.2.1.2.3.1. However, in the multi-hop L3 U2U relay discovery procedure for IP data unit type with Model A (8c.2.1.2.2.3), the potential use of signal strength measurement of the discovery message is only specified for a 5G ProSe multi-hop end UE, and not specifed for a 5G ProSe multi-hop layer-3 UE-to-UE relay UE. Thus the poroposed change for consistency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the NOTE in the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery initiation for IP data unit type with Model A, so that 5G ProSe multi-hop layer-3 UE-to-UE relay may use the procedure to measure the signal strength of the discovery message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Behavior not specified for the 5G ProSe multi-hop layer-3 UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8c.2.1.2.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

8c.2.1.2.2.3.2	Monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery initiation for IP data unit type

The UE is authorised to perform the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery using monitoring when the UE is not served by NG-RAN and is configured with the radio parameters to be used for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery for IP data unit type when not served by NG-RAN;

2)	the UE is served by NG-RAN and is authorised to perform 5G ProSe layer-3 UE-to-UE relay discovery monitoring in at least one PLMN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14] and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe multi-hop layer-3 UE-to-UE relay discovery using monitoring when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery when not served by NG-RAN; and

b)	the UE is configured with the relay service code parameter identifying the connectivity service to be monitored, as specified in clause 5.2.9;

otherwise, the UE is not authorised to perform the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.3.2.1 illustrates the interaction of the UEs in the monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type.

Figure 8c.2.1.2.2.3.2.1: Monitoring UE procedure for multi-hop layer-3 UE-to-UE relay discovery for IP data unit type

When the UE, acting as a 5G ProSe multi-hop layer-3 UE-to-UE relay or as a 5G ProSe multi-hop end UE, is triggered by the upper layers to monitor the proximity of a connectivity service provided by another 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type, and if the UE is authorised to perform the monitoring UE procedure for multi-hop UE-to-UE relay discovery for IP data unit type, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages with the default destination layer-2 ID as specified in clause 5.2.9.

Upon the reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type, for the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall obtain a valid UTC time from the lower layers and generate a UTC-based counter corresponding to this UTC time. The UE shall use the associated DUSK, if received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to unscramble the PROSE PC5 DISCOVERY message as described in clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used), the UE shall use the DUCK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to decrypt the configured message-specific confidentiality-protected portion, as described in clause 6.1.3.5.3.1 of 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF (if security procedure over control plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used) or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop layer-3 UE-to-UE relay for IP data unit type is used), the UE shall use the DUIK and the UTC-based counter associated with the PROSE PC5 DISCOVERY message to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for UE-to-UE relay discovery announcement.

Then if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type is the same as the relay service code parameter configured as specified in clause 5 for the connectivity service being monitored, the UE shall consider that the connectivity service the UE seeks to monitor has been discovered.

NOTE: 	This procedure may be used by a 5G ProSe multi-hop end UE or a 5G ProSe multi-hop layer-3 UE-to-UE relay to measure the signal strength of the PROSE PC5 DISCOVERY message from a 5G ProSe multi-hop layer-3 UE-to-UE relay. The measurement may be considered for 5G ProSe multi-hop layer-3 UE-to-UE relay selection/reselection.

* * * End of Changes * * * *