3GPP TSG-CT WG1 Meeting #157	C1-256224

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7021 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice value setting in EHPLMN case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetShare |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in S2-2508017 (23.501 CR#6351) and captured in latest version of TS 23.501: “NOTE 1: In the case of Indirect Network Sharing, to support EHPLMN ID broadcast option, as deployment requirement for the participating operator, the S-NSSAI value supported by the EHPLMN is the same as the HPLMN S-NSSAI value. The serving AMF sets the value(s) of the HPLMN S-NSSAI(s) to the value(s) of the EHPLMN S-NSSAI(s) when it determines the value(s) of S-NSSAI(s) of the hosting operator.” However, existing description in TS 24.501 clause 4.29 is not aligned with above stage 2 requirement: “If the selected PLMN ID is the PLMN ID of an EHPLMN of the UE whose PLMN code is not derived from the IMSI, the AMF shall set value(s) of the S-NSSAI(s) of the serving EHPLMN to value(s) of the mapped S-NSSAI(s) and shall only use the corresponding S-NSSAI(s) of the serving EHPLMN, i.e. S-NSSAI(s) of participating operator without mapped S-NSSAI(s) towards the UE”. In case of indirect network sharing, for EHPLMN case whose PLMN code is not derived from IMSI, the slice value sent by UE to AMF and sent by AMF to UE is EHPLMN S-NSSAI without mapped S-NSSAI. After receiving EHPLMN S-NSSAI from UE: - the serving AMF shall set value of mapped S-NSSAI to value of EHPLMN S-NSSAI as received from UE; and - the serving AMF uses the locally determined value of mapped S-NSSAI to determine the value of corresponding S-NSSAI of hosting operator (e.g. by querying NSSF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align with stage 2 requirement that in case of indirect network sharing, for EHPLMN case whose PLMN code is not derived from IMSI, the serving AMF shall set value(s) of the mapped S-NSSAI(s) to value(s) of the EHPLMN S-NSSAI(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect AMF behavior in indirect network sharing EHPLMN case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.29	Support of indirect network sharing

A 5GS can support indirect network sharing between a hosting operator and participating operators as specified in 3GPP TS 23.501 [8]. The AMF may support indirect network sharing.

NOTE 1:	In this version of the specification, the support of indirect network sharing has no UE impact.

NOTE 2:	In indirect network sharing, the AMF of hosting operator acts towards to the UE as the AMF of the selected PLMN or the RPLMN.

NOTE 3:	In order to support indirect network sharing deployment, when a UE of participating operator registers in the HPLMN, the HPLMN can include a PLMN ID representing the participating operator in the list of equivalent PLMNs.

NOTE 4:	In indirect network sharing, if the UE is not allowed to access the selected PLMN which represents the participating operator, the AMF of hosting operator can reject the registration request using an existing cause value.

NOTE 5:	In indirect network sharing, if the UE has registered to HPLMN 5GCN over 3GPP access and is registering to the HPLMN 5GCN over non-3GPP access, and the network determines that the UE cannot be registered to the HPLMN 5GCN over 3GPP access and non-3GPP access simultaneously, then the AMF of HPLMN can reject the registration request over non-3GPP access using 5GMM cause value #111 "protocol error, unspecified".

In indirect network sharing, during the generic UE configuration update procedure (see subclause 5.4.4), the registration procedure for initial registration (see subclause 5.5.1.2) or the registration procedure for mobility and periodic registration update (see subclause 5.5.1.3), if the selected PLMN ID is the PLMN ID of the HPLMN or an EHPLMN of the UE, the AMF of hosting operator shall determine the available S-NSSAI(s) of hosting operator corresponding to the S-NSSAI(s) of participating operator provided by the UE. If the selected PLMN ID is the PLMN ID of an EHPLMN of the UE whose PLMN code is not derived from the IMSI, the AMF shall set value(s) of the mapped S-NSSAI(s) to value(s) of the S-NSSAI(s) of the serving EHPLMN and shall only use the corresponding S-NSSAI(s) of the serving EHPLMN, i.e. S-NSSAI(s) of participating operator without mapped S-NSSAI(s) towards the UE.

NOTE 6:	If the selected PLMN ID is the PLMN ID of the HPLMN of the UE, the AMF uses only the corresponding HPLMN S-NSSAI(s), i.e. S-NSSAI(s) of participating operator, towards the UE.

NOTE 6A:	For a deployment requirement regarding network slicing for EHPLMNs in indirect network sharing see 3GPP TS 23.501 [8], subclause 5.18.5.

In indirect network sharing, during the UE-requested PDU session establishment procedure, if the serving PLMN ID is the PLMN ID of the HPLMN or an EHPLMN of the UE, the AMF of hosting operator performs SMF selection using the determined S-NSSAI of hosting operator and the corresponding S-NSSAI of participating operator. If the serving PLMN ID is the PLMN ID of an EHPLMN of the UE whose PLMN code is not derived from the IMSI, the SMF of hosting operator shall only use the corresponding S-NSSAI of participating operator without mapped S-NSSAI towards the UE.

NOTE 7:	If the serving PLMN ID is the PLMN ID of the HPLMN of the UE, the SMF of hosting operator uses only the corresponding HPLMN S-NSSAI, i.e. S-NSSAI of participating operator, towards the UE.

NOTE 8:	If the selected PLMN ID is the PLMN ID of the HPLMN or an EHPLMN of the UE, the AMF of hosting operator uses the determined S-NSSAI(s) of hosting operator and the corresponding S-NSSAI(s) of participating operator towards other core-network functions (e.g. SMF).

NOTE 9:	If the selected PLMN ID is different from the PLMN ID of the HPLMN and EHPLMN(s) of the UE, the AMF and the SMF of hosting operator can use S-NSSAI(s) of hosting operator as S-NSSAI(s) of serving PLMN, and corresponding mapped S-NSSAI(s).

* * * End of Change * * * *