3GPP TSG-CT WG1 Meeting #157	C1-256247

Sophia-Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Resolve references in Spatial Map CUD service operations

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

This p-CR resolves the ENs in 3GPP TS 24.550 in Spatial Map CUD service operations.

2. Reason for Change

In the TS 24.550v1.1.0, in Spatial Map create, update and delete (CUD) service operations the references to the clause numbers are under "TBD" and needs to be resolved with the correct clause numbers defined in 29.437, which is addressed in this pCR.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

###### 5.3.1.2.2.2	SM client creating spatial map on SM server using SS_SmManagement_Create operation

Upon receiving the request from the VAL client for the creation of spatial map, the SM client shall send an HTTP POST request to the SM server on the resource URI identifying the "Spatial Maps" collection resource as specified in 3GPP TS 29.437 [6] in clause "6.2.1.3.2.2". The body of the HTTP POST message shall include the "SpatialMapCreateReq" data structure as specified in 3GPP TS 29.437 [6] in clause "6.2.1.3.2.3.1".

Upon reception of the HTTP POST message from the SM client, the SM server shall:

a)	process the spatial map create request;

b)	the SM server verifies and checks if the SM client is authorized to create the spatial map; and

c)	if the requestor is authorized, the SM server shall perform the creation of the spatial map. If the create operation:

1)	is successful, then the SM server shall further generate a globally unique spatial map identifier for the newly created spatial map and the layer identifier for the corresponding layers of the map. The SM server shall send the POST response with HTTP "201 Created" status code and the POST response body including the "SpatialMap" data structure as specified in 3GPP TS 29.437 [6] in clause "6.2.1.6.2.5", which includes the spatial map information of newly created spatial map.

NOTE:	Associating the spatial map identifier and its associated layer identifiers with the VAL service information and the requestor identifier is up to implementation.

2) 	is in progress, then the SM server shall further generate a globally unique spatial map identifier for the newly created spatial map and shall send the map identifier in the POST response with HTTP "202 Accepted" status code and the POST response body including the "SpatialMapTempResp" data structure as specified in 3GPP TS 29.437 [6] in clause "6.2.1.6.2.21". When the request is processed the SM server shall send a notification to the SM client as specified in 3GPP TS 29.437 [6]in clause "5.3.1.2.7.2".

3)	fails, then the SM server shall send the POST response set with the appropriate HTTP status code indicating the "failure" and the data structure as specified in 3GPP TS 29.437 [6]in clause "6.2.1.7".

* * * Next Change * * * *

###### 5.3.1.2.3.2	SM client updating spatial map on SM server

Upon receiving the request from the VAL client to update the spatial map resource, the SM client shall send:

a)	an HTTP PATCH request (for partial update) to the SM server on the resource URI identifying the "Individual Spatial Map" resource as specified in 3GPP TS 29.437 [6] in clause "6.2.1.3.3.2" with the "SpatialMapPatch" data structure in the request message as specified in 3GPP TS 29.437 [6] in clause "6.2.1.6.2.11" or

b)	HTTP PUT request (for full replacement) to the SM server on the resource URI identifying the "Individual Spatial Map" resource as specified in 3GPP TS 29.437 [6] in clause "6.2.1.3.3.2" with the "SpatialMap" data structure in the request message as specified in 3GPP TS 29.437 [6] in clause "6.2.1.6.2.5".

Upon reception of the HTTP PATCH or PUT request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to update the spatial map; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, then SM server shall check for the spatial map resources. If the matching spatial map resource is;

1)		found, the SM server shall;

i)	update the attributes of the matching spatial map resource with the received "SpatialMapPatch" information in the request body for the case of HTTP PATCH request; or

ii)	replace the matching spatial map resource with the received "SpatialMap" information in the request body for the case of HTTP PUT request; and

iii)	if the update operation:

A)	is successful, then the SM server shall send the HTTP "204 No Content" response to the SM client; and

B)	is in progress, then the SM server shall send the HTTP "202 Accepted" response to the SM client. When the request is processed the SM server shall send a notification to the SM client as described in 3GPP TS 29.437 [6] in clause "5.3.1.2.7"; and

2)	not found, return HTTP "404 Not Found" response to the SM client.

* * * Next Change * * * *

###### 5.3.1.2.4.2	SM client deleting spatial map on SM server

Upon receiving the request from the VAL client to delete the spatial map resource, the SM client shall send an HTTP DELETE request to the SM server on the resource URI identifying the "Individual Spatial Map" resource as specified in 3GPP TS 29.437 [6] in clause "6.2.1.3.3.2".

Upon reception of the HTTP DELETE request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to delete the spatial map; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, then SM server shall check for the spatial map resources. If the matching spatial map resource is;

1)	not found, return the HTTP "404 Not Found" response to the SM client; or

2)	found, delete the identified spatial map resource and shall send the HTTP "204 No Content" response to the SM client.

* * * End of Changes * * * *