3GPP TSG-CT WG1 Meeting #157	C1-256257

# Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	Discussion on the issue regarding no Command Response in NAS

Agenda item:	19.70

Document for:	DECISION

# 1. Abstract

This paper points out the issue regarding no Command Response in NAS due to the requirement of handling of unknown, unforeseen, and erroneous protocol data in AIoT NAS and suggests informing RAN2 to study this issue.

# 2. Discussion

## 2.1 The impacts on the AS due to no Command Response in NAS

As indicated in the RAN#131Meeting report/Chair notes, no NAS data available will cause the reader to schedule another D2R Upper Layer Data Transfer message from the device, which leads to an issue to a reader, i.e. repeatedly schedule another D2R Upper Layer Data Transfer message when the reader keeps receiving 0 SDU due to no command response in NAS at all. It’s different from the case where the AIoT device lacks energy, in which no 0 SDU is received from the AIoT device and no impacts to the reader.

*****

Copied from RAN#131Meeting report/Chair notes:

8.2.4	A-IoT Data Transmission and Other general aspects

...

Follow-up discussion on CT1 LS

...

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

## 2.2 The requirement on handling of unknown, unforeseen, and erroneous protocol data

According to TS 24.369 clause 6, in the case of message too short or too long, unknown or unforeseen message type and so on, the message shall be ignored, which means the scenario that no Command Response in NAS could happen when the the AIoT device handles the protocol errors.

- Proposal

Based on the discussion in clause 2, it is proposed to send an LS to RAN2 to inform them of the existence of no Command Response in NAS.

- This DP is related to C1-256258.