3GPP TSG-CT WG1 Meeting #157	C1-256309

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Inventory procedure, authentication abnormal case

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

It has been added in 24.369 cl. 5.2.5, Inventory procedure, Abnormal cases in the AIoT device, that the AIoT device shall not respond to the AIoT paging message if it fails authentication. It has however been specified in 33.369 v1.0.0 that there is no explicit authentication of the network by the AIoT device in the inventory procedure. The authentication of the network by the AIoT device is done implicitly at verification of integrity protection in a subsequent command message:

In 33.369 there is no authentication of the network by the AIoT device in the Inventory procedure. Authentication of the network by the AIoT device is captured as follows (33.369, cl. 5.2.2):

“For the command procedure, the AIoT device implicitly authenticates the network via the verification of MAC which is derived using the KCommand_int as specified in clause 5.3.2 of present document.”

In pCR S3-252942 to 33.369, the deletion of the Editor’s Note on mutual authentication is justified as follows:

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

“To save the calculation resources and transmission resources of AIoT device, the implicit authentication method can be used for mutual authentication. By verifying the security of command request message, the AIoT device can implicitly determine the authentication result to the network.”

3. Conclusions

Following SA3 progress and 33.369 v1.0.0, it can be concluded that there is no explicit authentication of the network by the AIoT devive in the Inventory procedure, and the abnormal case in 24.369 can be deleted.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a

)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

* * * End of Changes * * *