3GPP TSG-CT WG1 Meeting #157	C1-256324

Sophia Antipolis, France, 13-17 October 2025

Source:	Lenovo

Title:	Pseudo-CR on Aimles_UeTLModelSelectionAssistance API

Spec:	3GPP TS 24.560V1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Aimles_UeTLModelSelectionAssistance API is missing in TS 24.560.

There are some errors in clauses related for Transfer Learning (TL) enablement service such as:

-	Some data types are missing in General clause 6.15.6.1

-	reordering Table 6.15.6.1-2

-	in clause 6.15.6.2.2, attribute mlModelProfile can have data type MlModelProfile.

-	in clause 6.15.6.2.5, attribute mlModelInfo is an array of data type AimlModelInfo.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24560V1.1.0.

* * * First Change * * * *

#### 6.15.6.1	General

This clause specifies the application data model supported by the Aimles_UeTLModelSelectionAssistance API.

Table 6.15.6.1-1 specifies the data types defined for the Aimles_UeTLModelSelectionAssistance API.

Table 6.15.6.1-1: Aimles_UeTLModelSelectionAssistance API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessType | 6.15.6.2.6 | Contains permission and restriction to access the pre-trained model. |  |
| EnvironmentType | 6.15.6.3.4 | Contains the environment for the target ML task. |  |
| MLModel | 6.15.6.2.5 | Contains the pre-trained ML models for the TL enablement service. |  |
| TlCriteria | 6.15.6.2.4 | Contains the criteria for transfer learning. |  |
| TlModelSelectAssistReq | 6.15.6.2.2 | Contains information to trigger TL enablement service. |  |
| TlModelSelectAssistResp | 6.15.6.2.3 | Contains one or more pre-trained ML model for the TL enablement service. |  |
| TlType | 6.15.6.3.3 | Contains the type of the transfer learning. |  |

Table 6.15.6.1-2 specifies data types re-used by the Aimles_UeTLModelSelectionAssistance API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimles_UeTLModelSelectionAssistance API.

Table 6.15.6.1-2: Aimles_UeTLModelSelectionAssistance API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [10] | Integer where the allowed values correspond to the value range of an unsigned 32-bit integer. |  |
| ValTargetUe | 3GPP TS 29.549 [9] | Unique identifier of a VAL user or a VAL UE. |  |

* * * Next Change * * * *

##### 6.15.6.2.2	Type: TlModelSelectAssistReq

Table 6.15.6.2.2-1: Definition of type TlModelSelectAssistReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | string | M | 1 | Identifies the AIMLE server |  |
| valSrvId | string | M | 1 | Identifying the VAL service, for which the TL enablement service is requested. |  |
| tlCriteria | TlCriteria | M | 1 | Identifying the criteria for transfer learning, for which the TL enablement service is requested. |  |
| mlTaskId | string | O | 0..1 | Identifying the ML task, for which the TL enablement service is requested. (NOTE) |  |
| adaeAnalyticsId | string | O | 0..1 | Identifying the ADAE analytics, for which the TL enablement service is requested. (NOTE) |  |
| mlModelProfile | MlModelProfile | O | 0..1 | Identifying the ML model profile, for which the TL enablement service is requested. (NOTE) |  |
| mlModelReq | array(string) | O | 0..N | Identifying the requirements for the ML model, for which the TL enablement service is requested. |  |
| valUeIds | array(ValTargetUe) | O | 0..N | Identifying a list of one or more VAL UEs associated with the target ML task, for which one or more pre-trained models are requested. |  |
| mlModelRateReq | array(string) | O | 0..N | Identifying the requirements for the rating of the ML model, for which the TL enablement service is requested. |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.15.6.2.3	Type: TlModelSelectAssistResp

Table 6.15.6.2.3-1: Definition of type TlModelSelectAssistResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelList | array(MlModel) | M | 1..N | Identifying the selected one or more pre-trained ML models, for which the TL enablement service is requested. |  |

* * * Next Change * * * *

##### 6.15.6.2.5	Type: MLModel

Table 6.15.6.2.5-1: Definition of type MLModel

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlRepositoryId | string | M | 1 | Identifies the unique repository identity of the ML model. |  |
| mlModelInfo | AimlModelInfo | M | 1 | Identifies the information on the ML model. |  |
| mlModelRating | Uint32 | O | 0..1 | Identifies the rating of the ML model. |  |

* * * Next Change * * * *

## A.16	Aimles_UeTLModelSelectionAssistance API

openapi: 3.0.0

info:

title: Aimles_UeTLModelSelectionAssistance

version: 1.0.0-alpha.1

description: |

API for Transfer Learning (TL) Enablement Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V1.1.0; Artificial Intelligence Machine Learning (AIML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimles-ue-tl-msa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/assist-tlms:

post:

summary: >

Used by the AIMLE client to request the AIMLE server to perform TL enablement service.

operationId: UeTlModSelectAssistReq

tags:

- UE transfer learning model selection assistance request

requestBody:

description: Contains information to trigger TL enabalement service.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TlModelSelectAssistReq'

responses:

'200':

description: Contains one or more pre-trained ML models for the TL enablement service.

content:

application/json:

schema:

$ref: '#/components/schemas/TlModelSelectAssistResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

TlModelSelectAssistReq:

description: Contains information to trigger TL enabalement service.

type: object

required:

- serverId

- valSrvId

- tlCriteria

properties:

serverId:

description: Represents the AIMLE server identity.

type: string

valSrvId:

description: Represents the VAL service for the TL enablement service.

type: string

tlCriteria:

description: Represents the criteria for the TL enablement service.

$ref: '#/components/schemas/TlCriteria'

mlTaskId:

description: Represents the task for the TL enablement service.

type: string

adaeAnalyticsId:

description: Represents the ADAE analytics for the TL enablement service.

type: string

mlModelProfile:

description: Represents the ML model profile for the TL enablement service.

$ref: 'TS24560_Aimlec_MLModelTrainingCapabilityEva.yaml#/components/schemas/MlModelProfile'

mlModelReq:

description: Represents the requirements for the ML model for the TL enablement service.

type: array

items:

type: string

minItems: 0

valUeIds:

description: Represents VAL UEs associated with the ML task for the TL enablement service.

type: array

items:

$ref: '#/components/schemas/ValTargetUe'

minItems: 0

mlModelRateReq:

description: Represents requirements for rating the ML model for TL enablement service.

type: array

items:

type: string

minItems: 0

TlModelSelectAssistResp:

description: Contains the pre-trained ML models for the TL enablement service.

type: object

required:

- mlModelList

properties:

mlModelList:

description: Represents the selected pre-trained ML models for the TL enablement service.

type: array

items:

$ref: '#/components/schemas/MlModel'

minItems: 1

TlCriteria:

description: Contains the criteria for the TL enablement service.

type: object

properties:

reqFeatures:

description: Represents the required features for a pre-trained model.

type: array

items:

type: string

minItems: 0

dataReq:

description: Represents the training data requirements.

type: array

items:

type: string

minItems: 0

tlType:

description: Represents the type of transfer learning.

$ref: '#/components/schemas/TlType'

environment:

description: Represents the environment which is associated with the target ML task.

$ref: '#/components/schemas/EnvironmentType'

access:

description: Permissions and restrictions to access the pre-trained model.

type: array

items:

$ref: '#/components/schemas/AccessType'

minItems: 0

MLModel:

description: Contains the pre-trained ML models for the TL enablement service.

type: object

required:

- mlRepositoryId

- mlModelInfo

properties:

mlRepositoryId:

description: Represents the unique repository identity of an ML model.

type: string

mlModelInfo:

description: Contains information about the ML model.

$ref: 'TS24560_Aimlec_MLModelTrainingCapabilityEva.yaml#/components/schemas/AimlModelInfo'

mlModelRating:

description: Represents the rating of an ML model.

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

AccessType:

description: Contains permission and restriction to access the pre-trained model.

type: object

properties:

modelLicense:

description: Represents the license of the pre-trained ML model.

type: string

dataTrainLicense:

description: Represents the license of the dataset for the pre-trained ML model.

type: string

codeTrainLicense:

description: Represents the license of the code for the pre-trained ML model.

type: string

# Enumerations

TlType:

anyOf:

- type: string

enum:

- INDUCTIVE

- TRANSDUCTIVE

- UNSUPERVISED

- type: string

description: >

This string provides the type of transfer learning.

description: |

Identifies how the pre-trained ML models can be related to the trained ML models for

the target ML task.

Possible values are:

- INDUCTIVE: For different task than the target ML task

- TRANSDUCTIVE: For similar task as target ML task but differet data distributions

- UNSUPERVISED: For similar tasks as target ML task but with unlabled data

EnvironmentType:

anyOf:

- type: string

enum:

- DOMAIN_SHIFT

- SIMULATED_REAL

- DYNAMIC

- HETEROGENEOUS

- ROBOTICS

- SMART

- type: string

description: >

This string provides the type of the environment.

description: |

Identifies how the environment and data distributions, suited for pre-trained ML models

in relationship with the trained ML model for the target ML task.

Possible values are:

- DOMAIN_SHIFT: Different data distributions

- SIMULATED_REAL: Simulated data vs. real data

- DYNAMIC: Data distributions changes over time

- HETEROGENEOUS: Significantly different data distributions

- ROBOTICS: Suitable for the target ML task for robotic platforms

- SMART: Suitable for the target ML task for smart platforms

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *