3GPP TSG-CT WG1 Meeting #157	C1-256331

# Sophia-Antipolis, France 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.193 | CR | 0231 | rev | 2 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of UE requested MA PDU session with invalid capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For deterministic network response to a case when at a UE request related to an MA PDU session which includes invalid UE ATSSS capabilities, it is proposed to add a new case in missing information elements that specifies which cause value to include in the reject. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addiiton of ATSSS status as the ATSSS parameter with status code "Semantically incorrect message". Addition of ATSSS status indicator in ATSSS response with the length of two octets PCO parameter. Description of the handling of the MA PDU connectivity service failure when the UE does not indicate support for any the ATSSS steering functionality during the session setup. Compatibility analysis: The proposal is backward compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No handling is defined for the PGW-C+SMF when receiving a request for an MA PDU session establishment without indicating any ATSSS steering functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3b (new), 5.3b.1(new), 5.3b.2(new), 5.3b.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** First Change *******************************

## 5.3b	Handling of multi-access PDU connectivity service failure

### 5.3b.1	General

Activating a multi-access PDU (MA PDU) connectivity service may fail if the UE and the network indicate incompatible supports for the ATSSS steering functionality during session setup. This clause specifies the necessary actions for the network when this problem occurs.

### 5.3b.2	Incompatible steering functionality between the UE and the network

#### 5.3b.2.1	Network procedures

Activating the MA PDU connectivity service may fail when the UE attempts to establish PDN connection over:

a)	E-UTRAN connected to EPC as a user-plane resource of an MA PDU session to be established as described in clause 5.3.1; or

b)	untrusted non-3GPP access network connected to EPC as a user-plane resource of an MA PDU session to be established as described in clause 5.3a.2,

if the PGW-C+SMF receives Create Session Request message containing the Extended protocol configuration options IE with the ATSSS request PCO parameter that does not indicate any steering functionality or steering mode, the PGW-C+SMF shall handle the Extended protocol configuration options IE according to clause 7.7.8 in 3GPP TS 29.274 [18] and reject by constructing the Create Session Response message with the reject cause value #69 "Mandatory IE incorrect" in the ATSSS response with the length of two octets PCO parameter container contents in the Extended protocol configuration options IE.

In case of the procedure a) above, when the MME receives the Create Session Response message with the reject cause value #69 "Mandatory IE incorrect" and the Extended protocol configuration options IE from the SGW as specified in 3GPP TS 29.274 [18], the MME shall construct the PDN CONNECTIVITY REJECT message with the ESM cause containing the cause value #30 "Request rejected by Serving GW or PDN GW" and include the received Extended protocol configuration options IE and forward the message towards the UE as specified in 3GPP TS 24.301 [4]. The MME uses the mapping of the rejection cause values listed in annex C of 3GPP TS 29.274 [18].

In case of the procedure b) above, when the ePDG receives the Create Session Response message with a reject cause value #69 "Mandatory IE incorrect" and the Extended protocol configuration options IE from the PGW-C+SMF as specified in 3GPP TS 29.274 [18], the ePDG shall construct the ATSSS_RESPONSE Notify payload as specified in 3GPP TS 24.302 [17], copy the ATSSS response with the length of two octets PCO parameter container contents into the ATSSS response information field of the ATSSS_RESPONSE Notify payload, and include the ATSSS_RESPONSE Notify payload in the IKE_AUTH response message. The ePDG shall also include a Notify payload with a private notify message to the UE, indicating cause #30 "Request rejected by PDN GW" in the IKE_AUTH response message.

****************************** End of Changes *******************************