3GPP TSG-CT WG1 Meeting #157	C1-256387

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0111 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's notes under clause 7.2.24.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains the following editor’s note under clause 7.2.24.2, quote: Editor's note [WID: XRM_Ph2_App, CR#: 0094]: The definition of multimodalSealddFlowId is FFS. Last meeting (CT1#156), the need of having a <sealdd-multi-modal-flow-id> element for the response message to an SEALDD multi-modal transmission connection establishment request was questioned. This resulted in the editor note above being added to the specification. Having a dedicated multi‑modal flow identifier is useful when messages have to reference, correlate, or manage the entire group of SEALDD flows as a single logical object. However, it is unnecessary if each response always contains exactly one grouping of sealdd flows and there is no reference to that grouping from elsewhere (any other message). Analysis stage-2 requirements from TS 23.433, a dedicated multi‑modal flow identifier is firstly optional. Furthermore, it is only indicated by the server to the client (direction server->client) and never from the client to the server. Also, for the the response message to an SEALDD multi-modal transmission connection establishment request only one grouping of sealdd flows is possible per message. However, the server can optionally reference a grouping of seal flows by using multi‑modal flow identifier in the SEALDD client policy configuration request (clause 9.10.3.7 of TS 23.433) and the Transmission quality measurement subscription request (clause 9.7.3.6 of TS 23.433). Hence, there is optinally reference to multi-modal grouping of sealdd flows from other messages, so if the multi‑modal flow identifier is provided in those messages, it could be beneficial to manage an entire group of sealdd flows as a single logical object using an identifier at the client side (UE). It is therefore proposed to support a dedicated multi‑modal flow identifier as per stage-2 description. Anyhow, note that an implementation can decide whether to use that provided multi‑modal flow identifier or use any other implementation method internally at the client side (UE) to reference a group of sealdd flows (e.g., memory pointers). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification is updated as per described in the reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification after freezing the release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.24.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.2.24.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <XR-establishment-req> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request SEALDD multi-modal transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; or

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-establishment-rsp> element within <anyExt> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD multi-modal transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error;

ii)	if <result> element is set to "success", the <multi-modal-sealdd-flow-info> element shall be included and set to the information of the multi modal SEALDD flow used by the SDDM-S and SDDM-C. The <multi-modal-sealdd-flow-info> shall include:

A)	a <sealdd-multi-modal-flow-id> child element set to the identity of the SEALDD flow used by the SDDM-C and SDDM-S to identify the multi-modal XR traffic; and

B)	a <sealdd-flows-list> child element set to list of SEALDD flows used by the SDDM-S and SDDM-C:

I)	shall include one or more <sealdd-flow-id> child elements set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal traffic; and

II)	may include a <traffic-descriptor-info> child element specifying the information of traffic descriptors for the multi-modal traffic. The <traffic-descriptor-info> may include a <user-plane-address> child element specifying the identity of the IP address of the traffic; a <port-number> child element specifying the identity of the port number of the traffic; a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

iii)	if <result> element is set to "success", the <protocol-descriptor-info> element may be included and set to the information of the protocol of the VAL traffic. In the <protocol-descriptor-info> element, the SDDM-S:

A)	may include a <header-ext-info> child element specifying the information of the header extension, e.g. RTP extension with PDU set. This child element is only included if the payload type is RTP. The<header-ext-info> element:

I)	may include a <header-ext-type> child element set to the type of the header extension protocol (e.g, RTP, SRTP); and

II)	may include a <header-ext-id> child element set to the identity of the header extension protocol with value from 1 to 255;

B)	may include a <packetization-indication> child element set to packetization indication;

C)	may include a <payload-type> child element set to the type of the payload, e.g. RTP, SRTP; and

D)	shall include a <payload-format> child element set to the format of the payload, e.g. H.264, H.265; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

* * * End of Changes * * * *