3GPP TSG-CT WG1 Meeting #157	C1-256400

# Sophia Antipolis, France, 13-17 October 2025

Source:	Ericsson

Title:	AIoT identification information

Agenda item:	19.70

Document for:	DISCUSSION

# 1. Background

SA2 agreed in TS 23.369 about that the network provides the AIoT identification information to AIoT device via NG-RAN for paging, and then SA3 agreed in TS 33.369 on the authentication procedure which requires the network to send the security related parameters e.g RANDAIOT_n to AIoT device in paging, and agreed in TS 33.369 that T-ID and type of T-ID handling are provided as device identification information  in paging for individual inventory with AIoT Device identifier protection.

# 2. Discussion

According to TS 23.369 subclause 6.2.2 step 4:

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

And step 9:

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

So the AIOTF sends the AIoT identification information to the AIoT device via NG-RAN, and the AIoT device uses the AIoT Identification Information to match it’s device id.

It indicates that AIoT identification information is provided by AIOTF to AIoT device via NG-RAN.

Based on the TS38.391 clause 5.2:

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices.

…

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

…

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate it to the upper layers;

3>	else:

4>	forward the content of the Paging ID field to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

…

TS38.391 clause 6.2.1.1:

-	Paging ID: If present, this field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

It indicates that the AIoT device AS layer passes the content of Paging ID field which contains the AIoT Identification information to the AIoT device NAS to match it’s device id, if it’s matched, the AIoT NAS indicates to the AS layer that the AIoT identification information is matched, and AIoT device determines to respond to the AIoT paging. Thus the AIoT Identification information is used by the AIoT NAS layer.

Observation 1: For inventory, the AIOTF generates the AIoT Identification information and sends it to NG-RAN. NG-RAN sends AIoT Identification information to the AIoT device AS layer in the Paging ID of paging message. The AIoT Identification information in paging ID is used by the AIoT device NAS layer to match it’s device id.

According to TS 23.369 subclause 6.2.2 step 4:

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

It indicates that the AIoT Identification information can be Filtering Information for group inventory or a single AIoT Device Identifier for individual inventory.

According to TS 33.369 subclause 5.4.1:

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

If privacy protection is NOT used, then it would be logical to include AIoT device permanent ID in AIoT Identification information in paging message for individual paging, since it would anyway be included by the UE in INVENTORY REPORT as stated in "the AIoT device includes its AIoT device permanent identifier as a device identification information".

And TS 33.369 subclause 5.4.3:

-	In step 2, 3 and 4, the T-ID shall be used as a device identification information.

So the AIoT Identification information can be one AIoT device permanent identifier or one T-ID.

For the T-ID, 33.369 subclause 5.4.3 further states:

-	In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message.

So the T-ID is either concealed type T-ID or stored type T-ID.

And RAN2 agreed the paging all use case in TS 38.391:

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate it to the upper layers;

3>	else:

4>	forward the content of the Paging ID field to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

Observation 2: The AIoT identification information can be:

1. Filtering Information for group paging;

2. One T-ID for individual paging when privacy protection is used;

3. One AIoT device permanent identifier for individual paging when privacy protection is not used; or

4. All AIoT devices.

Since there are different types for the contents of the AIoT Identification information, there are different actions for the AIoT device based on the type of the contents of the AIoT Identification information, e.g if the AIoT Identification information contains a filtering information, the AIoT device shall start to match the PLMN ID field, NID field, .etc; if the AIoT Identification information contains a concealed T-ID, the AIoT device shall retrieve a T-ID and compare the derived T-ID with AIOT device identity in paging message.

Thus the the AIoT Identification information is different based on different types/contents included in the AIoT Identification information, and such type information is essential for the AIoT device to perform the matching, and shall be sent from AIOTF to AIoT device.

Observation 3 : When receiving the AIoT Identification information, the AIoT device performs different handlings based on the type of AIoT Identification information.

And according to 33.369 subclause 5.4.3:

-	In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message.

So the T-ID can be concealed type or stored type for individual paging when privacy is used.

And the RANDAIOT_n is included in the paging message as well.

AIOTF shall send the RANDAIOT_n, type of AIoT Identification information, AIoT Identification information to the AIoT device, information for T-ID handling in the paging message to the AIoT device via NG-RAN, and these information are used by AIoT device NAS to perform matching and decides what to be included in the INVENTORY REPORT message.

Observation 4: The RANDAIOT_n, other security parameters for T-ID handling, filtering information, AIoT device permanent identifier and T-ID are used by the AIoT device NAS only, and should be transparent for NG-RAN, reader and AIoT device AS.

There are 2 options for AIOTF to send the AIoT identification information, RANDAIOT_n, other security parameters for T-ID handling, filtering information, AIoT device permanent identifier and T-ID to AIoT device NAS:

Option 1: in 24.369, define a AIoT identification information container which is transparent for NG-RAN, reader, AIoT AS, and AIoT identification information container includes the RANDAIOT_n, content of AIoT identification information, and the information to help the AIoT device to decode AIoT identification Information, such as paging type, parameters for T-ID handling;

Option 2: 1). Extend the A-IoT Device Identification Requested in NGAP, to add Device type e.g [Type=Stored T-ID/concealed T-TD/permanent ID] under Single Device, and extend 23.003 to add the T-ID .etc; 2). Add support for the security parameter RANDAIOT_n in NGAP message; 3). RAN2 decides to add the RANDAIOT_n in paging ID or introduce a new parameter outside the paging ID.

There are 4 reasons for choosing Option 1:

1) Considering that RANDAIOT_n, type of AIoT identification information, AIoT identification information, information for T-ID handling in the paging message are used by AIoT NAS only, for option 2, the AIoT NAS handling depends on NGAP for A-IoT Device Identification Requested and 23.003 for the T-ID, permanent identifier, filtering information definitions, the details of the AIoT identification information is distributed in several specs belong to different working groups, it’s hard for understanding and trouble shooting, to make the design simple, Option 1 is preferred.

2) For group paging and individual paging, whether the A-IoT Device should respond to the paging is decided by A-IoT Device NAS layer, but for the paging all, A-IoT Device AS layer decides to respond to the paging, it’s not a good protocol design, it’s better that one layer which makes decision to reply to the paging for all different cases.

3) The RANDAIOT_n is used by A-IoT Device NAS only, and logically it should be included in the paging id as the other parameters, so the paging ID would never be empty since RANDAIOT_n is always included in the inventory procedure, so what defined in RAN2 now about paging all for empty paging ID would never happen, unless another parameter carrying the RANDAIOT_n is defined in RAN2 and it is sent to A-IoT Device NAS with paging ID.

4) In 23.003 clause 31.2, the structure of AIoT Device Permanent Identifier is defined as not byte-aligned due to the NID is 44 bit, 4 bits of 0000 will be padded, but it can be padded in the end of the NID field, or in the end of the whole AIoT Device Permanent Identifier, 38.413 refers to the 23.369, and 23.369 is not clear about the AIoT Device Permanent Identifier in paging message, and AIoT NAS will define the AIoT Device Permanent Identifier in the INVENTORY REPORT message, the definition should be aligned with the paging message and the INVENTORY REPORT message, so it’s better to define such details of the AIoT Identification Information in one place which is AIoT NAS.

Observation 5: The option to define a AIoT Identification Information container which includes the RANDAIOT_n, content of AIoT Identification Information, and other information such as paging type, parameters for T-ID handling in 24.369 is preferred.

# 3. Conclusion and proposal

In the above clauses the following observations were made:

Observation 1: For inventory, the AIOTF generates the AIoT Identification Information and sends it to NG-RAN. NG-RAN sends AIoT Identification Information to the AIoT device AS layer in the Paging ID of paging message. The AIoT Identification Information is used by the AIoT device NAS layer to match it’s device id.

Observation 2: The AIoT Identification Information can be:

1. Filtering Information for group paging;

2. One T-ID for individual paging when privacy protection is used;

3. One AIoT device permanent identifier for individual paging when privacy protection is not used; or

4. All AIoT devices.

Observation 3 : When receiving the AIoT Identification Information, the AIoT device performs different handlings based on the type of AIoT Identification Information.

Proposal 1: It is proposed to add the handling for the AIoT device to perform matching based on the type/content of the received AIoT Identification Information.

Observation 4 : The RANDAIOT_n, other security parameters for T-ID handling, filtering information, AIoT device permanent identifier and T-ID are used by the AIoT device NAS only, and should be transparent for NG-RAN, reader and AIoT device AS.

Observation 5: The option to define a AIoT Identification Information container which includes the RANDAIOT_n, content of AIoT Identification Information, and other information such as paging type, parameters for T-ID handling in 24.369 is preferred.

Proposal 2: It is proposed to define the encoding of the AIoT Identification Information which contains RANDAIOT_n, the AIoT identification information with the type of AIoT Identification information  in 24.369.

Due to content of paging message defined in 38.391 depends on the maximum length of the AIoT Identification information  which is the content of paging id, it is proposed to send an LS to RAN 2 notifying the maximum length of the AIoT Identification information .

Proposal 3: It is proposed to send an LS to RAN 2 notifying the maximum length of the AIoT Identification information .

Proposal #1 and #2 are captured in C1-256401. Proposal #3 is captured in C1-256402.