3GPP TSG-CT WG1 Meeting #157	C1-256407

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7044 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of NS-AoS enforcement and unavailable S-NSSAI handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2#170 agreed on S2-2507958 (3GPP TS 23.501 CR 6018), which revised handling of unavailable S-NSSAIs for UEs not supporting time validity information, as well as handling of S-NSSAIs when the UE is outside of the NS-AoS. This was done in order to address issues arising from potential UE deregistration, and subsequent loss of UE context in the AMF. The issue is addressed by handling the S-NSSAI as part of the rejected NSSAI (and as part of allowed NSSAI when applicable), instead of updating the configured NSSAI. The updated handling requires alignment of Stage-3 specification for temporarily available network slices and NS-AoS enforcement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned handling of unavailable S-NSSAI for UEs not supporting time validity information, and NS-AoS enforcement-based S-NSSAI handling, to use allowed- and rejected NSSAI instead of updating the configured NSSAI. Backward compatibility analysis: The changes are backward compatible since only AMF behaviour is impacted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot request S-NSSAI previously removed from configured NSSAI even when it has become available by change in time and/or location, after deregistration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.8, 4.6.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.2.8	Mobility management for optimised handling of temporarily available network slices

The UE and the network may support optimised handling of temporarily available network slices. The support for S-NSSAI time validity information by the UE and the network, respectively, is optional.

If the UE has indicated that it supports S-NSSAI time validity information, then the AMF may include the S-NSSAI time validity information for one or more S-NSSAIs included in the configured NSSAI in the Registration Accept message or the CONFIGURATION UPDATE COMMAND message. If the AMF determines that the S-NSSAI time validity information for an S-NSSAI in the configured NSSAI is changed, the AMF may provide the UE with a new S-NSSAI time validity information for that S-NSSAI via the CONFIGURATION UPDATE COMMAND message.

If the UE supporting S-NSSAI time validity information, is configured with S-NSSAI time validity information for an S-NSSAI and:

a)	the S-NSSAI time validity information indicates that the S-NSSAI is available, then the UE may request the S-NSSAI in the requested NSSAI in the REGISTRATION REQUEST message;

b)	the S-NSSAI time validity information indicates that the S-NSSAI is not available, then:

i)	the UE shall not include the S-NSSAI in the requested NSSAI in the REGISTRATION REQUEST message;

ii)	the UE shall remove the S-NSSAI from the stored allowed NSSAI (if any) and the stored partially allowed NSSAI (if any) in the non-volatile memory in the ME, as specified in annex C;

iii)	the S-NSSAI time validity information indicates that the S-NSSAI will not become available again, then the UE shall remove the S-NSSAI from the stored configured NSSAI in the non-volatile memory in the ME, as specified in annex C; and

iv)	the UE shall delete the entry from the alternative NSSAI (if any) containing the S-NSSAI in the replaced S-NSSAI or the alternative S-NSSAI as specified in subclause 4.6.2.2.

NOTE 1:	If the S-NSSAI to be removed is the only S-NSSAI in the allowed NSSAI and partially allowed NSSAI, if applicable, the UE can before the time validity expiry initiate the registration procedure for mobility and periodic registration update and include a different S-NSSAI from the configured NSSAI in the Requested NSSAI IE of the REGISTRATION REQUEST message. If the UE does not initiate the registration procedure for mobility and periodic registration update and the time validity for the only S-NSSAI in the allowed NSSAI and partially allowed NSSAI expires, the UE locally enters the state 5GMM-DEREGISTERED.

When the S-NSSAI time validity information of an S-NSSAI indicates that the S-NSSAI is not available, then:

a)	if the AMF receives a requested NSSAI in the REGISTRATION REQUEST message with the S-NSSAI identifying the network slice, the AMF shall:

i)	to a UE which has indicated that it supports S-NSSAI time validity information, provide:

1)	a configured NSSAI including the S-NSSAI together with the S-NSSAI time validity information in the REGISTRATION ACCEPT message if the S-NSSAI will become available again; or

2)	a configured NSSAI not including the S-NSSAI in the REGISTRATION ACCEPT message if the S-NSSAI will not become available again; or

ii)	to a UE which has not indicated that it supports S-NSSAI time validity information, reject the S-NSSAI for the current PLMN or SNPN. If the registration request is accepted, the AMF shall include the rejected NSSAI including the S-NSSAI;

b)	if the AMF detects that the S-NSSAI is included in the allowed NSSAI or the partially allowed NSSAI of a UE which has:

i)	indicated that it supports S-NSSAI time validity information, the AMF shall locally remove the S-NSSAI from the allowed NSSAI (if any), the partially allowed NSSAI (if any) and the entry from the alternative NSSAI (if any) containing the S-NSSAI in the replaced S-NSSAI or the alternative S-NSSAI; or

NOTE 2:	If there is no S-NSSAI included in the allowed NSSAI and partially allowed NSSAI after local removal of the S-NSSAI from the allowed NSSAI or the partially allowed NSSAI, the network locally enters the state 5GMM-DEREGISTERED for the UE.

ii)	not indicated that it supports S-NSSAI time validity information, the AMF shall remove the S-NSSAI from the stored allowed NSSAI (if any), and partially allowed NSSAI (if any) by sending the CONFIGURATION UPDATE COMMAND message, optionally including the rejected NSSAI including the S-NSSAI.

NOTE 3:	If there is no S-NSSAI included in the allowed NSSAI and partially allowed NSSAI after removal of the S-NSSAI from the allowed NSSAI or the partially allowed NSSAI, then the AMF performs the network-initiated de-registration procedure and sets the 5GMM cause value to #62 "No network slices available" in the DEREGISTRATION REQUEST message".

When the S-NSSAI time validity information of an S-NSSAI indicates that the S-NSSAI becomes available again, the AMF shall provide the allowed NSSAI including the S-NSSAI to a UE which has not indicated that it supports S-NSSAI time validity information by sending CONFIGURATION UPDATE COMMAND message if the UE is subscribed to the S-NSSAI.

The S-NSSAI time validity information is applicable for the current PLMN or SNPN regardless of the access type.

* * * Next Change * * * *

#### 4.6.2.10	Mobility management aspects of handling network slices with NS-AoS not matching deployed tracking areas

An operator can choose to let the NS-AoS of an S-NSSAI not match the existing tracking area boundaries (see subclause 5.15.18 of 3GPP TS 23.501 [8]). In order to support this deployment option, the operator has to ensure that an AMF covering the NS-AoS operates as described below.

The support for S-NSSAI location validity information by the UE and the network, respectively, is optional. If a UE supports S-NSSAI location validity information, the UE indicates that it supports S-NSSAI location validity information during the registration procedure (see subclause 5.5.1). The AMF can provide a UE which has indicated that it supports S-NSSAI location validity information with S-NSSAI location validity information (see subclauses 5.4.4 and 5.5.1). The S-NSSAI location validity information consists of, for each of the applicable S-NSSAI(s) in the configured NSSAI:

a)	an S-NSSAI; and

b)	a list of cell identities of TA(s) belonging to the registration area where the related S-NSSAI(s) is available in some cells but not all cells of one or more TAs, which represents the NS-AoS of the S-NSSAI.

The UE shall consider itself to be inside the NS-AoS of the S-NSSAI if the cell identity of the current serving cell matches any of the identities in the S-NSSAI location validity information. Otherwise, the UE shall consider itself to be outside the NS-AoS of the S-NSSAI.

NOTE 1:	The cell identity of the current serving cell is received from the lower layers.

For an S-NSSAI in the S-NSSAI location validity information, even if the S-NSSAI is included in the rejected NSSAI with a rejection cause value set to "S-NSSAI not available in the current registration area" or is included in the partially rejected NSSAI, the UE is allowed to request the S-NSSAI if the UE determines that it is inside the NS-AoS of the S-NSSAI.

NOTE 2:	If the service area list for the service area restrictions is available in the UE, the service area list is evaluated before the S-NSSAI location validity information.

For an S-NSSAI limited by NS-AoS, if the UE in 5GMM-CONNECTED mode does not support S-NSSAI location validity information and the AMF determines that:

a)	the UE is outside the NS-AoS, then the AMF may:

1)	provide the UE with an allowed NSSAI or a partially allowed NSSAI excluding the S-NSSAI, and optionally a rejected NSSAI including the S-NSSAI; and

NOTE 3:	If excluding the S-NSSAI limited by NS-AoS results in an empty allowed NSSAI or partially allowed NSSAI, the AMF includes one or more default S-NSSAIs in the provided allowed NSSAI or partially allowed NSSAI.

2)	indicate to the SMF to release all PDU sessions associated with the S-NSSAI; or

b)	the UE is in the NS-AoS, then the AMF may provide the allowed NSSAI including the S-NSSAI in the allowed NSSAI.

If the UE that does not support S-NSSAI location validity information requests a PDU session establishment for an S-NSSAI limited by NS-AoS and the AMF determines that the UE is outside the NS-AoS of the S-NSSAI, the AMF may perform S-NSSAI based congestion control for the S-NSSAI as specified in subclauses 5.3.11 and 5.4.5.

The S-NSSAI location validity information is only applicable to 3GPP access.

* * * End of Changes * * * *