3GPP TSG-CT WG1 Meeting #157	C1-256424

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 7047 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session inactive handling for DL TRANSPORT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE can receive PDU session identity in 5GSM message header as well (e.g., DL NAS TRANSPORT message) for sending CIoT user data (as seen in below table). Table 8.2.11.1.1: DL NAS TRANSPORT message content IEI; Information Element; Type/Reference. Extended protocol discriminator; Extended protocol discriminator 9.2. Security header type; Security header type 9.3. Spare half octet; Spare half octet 9.5. DL NAS TRANSPORT message identity; Message type 9.7. Payload container type; Payload container type 9.11.3.40. Spare half octet; Spare half octet 9.5. Payload container; Payload container 9.11.3.39. 12; PDU session ID; PDU session identity 2 9.11.3.41. 24; Additional information; Additional information 9.11.2.1. 58; 5GMM cause; 5GMM cause 9.11.3.2. 37; Back-off timer value; GPRS timer 3 9.11.2.5. 3A; Lower bound timer value; GPRS timer 3 9.11.2.5. #### 8.2.11.2 PDU session ID The AMF shall include this IE when the Payload container type IE is set to "N1 SM information" or "CIoT user data container". The current specification handles only 5GSM message when 5GSM messages are received and PDU session identity mentioned in 5GSM header is inactive in UE, but it does not handle for 5GMM message like DL NAS TRANSPORT message which header also contains PDU session identity value. Whenever such condition occur, UE handling is not specified which can result into unsync between UE and network where PDU is active at network and inactive at UE. |
| Summary of change: |  | If the UE receives a DL NAS TRANSPORT message for CIoT user data container which includes a PDU session identity belonging to any PDU session in state PDU SESSION INACTIVE in the UE, the UE shall respond with a 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network shall remain in unsync and network will keep sending CIoT user data for PDU which is inactive at UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.2	PDU Session identity

The following network procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message (specified as the header of a standard L3 message, see 3GPP TS 24.007 [11]):

a)	If the network receives a PDU SESSION MODIFICATION REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION MODIFICATION REJECT message including 5GSM cause #43 "invalid PDU session identity".

b)	If the network receives PDU SESSION RELEASE REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION RELEASE REJECT message including 5GSM cause #43 "invalid PDU session identity".

c)	Upon receipt of an UL NAS TRANSPORT message, the network takes the following actions:

1)	If the Request type IE is set to "initial request" or "initial emergency request" and the message includes a reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded";

2)	otherwise, if the message includes an unassigned or reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded".

d)	If the network receives a 5GSM message other than those listed in items a) through c) above in which the message includes a reserved PDU session identity value or an assigned value that does not match an existing PDU session, the network shall ignore the message.

The following UE procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message or in the header of DL NAS TRANSPORT message (see clause 5.4.5):

a)	If the UE receives a 5GSM message which includes an unassigned or reserved PDU session identity value, the UE shall ignore the message.

b)	If the UE receives a 5GSM message or DL NAS TRANSPORT message which includes a PDU session identity belonging to any PDU session in state PDU SESSION INACTIVE in the UE, the UE shall respond with a 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity".