3GPP TSG-CT WG1 Meeting #157	C1-256425

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 7048 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session inactive handling for UL TRANSPORT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5Gprotoc19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The network can receive PDU session identity in the header of 5GMM message as well e.g., UL NAS TRANSPORT message. (As seen in below table) Table 8.2.10.1.1: UL NAS TRANSPORT message content IEI; Information Element; Type/Reference. Extended protocol discriminator; Extended protocol discriminator 9.2. Security header type; Security header type 9.3. Spare half octet; Spare half octet 9.5. UL NAS TRANSPORT message identity; Message type 9.7. Payload container type; Payload container type 9.11.3.40. Spare half octet; Spare half octet 9.5. Payload container; Payload container 9.11.3.39. 12; PDU session ID; PDU session identity 2 9.11.3.41. 59; Old PDU session ID; PDU session identity 2 9.11.3.41. 8-; Request type; Request type 9.11.3.47. 22; S-NSSAI; S-NSSAI 9.11.2.8. 25; DNN; DNN 9.11.2.1B. 24; Additional information; Additional information 9.11.2.1. In curren specification, it is mentioned that when the message includes an unassigned or reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded", when the UE receives cause #90 in DL NAS TRANSPORT message for CIoT user data, then UE does not release PDU and keep retrying CIoT user data with same PDU session identity. But the UE can receive #90 in DL NAS TRANSPORT message due to other reasons as well e.g., routing failure as well. For case l1) in subclause 5.4.5.3.1, i.e. upon sending a single uplink CIoT user data container or control plane user data which was not forwarded due to routing failure, the AMF shall: a) include the PDU session ID in the PDU session ID IE; b) set the Payload container type IE to "CIoT user data container"; c) set the Payload container IE to the CIoT user data container or control plane user data which was not forwarded; and d) set the 5GMM cause IE to the 5GMM cause #90 "payload was not forwarded". In current specification, when network receives inactive PDU session identity, then the network send 5GSM reject message with cause #43 or 5GSM STATUS message with cause #43, but similar handling is not defined when PDU session identity is present in header of 5GMM message like UL NAS TRANSPORT message and network receives CIoT user data for inactive PDU which result into unsync between UE and network where PDU is active at UE, but inactive at network and the UE will keep sending CIoT user data for inactive PDU session identity value. When CIoT user data container is sent in Control plane service request message, then network indicates that this PDU session ID is inactive using the PDU session status IE so that UE and network PDU session is synchronized. The current specification miss handling for UL NAS TRANSPORT message for CIoT user data container. |
| Summary of change: |  | If the payload container type IE is set to "CIoT user data container" and the message includes a reserved PDU session identity value,the network shall respond with 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity"; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network PDU session status will remain unsync and the UE will keep sending UL NAS TRANSPORT message with CIoT user data container for PDU session identity which is inactive at network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.2	PDU Session identity

The following network procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message (specified as the header of a standard L3 message, see 3GPP TS 24.007 [11]) or in the header of UL NAS TRANSPORT message (see clause 5.4.5):

a)	If the network receives a PDU SESSION MODIFICATION REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION MODIFICATION REJECT message including 5GSM cause #43 "invalid PDU session identity".

b)	If the network receives PDU SESSION RELEASE REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION RELEASE REJECT message including 5GSM cause #43 "invalid PDU session identity".

c)	Upon receipt of an UL NAS TRANSPORT message, the network takes the following actions:

1)	If the Request type IE is set to "initial request" or "initial emergency request" and the message includes a reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded";

2)	If the payload container type IE is set to "CIoT user data container" and the message includes a reserved PDU session identity value, the network shall respond with 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity";

3)	otherwise, if the message includes an unassigned or reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded".

d)	If the network receives a 5GSM message other than those listed in items a) through c) above in which the message includes a reserved PDU session identity value or an assigned value that does not match an existing PDU session, the network shall ignore the message.

The following UE procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message:

a)	If the UE receives a 5GSM message which includes an unassigned or reserved PDU session identity value, the UE shall ignore the message.

b)	If the UE receives a 5GSM message which includes a PDU session identity belonging to any PDU session in state PDU SESSION INACTIVE in the UE, the UE shall respond with a 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity".