3GPP TSG-CT1 Meeting #157	C1-256436

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | 3359 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extension of Facility Information element for component size greater than 255 bytes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicion |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Facility information element is a type 4 element with a length field of 1 byte. The maximum size it can carry is 255 bytes. This has been the size of the IE from rel-8. For 2G, 3G and 4G this size seemed enough. But for 5G, the component can carry a much higher size and for that it is required to extend the Facility information element. The extended Facility information will be added in the CT4 specification TS 24.080 with the CR 0127. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new type 6 IE to carry facility information element greater than 255 bytes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to carry component greater than 255 bytes ending up in wrong encoding of Facility information element when the length is greater. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.4.36, K.3, K.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.080. CR .0127. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.5.4.36	Extended Facility

The purpose of the extended facility information element is to transport supplementary service related information, and it can transmit content with a length greater than 255 bytes but less than or equal to 65535 bytes. Within the scope of 3GPP TS 24.008 the content of the Extended Facility information field is an array of octets. The usage of this transportation mechanism is defined in 3GPP TS 24.080 [24].

The extended facility information element is coded as shown in figure 10.5.119/3GPP TS 24.008.

The extended facility is a type 6 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended Facility IEI |  |  |  |  |  |  |  | octet 1 |
| Length of extended facility contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Extended Facility information (see 3GPP TS 24.080 [24]) |  |  |  |  |  |  |  | octet 4-?* |

Figure 10.5.119/3GPP TS 24.008

* * * Next Change * * * *

# K.3	Mobility management information elements.

For the mobility management information elements listed below, the default coding of the information element identifier bits is summarized in table K.3/3GPP TS 24.008.

Table K.3/3GPP TS 24.008: Default information element identifier coding for mobility management information elements

|  |  |  |  |  |  |  |  |  |  | Reference clause |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  | Type 1 info elements |  |
| 1 | 0 | 0 | 1 | - | - | - | - |  | Note |  |
| 1 | 1 | 0 | 0 | - | - | - | - |  | Note |  |
| 1 | 1 | 1 | 0 | - | - | - | - |  | Note |  |
| 1 | 0 | 1 | 0 | - | - | - | - |  | Type 2 info elements |  |
|  |  |  |  | 0 | 0 | 0 | 1 |  | Follow-on Proceed | 10.5.3.7 |
|  |  |  |  | 0 | 0 | 1 | 0 |  | CTS Permission | 10.5.3.10 |
|  |  |  |  |  |  |  |  |  | Type 3 & 4 info elements |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Note |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Note |  |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Note |  |
| 1 | 1 | 1 | 1 | - 0 | - 0 | - 0 | - 0 |  | Type 6 info elements Note |  |
|  | All other values are reserved |  |  |  |  |  |  |  |  |  |

NOTE:	These values were allocated but never used in earlier versions of the protocol

* * * Next Change * * * *

# K.4	Call control information elements.

For the call control information elements listed below, the default coding of the information element identifiers is defined in table K.4/3GPP TS 24.008.

Table K.4/3GPP TS 24.008: Default information element identifier coding for call control information elements

|  |  |  |  |  |  |  |  |  |  | Reference clause |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| 1 | : | : | : | - | - | - | - |  | Type 1 info elements |  |
|  | 0 | 0 | 1 | - | - | - | - |  | shift | 10.5.4.2 and .3 |
|  | 0 | 1 | 1 | - | - | - | - |  | Note |  |
|  | 1 | 0 | 1 | - | - | - | - |  | Repeat indicator | 10.5.4.22 |
| 1 | 0 | 1 | 0 | : | : | : | : |  | Type 2 information elements |  |
|  |  |  |  | 0 | 0 | 0 | 0 |  | More data | 10.5.4.19 |
|  |  |  |  | 0 | 0 | 0 | 1 |  | CLIR Suppression | 10.5.4.11a |
|  |  |  |  | 0 | 0 | 1 | 0 |  | CLIR Invocation | 10.5.4.11b |
|  |  |  |  | 0 | 0 | 1 | 1 |  | Reverse call setup direction | 10.5.4.22a |
| 0 | : | : | : | : | : | : | : |  | Type 3 & 4 info elements |  |
|  | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | Bearer capability | 10.5.4.5 |
|  | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Cause | 10.5.4.11 |
|  | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | Note |  |
|  | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | Call Control Capabilities | 10.5.4.5a |
|  | 0 | 0 | 1 | 1 | 1 | 0 | 0 |  | Facility | 10.5.4.15 |
|  | 0 | 0 | 1 | 1 | 1 | 1 | 0 |  | Progress indicator | 10.5.4.21 |
|  | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | Auxiliary states | 10.5.4.4 |
|  | 0 | 1 | 0 | 0 | 1 | 1 | 1 |  | Note |  |
|  | 0 | 1 | 0 | 1 | 1 | 0 | 0 |  | Keypad facility | 10.5.4.17 |
|  | 0 | 1 | 1 | 0 | 1 | 0 | 0 |  | Signal | 10.5.4.23 |
|  | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | Connected number | 10.5.4.13 |
|  | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | Connected subaddress | 10.5.4.14 |
|  | 1 | 0 | 1 | 1 | 1 | 0 | 0 |  | Calling party BCD number | 10.5.4.9 |
|  | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  | Calling party subaddress | 10.5.4.10 |
|  | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  | Called party BCD number | 10.5.4.7 |
|  | 1 | 1 | 0 | 1 | 1 | 0 | 1 |  | Called party subaddress | 10.5.4.8 |
|  | 1 | 1 | 1 | 0 | 1 | 0 | 0 |  | Redirecting Party BCD | 10.5.4.21b |
|  | 1 | 1 | 1 | 1 | 1 | 0 | 1 |  | Redirecting Party subaddress | 10.5.4.21c |
|  | 1 | 1 | 1 | 1 | 1 | 0 | 0 |  | Low layer compatib. | 10.5.4.18 |
|  | 1 | 1 | 1 | 1 | 1 | 0 | 1 |  | High layer compatib. | 10.5.4.16 |
|  | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  | User-user | 10.5.4.25 |
| 1 | 1 1 | 1 1 | 1 1 | 1 : 0 | 1 : 0 | 1 : 0 | 1 : 1 |  | SS version indicator Type 6 info elements Extended Facility | 10.5.4.24 10.5.4.36 |

NOTE:	These values were allocated but never used in earlier phases of the protocol.

* * * End of Changes * * * *