3GPP TSG-CT WG1 Meeting #157	C1-256446

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4588 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | End of disaster condition during an emergency PDN connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE which is attached for disaster roaming might have an ongoing PDN connection for emergency services. The disaster condition may have ended while the emergency connection is ongoing. However, since the emergency connection is ongoing, the UE cannot be deattached immediately but at the same time the UE should not be getting disaster roaming service since the disaster condition is ended. In this case, the UE should be considered attached for emergency services only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the disaster condition has ended and the UE has on ongoing emergency PDN connection, the MME indicates to the UE that it is attached for emergency services only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE continues to use disaster roaming service although the disaster condition has ended which is not in accordance with the MINT_Ph2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12, 5.4.1.1, 5.4.1.2, 8.2.16, 9.9.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** first change *****

4.12	MINT-EPS for UE to obtain service in EPS under disaster condition

The UE and the network may support MINT-EPS.

If the UE supports MINT-EPS, the indication of whether disaster roaming in EPS is enabled in the UE, the one or more "list of PLMN(s) to be used in disaster condition", if available, are stored in the non-volatile memory in the ME as specified in annex C and are kept when the UE enters EMM-DEREGISTERED state. Annex C specifies the condition under which the indication of whether disaster roaming in EPS is enabled in the UE, the one or more "lists of PLMN(s) to be used in disaster condition", stored in the ME are deleted.

If the UE supports MINT-EPS, the UE shall indicate this capability to the network during the attach procedure or tracking area updating procedure. Upon selecting a PLMN for disaster roaming in EPS as specified in 3GPP TS 23.122 [6], the UE shall perform an attach procedure with the EPS attach type set to "disaster roaming attach", or a tracking area updating procedure with the EPS update type set to "disaster roaming update" for an inter-system change from 5GS to EPS, for disaster roaming services. When the UE is attached for disaster roaming services, the MME shall only include TAIs covering the area with the disaster condition.

Upon selecting a PLMN for disaster roaming as specified in 3GPP TS 23.122 [6]:

a)	if the UE does not have a stored disaster roaming wait range the UE shall perform an attach procedure for disaster roaming services on the selected PLMN as described in subclause 5.5.1; and

b)	if the UE has a stored disaster roaming wait range the UE shall generate a random number within the disaster roaming wait range and start a timer with the generated random number. While the timer is running, the UE shall not initiate attach on the selected PLMN except if the UE needs to request a PDN connection for emergency bearer services, in which case the UE shall initiate the attach procedure, set the EPS attach type IE to "EPS emergency attach" in the ATTACH REQUEST message and keep the timer running. Upon expiration of the timer, if the UE does not have a PDN connection for emergency bearer services, the UE shall perform an attach procedure for disaster roaming services as described in subclause 5.5.1 if still camped on the selected PLMN. If the UE has a PDN connection for emergency bearer services when the timer expires, the attach procedure for disaster roaming services as described in subclause 5.5.1 shall be performed after the release of the PDN connection for emergency bearer services, if the UE is still camped on the selected PLMN.

If the UE is switched off when the timer for disaster roaming wait range is running, the UE shall behave as follows when the UE is switched on, the USIM in the UE remains the same and the UE selects the PLMN for disaster roaming:

-	let t1 be the time remaining for the timer for disaster roaming wait range timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

Upon expiry of the timer for disaster roaming wait range, the UE shall not start a timer with the disaster roaming wait range value again until disaster condition has ended.

Upon determining that a disaster condition has ended as specified in 3GPP TS 23.122 [5]:

a)	the UE shall stop the timer started with a generated random number within the disaster roaming wait range, if running;

b)	the UE shall perform PLMN selection as specified in 3GPP TS 23.122 [5], except if the UE already selected an allowable PLMN as specified in 3GPP TS 23.122 [5]; and

c)	if the UE selects the UE determined PLMN with disaster condition and has a stored disaster return wait range, which is provided by the PLMN providing disaster roaming services or by the selected PLMN; and the UE shall generate a random number within the disaster return wait range and start a timer with the generated random number value. While the timer is running, the UE shall not initiate registration on the selected PLMN except if the UE needs to request an emergency PDU session, in which case the UE shall initiate the registration procedure, set the 5GS registration type IE to "emergency registration" in the REGISTRATION REQUEST message and keep the timer running. Upon expiration of the timer, if the UE does not have an emergency PDU session, the UE shall perform the registration procedure if still camped on the selected PLMN. If the UE has an emergency PDU session when the timer expires, the registration procedure shall be performed after the release of the emergency PDU session, if the UE is still camped on the selected PLMN.

Otherwise, the UE shall perform the registration procedure in the selected PLMN.

If the UE is switched off when the timer for disaster return wait range is running, the UE shall behave as follows when the UE is switched on, the USIM in the UE remains the same and the UE selects the UE determined PLMN with disaster condition:

-	let t1 be the time remaining for the timer for disaster return wait range timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

Upon expiry of the timer for disaster return wait range, the UE shall not start a timer with the disaster return wait range value again until the UE selects a PLMN for disaster roaming services.

When the MME assigns a tracking area to the UE attached for disaster roaming services, the MME shall only include TAIs covering the area with the disaster condition.

When the MME determines that the disaster condition has ended and the UE which is attached for disaster roaming services has an emergency PDN connection, the MME shall initiate the GUTI reallocation procedure to indicate that the UE is attached for emergency services as described in subclause 5.4.1.2.

If the UE is attached for disaster roaming services and the registered PLMN is removed from forbidden PLMN lists due to reasons specified in subclause 4.4.6 or Annex C in 3GPP TS 23.122 [6], then UE shall initiate the detach procedure and perform PLMN selection as specified in 3GPP TS 23.122 [6].

***** second change *****

5.4.1.1	General

The purpose of the GUTI reallocation procedure is to allocate a GUTI and optionally to provide one or more of the following to a particular UE:

-	a new TAI list;

-	a new DCN-ID;

-	in WB-S1 mode, if the UE supports RACS, either a UE radio capability ID deletion indication or a UE radio capability ID;

-	if the UE supports access technology utilization control, new access technology utilization control information; and

-	if the UE supports MINT-EPS, new EPS attach result information

The reallocation of a GUTI is performed by the unique procedure defined in this clause. This procedure can only be initiated by the MME in state EMM-REGISTERED.

The GUTI can also be implicitly reallocated at attach or tracking area updating procedures. The implicit reallocation of a GUTI is described in the clauses which specify these procedures (see clause 5.5.1 and 5.5.3).

The PLMN identity in the GUTI indicates the current registered PLMN.

NOTE 1:	The GUTI reallocation procedure is usually performed in ciphered mode.

NOTE 2:	Normally, the GUTI reallocation will take place in conjunction with another mobility management procedure, e.g. as part of tracking area updating.

***** third change *****

5.4.1.2	GUTI reallocation initiation by the network

The MME shall initiate the GUTI reallocation procedure by sending a GUTI REALLOCATION COMMAND message to the UE and starting the timer T3450 (see example in figure 5.4.1.2.1).

The GUTI REALLOCATION COMMAND message shall include a GUTI and may include one or more of the following:

-	a TAI list;

-	a DCN-ID;

-	in WB-S1 mode, if the UE supports RACS, either a UE radio capability ID deletion indication or a UE radio capability ID;

-	if the UE supports access technology utilization control, new access technology utilization control information; and

-	if the UE supports MINT-EPS, the EPS attach result information.

The MME can include the Access technology utilization control IE with the length of access technology utilization control contents field set to a non-zero value to update the access technology utilization control information. The MME can include the Access technology utilization control IE with the length of access technology utilization control contents field set to zero for the UE to remove the previously stored access technology utilization control information associated with the current PLMN, if any.

If the MME is initiating the GUTI reallocation procedure to indicate to a UE which is attached for disaster roaming services, and which has an ongoing emergency PDN connection, that the UE is attached for emergency services as described in subclause 4.12, the MME shall set the emergency attached bit of the EPS attach result IE to "Attached for emergency services" in the GUTI REALLOCATION COMMAND message.

NOTE:	As an implementation option, the type of access technology utilization control is not set to "current PLMN and its equivalent PLMN(s)" by the MME in the Access technology utilization control IE if the MME does not include a list of equivalent PLMNs in the last ATTACH ACCEPT or TRACKING AREA UPDATING ACCEPT message.

Figure 5.4.1.2.1: GUTI reallocation procedure

***** fourth change *****

8.2.16	GUTI reallocation command

8.2.16.1	Message definition

This message is sent by the network to the UE to reallocate a GUTI and optionally to provide a new TAI list or a new DCN-ID or both. See table 8.2.16.1.

Message type:	GUTI REALLOCATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.16.1: GUTI REALLOCATION COMMAND message content

| IEI |  | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  |  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  |  | GUTI reallocation command message identity | Message type 9.8 | M | V | 1 |
|  |  | GUTI | EPS mobile identity 9.9.3.12 | M | LV | 12 |
| 54 |  | TAI list | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 65 |  | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| 66 |  | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- |  | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| 20 | Access technology utilization control | Access technology utilization control 9.9.3.3A | O | TLV | 2-5 |
| xx | EPS attach result | EPS attach result 9.9.3.10 | O | TLV | 1 |

8.2.16.2	TAI list

This IE may be included to assign a TAI list to the UE.

8.2.16.3	DCN-ID

This IE may be included to assign a new DCN-ID to the UE.

8.2.16.4	UE radio capability ID

This IE may be included in WB-S1 mode if both the UE and the network support RACS and the network needs to assign a network-assigned UE radio capability ID to the UE.

8.2.16.5	UE radio capability ID deletion indication

This IE may be included in WB-S1 mode if both the UE and the network support RACS and the network needs to trigger the UE to delete all network-assigned UE radio capability IDs stored at the UE for the serving PLMN.

8.2.16.6	Access technology utilization control

This IE is included to indicate the restricted access technologies.

8.2.16.7	EPS attach result

This IE may be included to specify the result of an attach procedure.

***** fifth change *****

9.9.3.10	EPS attach result

The purpose of the EPS attach result information element is to specify the result of an attach procedure.

The EPS attach result information element is coded as shown in figure 9.9.3.10.1 and table 9.9.3.10.1.

The EPS attach result is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPS attach result IEI |  |  |  | Emergency attached | EPS attach result value |  |  | octet 1 |

Figure 9.9.3.10.1: EPS attach result information element

Table 9.9.3.10.1: EPS attach result information element

| EPS attach result value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | EPS only |
| 0 | 1 | 0 |  | combined EPS/IMSI attach |
| 0 | 1 | 1 |  | request for attach for disaster roaming service accepted as attach not for disaster roaming service |
| Emergency attached (octet 1, bit 4) Bit 4 |  |  |  |  |
| 0 Not attached for emergency services 1 Attached for emergency services |  |  |  |  |
| All other values are reserved. |  |  |  |  |

***** end change *****