3GPP TSG-WG CT1 Meeting #157	C1-256456

# Sophia-Antipolis, FR, 13th Oct – 17th Oct, 2025	 (revision of C1-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0105 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on data channel multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.228 clause AC.7.10.2.1 states that: - If the S-CSCF has not included the data channel multiplexing capability indication in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message, the UE shall not include the media feature tag indicating its capability of supporting data channel multiplexing in the Contact header field of the initial INVITE request or any subsequent re-INVITE request message. it means that only when the network supports data channel multiplexing, the UE could initate a multiplexed data channel estabilishment. It means the scenario in clause 9.3.2.18 of TS 24.186 does not exist. Upon receipt of the 18x or 2xx response including SDP answer to the initial INVITE, re-INVITE or UPDATE request, if the port number(s) of the multiplexed data channel media description(s) is set to zero and the "g.3gpp.dc-mux" header field parameter as specified in 3GPP TS 26.114 [4] is not included in the Contact header field, the UE may modify the SDP offer with de-multiplexed data channel media description and resend the corresponding request. If the multiplexed data channel is not rejected, it means that the terminating side does not want this media, the UE did not need to resend the request. This proposal is to solve this problem. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update procedure of data channel multiplexing on UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification for data channel multiplexing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

9.3.2.1.8	Support of data channel multiplexing

If a UE determines both the UE and its home IMS network support IMS data channel multiplexing capability, and wants to multiplex IMS data channels during IMS data channel establishment, the UE shall:

1)	follow the procedures of clause 9.3.2.1.2, clause 9.3.2.1.3 and clause 9.3.2.1.6.4 to generate an initial INVITE, a re-INVITE or a UPDATE request;

2)	include the "g.3gpp.dc-mux" media feature tag defined in 3GPP TS 26.114 [4] in the Contact header field; and

3)	include in the SDP offer the multiplexed data channel media description(s) according to 3GPP TS 26.114 [4], using:

a)	a single m line for both the local and remote bootstrap data channels; and

b)	a single m line for different application data channels for applications with compatible QoS requirements.

Upon receiving a re-INVITE request with an SDP offer, which contains multiplexed data channel media description(s), the procedure defined in clause 9.3.3.1.8 applies.

If a UE wants to close an established multiplexed application data channel during the session modification by sending re-INVITE request with the subsequent SDP offer, the procedure defined in clause 9.3.2.1.4 applies.

# * * * * End of changes * * * *