3GPP TSG-CT WG1 Meeting #157	C1-256463

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.368 | CR | 0088 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing value for periodic PLMN search timers while camping on the lower selection-priority PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LoSePLMN-CT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in the discussion paper in C1-256461, there can be some potential issues when a single timer value for periodic PLMN search timer is used for satellites from different orbits, when the UE is camping on a PLMN from the lower selection-priority PLMN selector. For example, when the UE is camping on a GEO based lower selection priority PLMN, short timer value for periodic PLMN search may increase the signalling as well as power consumption. In order to resolve such issues, we would like to propose to provide mutiple timer values for periodic PLMN search timer while camping on the lower selection-priority PLMN, one per orbits. So in this paper Note that this CR is based on the proposed changes in C1-255159 that was submitted by Vodafone to the CT1#156 meeting, as the only difference is the number of the timer values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In addition to the value for LowerSelectionPriorityPLMNPeriodicSearchTimer, the UE might configured for LowerSelectionPriorityPLMNPeriodicSearchTimer_LEO, LowerSelectionPriorityPLMNPeriodicSearchTimer_MEO and LowerSelectionPriorityPLMNPeriodicSearchTimer_GEO values for timer T while camping on a LEO-based PLMN, MEO-based PLMN and GEO-based PLMN respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Usage of single value for all orbits may cause some issues, e.g. power drain, increased signalling and network load, and bad user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.10x (new), 5.10xe (new), 5.10xf (new), 5.10xg (new), 5.10xh (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

# 4	NAS configuration MO

The NAS configuration MO is used to manage configuration parameters related to NAS functionality for a UE supporting provisioning of such information. The presence and format of the non-access stratum configuration file on the USIM is specified in 3GPP TS 31.102 [6].

The MO identifier is: urn:oma:mo:ext-3gpp-nas-config:1.0.

The OMA DM Access Control List (ACL) property mechanism (see OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the NAS configuration MO.

The following nodes and leaf objects are possible in the NAS configuration MO as described in figure 4-1:

Figure 4-1: The NAS configuration Management Object (1 of 6)

Figure 4-2: The NAS configuration Management Object (2 of 6)

Figure 4-3: The NAS configuration Management Object (3 of 6)

Figure 4-4: The NAS configuration Management Object (4 of 6)

Figure 4-5: The NAS configuration Management Object (5 of 6)

Figure 4-y: The NAS configuration Management Object (6 of 6)

***** Next change *****

## 5.10x	/<X>/Lower-selection-priority-PLMN

This interior node acts as a placeholder for one or more PLMNs configured to the UE for selection of a PLMN with lower selection priority as specified in 3GPP TS 23.122 [3].

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

***** Next change *****

## 5.10xe	/<X>/Lower-selection-priority-PLMN/LowerSelectionPriorityPLMNPeriodicSearchTimer

The LowerSelectionPriorityPLMNPeriodicSearchTimer leaf gives a value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs as specified in 3GPP TS 23.122 [3], when the UE is camping on a LEO-based PLMN.

-	Occurrence: ZeroOrOne

-	Format: Int

-	Access Types: Get, Replace

-	Values: 1-255

***** Next change *****

## 5.10xf	/<X>/Lower-selection-priority-PLMN/LowerSelectionPriorityPLMNPeriodicSearchTimer_LEO

The LowerSelectionPriorityPLMNPeriodicSearchTimer_LEO leaf gives a value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs as specified in 3GPP TS 23.122 [3] , when the UE is camping on a MEO-based PLMN.

-	Occurrence: ZeroOrOne

-	Format: Int

-	Access Types: Get, Replace

-	Values: 1-255

***** Next change *****

## 5.10xg	/<X>/Lower-selection-priority-PLMN/LowerSelectionPriorityPLMNPeriodicSearchTimer_MEO

The LowerSelectionPriorityPLMNPeriodicSearchTimer_MEO leaf gives a value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs as specified in 3GPP TS 23.122 [3] , when the UE is camping on a GEO-based PLMN.

-	Occurrence: ZeroOrOne

-	Format: Int

-	Access Types: Get, Replace

-	Values: 1-255

***** Next change *****

## 5.10xh	/<X>/Lower-selection-priority-PLMN/LowerSelectionPriorityPLMNPeriodicSearchTimer_GEO

The LowerSelectionPriorityPLMNPeriodicSearchTimer_GEO leaf gives a value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs as specified in 3GPP TS 23.122 [3].

-	Occurrence: ZeroOrOne

-	Format: Int

-	Access Types: Get, Replace

-	Values: 1-255

***** Change ends *****