3GPP TSG-CT WG1 Meeting #158	C1-257069

Dallas, US, 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 1044 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of encryption of <group-geo-area-ind> |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Firstnet, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc18 |  |  |  |  |  | Date: |  |  | 2025-10-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The <group-geo-area-ind> element is defined in the mcpttinfo XML schema. The element is identified as one element that can be encrypted within a SIP body. All elements that are marked as encryptable use the type “mcpttinfo:contentType”, however the <group-geo-area-ind> is defined with type “xs:boolean”. Elements of type “xs:boolean” can not be encrypted and at the same time be successfully validated against the current mcpttinfo xml schema. There are two possible solutions for this issue. - Change the xml schema so the element use “mcpttinfo:contentType”. This will break backward compatibility from Rel-15 for clients that use this feature. The possibility to encrypt this element was introduced in Rel-18. - Remove the possibility to encrypt the element <group-geo-area-ind> from Rel-18. This does not cause any backward compatible issues, since the encryption of this element does not currently work and the xml schema is kept as is. This CR proposes to remove the option to encrypt this element. The rationalities of that are the following: - The SIP traffic is anyway protected with IPSec between the UE and the SIP Proxy. The encryption of specific elements is used to protect sensitive information to the SIP core layer only. - The <group-geo-area-ind> is used in notification between the MCPTT server to the MCPTT client, to indicate that the client/user is entering (the element set to true) or leaving (the element set to false) a specific area. - The client shall according to the procedure in clause 12.1.1.4: - Affiliate to a group if the element is set to true - Deaffiliate to a group if the element is set to false. - The client’s action to affiliate or deaffiliate is not encrypted so there is no point in encrypting the message to the client when the client anyway does not encrypt what action it takes. - The sensitive part to encrypt is the group identity, which is already supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The <group-geo-area-ind> is removed from the list of encryptable elements in the semantic clause for the xml schema. Backward compatibility analysis This change does not cause any backward compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specified functionality to encrypt <group-geo-area-ind> does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## F.1.3	Semantic

The <mcpttinfo> element is the root element of the XML document. The <mcpttinfo> element can contain subelements.

NOTE 1:	The subelements of the <mcpttinfo> are validated by the <xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <mcpttinfo> element

If the <mcpttinfo> contains the <mcptt-Params> element then:

1)	the <mcptt-access-token>, <mcptt-request-uri>, <mcptt-calling-user-id>, <mcptt-called-party-id>, <mcptt-calling-group-id>, <emergency-ind>, <alert-ind>, <imminentperil-ind>, <originated-by>, <mcptt-client-id>, <functional-alias-URI>, <called-functional-alias-URI>, <non-acknowledged-user>, <multiple-devices-ind>, <associated-group-id>,  <migration-auth-result> , <partner-mcptt-id>, <primary-mcptt-id>, <selected-user-profile-index>, <req-type>, <resp-type> and <connected-functional-alias-URI> elements can be included with encrypted content;

2)	for each element in 1) that is included with content that is not encrypted:

a)	the element has the "type" attribute set to "Normal";

b)	if the element is one of the following elements: <mcptt-request-uri>, <mcptt-calling-user-id>, <mcptt-called-party-id>, <mcptt-calling-group-id>, <originated-by>, <functional-alias-URI>, <called-functional-alias-URI>, <non-acknowledged-user>, <associated-group-id>, <partner-mcptt-id>, <primary-mcptt-id>, or <connected-functional-alias-URI>, then the <mcpttURI> element is included;

c)	if the element is one of the following elements:<mcptt-access-token>, <mcptt-client-id>,  <req-type>, or <resp-type> then the <mcpttString> element is included;

d)	if the element is one of the following elements: <emergency-ind>, <alert-ind>, <alert-ind-rcvd>, <imminentperil-ind>, <emergency-ind-rcvd>, <multiple-devices-ind>, or <remotely-initiated-call-request-ind>, or <migration-auth-result>, then the <mcpttBoolean> element is included; and

e)	if the element is <selected-user-profile-index>, then the <user-profile-index> element is included; and

3)	for each element in 1) that is included with content that is encrypted:

a)	the element has the "type" attribute set to "Encrypted";

b)	the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE 2:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

If the <mcpttinfo> contains the <mcptt-Params> element then:

1)	the <mcptt-access-token> can be included with the access token received during authentication procedure as described in 3GPP TS 24.482 [49];

2)	the <session-type> can be included with:

a)	a value of "chat" to indicate that the MCPTT client wants to join a chat group call

b)	a value of "prearranged" to indicate the MCPTT client wants to make a prearranged group call;

c)	a value of "private" to indicate the MCPTT client wants to make a private call;

d)	a value of "first-to-answer" to indicate that the MCPTT client wants to make a first-to-answer call;

e)	a value of "ambient-listening" to indicate the MCPTT client wants to make an ambient listening call; or

f)	a value of "adhoc" to indicate the MCPTT client wants to make an adhoc group call;

3)	the <mcptt-request-uri> can be included with:

a)	a value set to an MCPTT group ID or temporary MCPTT group ID when the <session-type> is set to a value of "prearranged" or "chat";

b)	a value set to the MCPTT ID of the called MCPTT user when the <session-type> is set to a value of "private"; and

c)	a value set to the MCPTT ID of the called MCPTT user or MCPTT group ID of adhoc group when the <session-type> is set to a value of "adhoc";

4)	the <mcptt-calling-user-id> can be included, set to MCPTT ID of the originating user;

5)	the <mcptt-called-party-id> can be included, set to the MCPTT ID of the terminating user;

6)	the <mcptt-calling-group-id> can be included to indicate the MCPTT group identity or MCPTT adhoc group identity to the terminating user;

7)	the <required> can be included in a SIP 183 (Session Progress) from a non-controlling MCPTT function of an MCPTT group to inform the controlling MCPTT function that the group on the non-controlling MCPTT function has group members in the group document which are marked as <on-network-required>, as specified in 3GPP TS 24.481 [31];

8)	the <emergency-ind> can be:

a)	set to "true" to indicate that the call that the MCPTT client is initiating is an emergency MCPTT call; or

b)	set to "false" to indicate that the MCPTT client is cancelling an emergency MCPTT call (i.e. converting it back to a non-emergency call)

9)	the <alert-ind> can be:

a)	set to "true" in an emergency call initiation to indicate that an alert to be sent; or

b)	set to "false" when cancelling an emergency call which requires an alert to be cancelled also

10)	if the <session-type> is set to "chat" or "prearranged":

a)	the <imminentperil-ind> can be set to "true" to indicate that the call that the MCPTT client is initiating is an imminent peril group MCPTT call;

11)	the <broadcast-ind> can be:

a)	set to "true" to indicate that the MCPTT client is initiating a broadcast group call; or

b)	set to "false" to indicate that the MCPTT client is initiating a non-broadcast group call;

12)	the <mc-org> can be:

a)	set to the MCPTT user's Mission Critical Organization in an emergency alert sent by the MCPTT server to terminating MCPTT clients;

13)	the <floor-state> can be:

a)	set to "floor-idle" if the floor is idle in a non-controlling MCPTT function; or

b)	set to "floor-taken" if the floor state in a non-controlling MCPTT function is taken;

14)	the <associated-group-id>:

a)	if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID;

15)	the <originated-by>:

a)	can be included, set to the MCPTT ID of the originating user of an MCPTT emergency alert when being cancelled by another authorised MCPTT user;

16)	the <MKFC-GKTPs>:

a)	contains a group key transport payload carrying one or more MKFC(s) and MKFC-ID(s) as described in3GPP TS 24.481 [31] clause 7.4, to be used for protection of multicast floor control signalling when the UE operates on the network;

17)	the <mcptt-client-id>:

a)	can be included, set to the MCPTT client ID of the MCPTT client that originated a SIP INVITE request, SIP REFER request, SIP REGISTER request, SIP PUBLISH request or SIP MESSAGE request;

18)	the <alert-ind-rcvd>:

a)	can be set to "true" and included in a SIP MESSAGE to indicate that the emergency alert or cancellation was received successfully;

18a)	void;

18b)	void

18a)	the <gw-mcptt-usage>

a)	can be set to true in a SIP REGISTER or a SIP PUBLISH to indicate to the MCPTT server that the MCPTT client uses a MCPTT gateway UE, which requires that network resources are allocated over Rx, N5 or N33; and

19)	the <anyExt> can be included with the following elements:

a)	an <ambient-listening-type> element set to:

i)	"remote-init" when the listening MCPTT user of an ambient listening call initiates the call; or

ii)	"local-init" when the listened-to MCPTT user of an ambient listening call initiates the call;

b)	a <release-reason> element set to:

i)	"private-call-expiry" when the ambient listening call is release due to the expiry of the private call timer;

ii)	"administrator-action" when the ambient listening call is released by an MCPTT administrator;

iii)	"not selected for call" when the when a dialog is released with an MCPTT client that was not selected as the terminating client of a first-to-answer call;

iv)	"call-request-for-listened-to-client" when there is a call request targeted to the listened-to client;

v)	"call-request-initiated-by-listened-to-client" when there is a call request initiated by the listened-to client; or

vi)	"authentication of the MIKEY-SAKE I_MESSAGE failed" by a MCPTT client when the signature cannot be verified;

c)	a <request-type> element set to:

i)	"private-call-call-back-request" when a client initiates a private call call-back request;

ii)	"private-call-call-back-cancel-request" when a client initiates a private call call-back cancel request;

iii)	"group-selection-change-request" when a client initiates a group selection change request;

iv)	"remotely-initiated-group-call-request" when a client initiates a remotely initiated group call request;

v)	"remotely-initiated-private-call-request" when a client initiates a remotely initiated private call request;

vi)	"transfer-private-call-request" when a client initiates a transfer private call request;

vii)	"functional-alias-status-determination" when a client initiates a subscription to FA status determination request;

viii)	"forward-private-call-request" when a client initiates a forward private call request; or

ix)	"fa-group-binding-req" when a client initiates a request for binding of a functional alias with the MCPTT group(s) for the MCPTT user;

d)	a <response-type> element set to:

i)	"private-call-call-back-response" when a client responds to a private call call-back request;

ii)	"private-call-call-back-cancel-response" when a client responds to a private call call-back cancel request;

iii)	"group-selection-change-response" when a client responds to a group selection change request;

iv)	"remotely-initiated-group-call-response" when a client responds to a remotely initiated call request;

v)	"remotely-initiated-private-call-response" when a client responds to a remotely initiated private call request;

vi)	"transfer-private-call-response" when a client responds to a transfer private call request;

vii)	"forward-private-call-response" when a client responds to a forward private call request; or

e)	an <urgency-ind> element:

i)	set to a value of "low", "normal" or "high" to indicate the urgency of a private call call-back request;

f)	a <time-of-request> element :

i)	set to the date and time at which the private call call-back request was initiated, in the form: "YYYY-MM-DDThh:mm:ss" where:

-	YYYY indicates the year;

-	MM indicates the month;

-	DD indicates the day;

-	T indicates the start of the required time section;

-	hh indicates the hour;

-	mm indicates the minute; and

-	ss indicates the second;

g)	a <selected-group-change-outcome> element set to:

i)	"success" when a client reports that it has successfully changed its selected group as requested by a received group selection change request; or

ii)	"fail" when a client reports that it has failed to change its selected group as requested by a received group selection change request;

h)	an <affiliation-required> element set to:

i)	"true" when received by a client in a group-selection-change-request to indicate that the client needs to affiliate to the specified group;

i)	a <remotely-initiated-call-outcome> element set to:

i)	"success" when a client reports that it has successfully initiated a call requested by a received remotely initiated call request; or

ii)	"fail" when a client reports that it has failed to initiate a call triggered as requested by a received group selection change request;

j)	a <notify-remote-user> element set to:

i)	"true" when the remote user is to be notified of a remotely initiated call request; or

ii)	"false" when the remote user is to be notified of a received remotely initiated call request;

k)	a <functional-alias-URI> element set to the value of the functional alias that is used together with the "mcptt-calling-user-id";

l)	an <emergency-alert-area-ind> element set to:

i)	"true" when the MCPTT client has entered an emergency alert area; or

ii)	"false" when the MCPTT client has exited an emergency alert area;

m)	a <group-geo-area-ind> element set to:

i)	"true" when the MCPTT client has entered a group geographic area; or

ii)	"false" when the MCPTT client has exited a group geographic area;

n)	one or more <non-acknowledged-user> elements set to the MCPTT IDs of invited members to a group call that have not sent a SIP 200 (OK) response;

o)	a <call-to-functional-alias-ind> element set to:

i)	"true" when the MCPTT client is using a functional alias to identify the MCPTT IDs of the potential target MCPTT users; or

ii)	"false" when the MCPTT client is using MCPTT IDs to identify the potential target MCPTT users;

p)	the <emergency-ind-rcvd> element set to:

i)	"true" and included in a SIP MESSAGE to indicate that the in-progress emergency cancellation request was received successfully;

q)	a <call-transfer-ind> element set to:

i)	"true" when the MCPTT client is making a private call as a result of a call transfer; or

ii)	"false" when the MCPTT client is making a normal private call;

r)	a <transfer-call-outcome> element set to:

i)	"success" when a client reports that it has successfully initiated a call requested by a received call transfer request; or

ii)	"fail" when a client reports that it has failed to initiate a call triggered as requested by a received call transfer request;

s)	a <called-functional-alias-URI> element set to the value of the functional alias to be called;

t)	a <call-forwarding-ind> element set to:

i)	"true" when the MCPTT client is making a private call as a result of a call forwarding; or

ii)	"false" when the MCPTT client is making a normal private call;

u)	a <forwarding-call-outcome> element set to:

i)	"success" when a client reports that it has successfully initiated a call requested by a received call forwarding request; or

ii)	"fail" when a client reports that it has failed to initiate a call triggered as requested by a received call forwarding request;

v)	a <forwarding- immediate-list> element containing the details of the history of the immediate call forwardings that have occurred in the same MCPTT call;

w)	a <forwarding-other-list> element containing the details of the history of the forwardings because of call forwarding on "no-answer" or "manual-input" forthe same MCPTT call;

x)	a <forwarding-reason> element set to:

i)	"immediate" for call forwarding immediate;

ii)	"no-answer" for call forwarding no answer;

iii)	"manual-input" for call forwarding based on manual user input; or

iv)	"migrated" for call forwarding due to user migration;

y)	a <multiple-devices-ind> element set to:

i)	"true" to indicate to the client that multiple clients are registered for the MCPTT user; or

ii)	"false" to indicate to the client that no other clients are registered for the MCPTT user;

z)	a <binding-ind> element set to:

i)	"true" when the user wants to create a binding of a particular functional alias with the specified list of MCPTT groups for the MCPTT client; or

ii)	"false" when the user wants to remove a binding of a particular functional alias from the specified list of MCPTT groups for the MCPTT client;

aa)	a <binding-fa-uri> element set to:

i)	a URI of a functional alias that shall be bound with the specified list of MCPTT groups for the MCPTT client;

ab)	a <unbinding-fa-uri> element set to:

i)	a URI of a functional alias that shall be unbound from the specified list of MCPTT groups for the MCPTT client;

ac)	a <transfer-announced-ind> set to:

i)	"true" indicating that the call is part of an announced MCPTT call transfer; or

ii)	"false" indicating that the call is not part of an announced MCPTT call transfer;

ad)	a<replaces-header-value> element set to the Call-ID SIP header field value, the from-tag, and the to-tag of the MCPTT private call to be transferred. The delimiter between the Call-ID, the from-tag, and the to-tag is the semicolon (;);

ae)	a <user-requested-priority> element set to the non-negative integer value requested by the user as priorityAbsence of the <emergency-ind>, <alert-ind> and <imminentperil-ind> in a SIP INVITE or a SIP REFER request indicates that the MCPTT client is initiating a non-emergency private call or non-emergency group call;

af)	a <forwarding-sequence-number> element set to the total number of forwardings including that forwarding attempt;

ag)	a <forwarded-by-mcptt-id> element set to the MCPTT ID of the MCPTT client to whom a private call has been placed and who has requested that the private call be forwarded either through settings at its participating MCPTT function or through manual user input.

ah)	a <forwarded-by-functional-alias> element set to the functional alias received by the MCPTT client to whom a private call has been placed and who has requested that the private call be forwarded either through settings at its participating MCPTT function or through manual user input, if a functional alias was used to call the client;

ai)	a <forwarding-target-id> element set to the uri of the target of the forwarding;

aj)	a<forwarding-target-is-functional-alias> element set to:

i)	"true" to indicate that the target of the forwarding is a functional alias; or

ii)	"false" to indicate that the target of the forwarding is a MCPTT ID;

ak)	a <forwarding-target-orig-id> element set to the target uri of the forwarding as read from the user profile or entered by the MCPTT user requesting the MCPTT private call forwarding.

al)	a<end-to-end-security> element set to:

i)	"true" to indicate to determine the preconfigured group from which security related information is used for secure adhoc group call; or

ii)	"false" to indicate that not to determine the preconfigured group from which security related information is used for secure adhoc group call;

am)	a <call-participants-criterias> element set to the criteria for determining the list of MCPTT users to be called in adhoc group call, or to be included in an adhoc group emergency alert. The delimiter between the criteria is the comma (,);

an)	a <preconfigured-group-id> element set to the preconfigured group identity of the preconfigured group from which security related materials can be used by the adhoc group call participants to communicate securely in the adhoc group session;

ao)	a <adhoc-grp-emg-alert-grp-ind> element set to:

i)	"true" to indicate that the identity of adhoc group used in the adhoc group call setup request is learned during an adhoc group emergency alert procedures;

ao1)	a <migration-auth-result> element set to:

i)	"true" to indicate that the MCPTT client is authorized to migrate; or

ii)	"false" to indicate that the MCPTT client is not authorized to migrate;

ao2)	a <partner-mcptt-id> element set to the MCPTT ID of a migrating user in the partner MCPTT system;

ap)	a <selected-user-profile-index> element set to the value contained in the "user-profile-index" attribute of the MCPTT user profile selected according to clause 4.2.2.1.2.3 of 3GPP TS 24.484 [50];

aq)	a <req-type> element set to:

i)	"mc-service-authorisation-notify-request" when a participating MCPTT function in the partner MCPTT system initiates a request to notify about the successful completion of MCPTT user service authorization after migrating to the partner MCPTT system;

ii)	"get-userlist-adhoc-group-call-request" when a controlling MCPTT function initiates a request to get userlist for adhoc group call from terminating participating MCPTT function;

iii)	"adhoc-group-call-add-participants-request" when a terminating participating MCPTT function initiates a request to add user to adhoc group call notification for controlling MCPTT function;

iv)	"adhoc-group-call-remove-participants-request" when a terminating participating MCPTT function initiates a request to remove user from adhoc group call notification for controlling MCPTT function;

v)	"adhoc-group-call-release-notification-request" when a controlling MCPTT function initiates a request to stop determining the participant list for terminating participating MCPTT function;

vi)	"migration-service-deauthorization-notification" when a participating MCPTT function in the primary MCPTT system initiates a request to notify that an MCPTT client that has been authorized for migration service in the partner MCPTT system is to be deauthorized;

vii)	"migration-service-authorization-request" when a participating MCPTT function in the partner MCPTT system sends a migration service authorization request;

viii)	"fa-user-status-determination" when a client initiates a subscription to functional alias user status determination request;

ix)	"ad-hoc-emergency-alert-participant-criteria-modification" when an MCPTT client sends a request to modify participant criteria for an MCPTT ad hoc emergency alert to a participating MCPTT function or the participating MCPTT function forwards the request to a controlling MCPTT function; or

x)	"emergency-remote-floor-request-trigger" when a requesting client sends a trigger to a remote user in emergency to request the floor;

ar)	void;

as)	a <resp-type> element set to:

i)	"get-userlist-adhoc-group-call-response" when a terminating participating MCPTT function responds to get userlist for adhoc group call request;

ii)	"migration-service-authorization-response" when a participating MCPTT function in the primary MCPTT system sends a migration service authorization response; or

iii)	"ad-hoc-emergency-alert-participant-criteria-modification" when a controlling MCPTT function sends a response to a request to modify participant criteria for an MCPTT ad hoc emergency alert to a participating MCPTT function or the participating MCPTT function forwards the response to an MCPTT client;

at)	a <primary-mcptt-id> element set to the MCPTT ID of the user in the primary MCPTT system;

au)	void

av)	a <adhoc-alert-participant-list> element containing the total list of MC service users who belong to the ad hoc group irrespective if they have acknowledged the ad hoc group emergency alert or not;

aw)	a <adhoc-alert-participant-not-ack-list> element containing the list of MC service users who belong to the ad hoc group but have not acknowledged the ad hoc group emergency alert;

ax)	a <adhoc-emergency-ind> element set to:

i)	set to "true" to indicate that the alert or call that the MCPTT client is initiating is an emergency MCPTT alert or call; or

ii)	set to "false" to indicate that the MCPTT client is cancelling an emergency MCPTT alert or call (i.e. converting it back to a non-emergency call);

ay)	a <adhoc-alert-ind> element set to:

i)	"true" in an adhoc group emergency alert initiation to indicate that an alert to be sent; or

ii)	"false" in an adhoc group emergency alert cancellation to indicate that an alert to be cancelled;

az)	<adhoc-alert-ind-rcvd> element:

i)	can be set to "true" and included in a SIP MESSAGE to indicate that the adhoc group emergency alert or cancellation was received successfully;

ba)	<adhoc-emergency-ind-rcvd> element set to:

i)	"true" and included in a SIP MESSAGE to indicate that the in-progress adhoc emergency cancellation request was received successfully;

bb)	an <adhoc-additional-information> element set to text information included with the adhoc group emergency alert.

bc)	a <location-of-functional-alias-URI> element set to a URI of a functional alias that is used to identify the MCPTT clients for which location information is being requested; and

bd)	a <connected-functional-alias-URI> element set to the value of the functional alias of the final destination returned by the called MCPTT client in an ad hoc group call.

Absence of the <broadcast-ind> in a SIP INVITE or a SIP REFER request indicates that the MCPTT client is initiating a non-broadcast group call.

Absence of the <floor-state> in a SIP 200 (OK) response from the non-controlling MCPTT function indicates that the floor is idle.

Absence of the <call-to-functional-alias-ind> in a SIP INVITE or a SIP REFER request for a first-to-answer call indicates the use of the MCPTT IDs of the potential target MCPTT users.

Absence of the <call-transfer-ind> in a SIP INVITE or a SIP REFER request for a private call indicates that the call is not caused by a request for call transfer.

Absence of the <call-forwarding-ind> in a SIP INVITE or a SIP REFER request for a private call indicates that the call is not caused by a request for call forwarding.

Absence of the <transfer-announced-ind> in a SIP INVITE or a SIP REFER request for a private call indicates that the call is not part of an announced call transfer.

Absence of the <forwarding-target-is-functional-alias> element in a SIP INVITE, a SIP REFER, or a SIP MESSAGE request indicates that the forwarding target contains an MCPTT ID.

The recipient of the XML ignores any unknown element and any unknown attribute.

* * * End of Change * * *