3GPP TSG-CT WG1 Meeting #158	C1-257098

# Dallas, US , 17-21 November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.555 | CR | 0094 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for octets numbering in the encoding of multiple fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3, 5G_ProSe_Ph2, 5G_ProSe |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Multiple multi-hop fields as well as other fields have wrong octets numbering indicated in the information elements coding of the UE policies of 5G ProSe. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the wrong octets numbering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong specification stays which may lead to wrong implementation due to the wrong octets numbering. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 5.6.2, 5.11.2, 5.12.2, 5.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 5.3.2	Information elements coding

| 8 | 7 |  | 6 |  | 5 | 4 |  |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 0 |  | 0 | ProSeP info type = {UE policies for 5G ProSe direct discovery} |  |  |  |  |  | octet k |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  |  |  |  |  | octet k+1 octet k+2 |
| Validity timer |  |  |  |  |  |  |  |  |  |  |  | octet k+3 octet k+7 |
| Served by NG-RAN |  |  |  |  |  |  |  |  |  |  |  | octet k+8 octet o1 |
| Not served by NG-RAN |  |  |  |  |  |  |  |  |  |  |  | octet o1+1 octet o2 |
| ProSe direct discovery UE ID |  |  |  |  |  |  |  |  |  |  |  | octet o2+1 octet o2+3 |
| Group member discovery parameters |  |  |  |  |  |  |  |  |  |  |  | octet o2+4 octet o3 |
| ProSe identifiers |  |  |  |  |  |  |  |  |  |  |  | octet o3+1 octet o4 |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules |  |  |  |  |  |  |  |  |  |  |  | octet o4+1 octet l |
| 0 Spare | 0 Spare | 0 Spare |  | 0 Spare |  |  | 0 Spare | H5DAI |  |  |  | octet l+1 |
| HPLMN 5G DDNMF address information |  |  |  |  |  |  |  |  |  |  |  | octet (l+2)* octet m* |

Figure 5.3.2.1: ProSeP Info = {UE policies for 5G ProSe direct discovery}

Table 5.3.2.1: ProSeP Info = {UE policies for 5G ProSe direct discovery}

| ProSeP info type (bit 1 to 4 of octet k) shall be set to "0001" (UE policies for 5G ProSe direct discovery) |  |  |  |
| --- | --- | --- | --- |
| Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents. |  |  |  |
| Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe direct discovery. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |  |
| Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.3.2.2 and table 5.3.2.2, and contains configuration parameters for 5G ProSe direct discovery when the UE is served by NG-RAN. |  |  |  |
| Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.3.2.6 and table 5.3.2.6, and contains configuration parameters for 5G ProSe direct discovery when the UE is not served by NG-RAN. |  |  |  |
| ProSe Direct Discovery UE ID (octet o2+1 to o2+3): The ProSe Direct Discovery UE ID is a 24-bit long bit string. |  |  |  |
| Group member discovery parameters (octet o2+4 to o3): The group member discovery parameters field is coded according to figure 5.3.2.12 and table 5.3.2.12 and contains group member discovery parameters. |  |  |  |
| ProSe identifiers (octet o3+1 to o4): The ProSe identifiers field is coded according to figure 5.3.2.14 and table 5.3.2.14 and contains ProSe identifiers. |  |  |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules (octet o4+1 to l) (NOTE 2): The ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules field is coded according to figure 5.3.2.15 and table 5.3.2.15 and contains ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules. The ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules field may contain a default ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule for the ProSe services that do not have dedicated mapping rules. |  |  |  |
| HPLMN 5G DDNMF address information indicator (H5DAI) (octet l+1 bit 1 to bit 3): (NOTE 1) Bits |  |  |  |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | HPLMN 5G DDNMF address information is absent |
| 0 | 0 | 1 | HPLMN 5G DDNMF FQDN is present |
| 0 | 1 | 0 | HPLMN 5G DDNMF IPv4 address is present |
| 1 | 0 | 0 | HPLMN 5G DDNMF IPv6 address is present |
| 1 | 1 | 0 | HPLMN 5G DDNMF IPv4 address and IPv6 address are present |
| All other values are reserved. |  |  |  |
| HLMN 5G DDNMF address information (octet l+2 to octet m): The HPLMN 5G DDNMF address information field is coded according to figure 5.3.2.17 and table 5.3.2.17 and contains the 5G DDNMF address information in HPLMN. If the length of ProSeP info contents field is bigger than indicated in figure 5.3.2.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents |  |  |  |
| NOTE 1: For backward compatibility with UEs compliant to earlier versions of present document, H5DAI values 011, 101 and 111 cannot be used |  |  |  |  |
| NOTE 2: This field is prioritized in decreasing order according to the local configuration of the network. The default mapping rule for the ProSe services that do not have dedicated mapping rules, if present, is recommended to be the last one and with the lowest priority of this field. |  |  |  |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| Authorization for direct discovery info 1 |  |  |  |  |  |  |  | octet (k+10)* octet o50* |
| Authorization for direct discovery info 2 |  |  |  |  |  |  |  | octet (o50+1)* octet o51* |
| … |  |  |  |  |  |  |  | octet (o51+1)* octet o52* |
| Authorization for direct discovery info n |  |  |  |  |  |  |  | octet (o52+1)* octet o1* |

Figure 5.3.2.2: Served by NG-RAN

Table 5.3.2.2: Served by NG-RAN

Authorization for direct discovery info: The authorization for direct discovery info field is coded according to figure 5.3.2.3 and table 5.3.2.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorization for direct discovery info contents |  |  |  |  |  |  |  | octet o50+1 octet o50+2 |
| 0 spare | 0 spare | 0 spare | 0 spare | SNAI | Role | Model | DDT | octet o50+3 |
| Authorized PLMN info |  |  |  |  |  |  |  | octet o50+4 octet o51 |

Figure 5.3.2.3: Authorization for direct discovery info

Table 5.3.2.3: Authorization for direct discovery info

| Direct discovery type (DDT) (octet o50+3 bit 1): Bit 1 0 Open 1 Restricted |  |
| --- | --- |
| Model (octet o50+3 bit 2): Bit 2 0 A 1 B |  |
| If Model bit is set to "A", Role (octet o50+3 bit 3): Bit 3 0 Announcing 1 Monitoring |  |
| If Model bit is set to "B", Role (octet o50+3 bit 3): Bit 3 0 Discoverer 1 Discoveree |  |
| Authorized PLMN info (octet o50+4 to o51): The authorized PLMN info field is coded according to figure 5.3.2.4 and table 5.3.2.4. |  |
| Subscribed SNPN authorization indicator (SNAI) (octet o50+3 bit 4) |  |
| Bit |  |
| 4 |  |
| 0 | Subscribed SNPN is not authorized |
| 1 | Subscribed SNPN is authorized |
| If the length of authorization for direct discovery info field is bigger than indicated in figure 5.3.2.3, receiving entity shall ignore any superfluous octets located at the end of the authorization for direct discovery info. |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorized PLMN info contents |  |  |  |  |  |  |  | octet o50+4 octet o50+5 |
| Authorized PLMN 1 |  |  |  |  |  |  |  | octet (o50+6)* octet (o50+8)* |
| Authorized PLMN 2 |  |  |  |  |  |  |  | octet (o50+9)* octet (o50+11)* |
| ... |  |  |  |  |  |  |  | octet (o50+12)* octet o150* |
| Authorized PLMN n |  |  |  |  |  |  |  | octet (o150+1)* octet o51* |

Figure 5.3.2.4: Authorized PLMN info

Table 5.3.2.4: Authorized PLMN

Authorized PLMN: The authorized PLMN field is coded according to figure 5.3.2.5 and table 5.3.2.5.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet o50+6 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet o50+7 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet o50+8 |

Figure 5.3.2.5: PLMN ID

Table 5.3.2.5: PLMN ID

Mobile country code (MCC) (octet o50+5, octet o50+6 bit 1 to 4): The MCC field is coded as in ITU-T Recommendation E.212 [5], annex A.

Mobile network code (MNC) (octet o50+6 bit 5 to 8, octet o50+7): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111".

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | PDNNI | octet o1+3 |
| NR radio parameters per geographical area list |  |  |  |  |  |  |  | octet (o1+4)* octet o10* |
| Default PC5 DRX configuration |  |  |  |  |  |  |  | octet (o10+1)* octet o2* |

Figure 5.3.2.6: Not served by NG-RAN

Table 5.3.2.6: Not served by NG-RAN

5G ProSe direct discovery when not served by NG-RAN indicator (PDNNI) (octet o1+3 bit 1): The PDNNI bit indicates whether the UE is authorized to perform 5G ProSe direct discovery when not served by NG-RAN. Bit 1 0 Not authorized 1 Authorized

NR radio parameters per geographical area list (octet o1+4 to o2): If PNNI bit is set to "Authorized", the NR radio parameters per geographical area list field is present otherwise the NR radio parameters per geographical area list field is absent. It is coded according to figure 5.3.2.7 and table 5.3.2.7. Default PC5 DRX configuration (octet o10+1 to o2): If PDNNI bit is set to "Authorized", the default PC5 DRX configuration is present otherwise the default PC5 DRX configuration is absent. It is coded according to figure 5.3.2.11a and table 5.3.2.11a.

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.3.2.6, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area list contents |  |  |  |  |  |  |  | octet o1+4 octet o1+5 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet (o1+6)* octet o6* |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o6+1)* octet o7* |
| ... |  |  |  |  |  |  |  | octet (o7+1)* octet o8* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o8+1)* octet o10* |

Figure 5.3.2.7: Radio parameters per geographical area list

Table 5.3.2.7: Radio parameters per geographical area list

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.3.2.8 and table 5.3.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o6+1 octet o6+2 |
| Geographical area |  |  |  |  |  |  |  | octet o6+3 octet o9 |
| Radio parameters |  |  |  |  |  |  |  | octet o9+1 octet o7-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o7 |

Figure 5.3.2.8: Radio parameters per geographical area info

Table 5.3.2.8: Radio parameters per geographical area info

Geographical area (octet o6+3 to o9): The geographical area field is coded according to figure 5.3.2.9 and table 5.3.2.9.

Radio parameters (octet o9 to o7-1): The radio parameters field is coded according to figure 5.3.2.11 and table 5.3.2.11, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o7 bit 8): The managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.3.2.8, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o6+3 octet o6+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o6+5)* octet (o6+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o6+11)* octet (o6+16)* |
| ... |  |  |  |  |  |  |  | octet (o6+17)* octet (o6-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o6-1+6*n)* octet (o6+4+6*n)* = octet o9* |

Figure 5.3.2.9: Geographical area

Table 5.3.2.9: Geographical area

Coordinate: The coordinate field is coded according to figure 5.3.2.10 and table 5.3.2.10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o6+11 octet o6+13 |
| Longitude |  |  |  |  |  |  |  | octet o6+14 octet o6+16 |

Figure 5.3.2.10: Coordinate area

Table 5.3.2.10: Coordinate area

Latitude: The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude: The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o9+1 octet o9+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o9+3 octet o7-1 |

Figure 5.3.2.11: Radio parameters

Table 5.3.2.11: Radio parameters

Radio parameters contents: Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default PC5 DRX configuration contents |  |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |  |
| Default PC5 DRX configuration contents |  |  |  |  |  |  |  |  | octet o10+3 octet o2 |  |

Figure 5.3.2.11a: Default PC5 DRX configuration

Table 5.3.2.11a: Default PC5 DRX configuration

Default PC5 DRX configuration contents: The default PC5 DRX configuration field is coded as sl-DefaultDRX-GC-BC-r17 in clause 6.3.5 of 3GPP TS 38.331 [7].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of group member discovery parameters contents |  |  |  |  |  |  |  |  | octet o2+4 octet o2+5 |  |
| Application layer group info 1 |  |  |  |  |  |  |  |  | octet (o2+6)* octet o51* |  |
| Application layer group info 2 |  |  |  |  |  |  |  |  | octet (o51+1)* octet o52* |  |
| … |  |  |  |  |  |  |  |  | octet (o52+1)* octet o53* |  |
| Application layer group info n |  |  |  |  |  |  |  |  | octet (o53+1)* octet o3* |  |

Figure 5.3.2.12: Group member discovery parameters

Table 5.3.2.12: Group member discovery parameters

Application layer group info: The application layer group info field is coded according to figure 5.3.2.13 and table 5.3.2.13.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of application layer group info contents |  |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |  |
| Application layer group identifier |  |  |  |  |  |  |  |  | octet o51+3 octet o151 |  |
| ProSe layer-2 group identifier |  |  |  |  |  |  |  |  | octet o151+1 octet o151+3 |  |
| User info ID |  |  |  |  |  |  |  |  | octet o151+4 octet (o151+9) = octet o52 |  |

Figure 5.3.2.13: Application layer group info

Table 5.3.2.13: Application layer group info

Application layer group identifier (octet o51+3 to o151): The first octet of application layer group identifier field is the length of application group identifier. The value of application group identifier field is a bit string. The format of application group identifier parameter is out of scope of this specification.

ProSe layer-2 group identifier (octet o151+1 to o151+3)

The ProSe layer-2 group identifier field is a binary coded layer-2 identifier.

User info ID (octet o151+4 to o52)

The value of the User info ID parameter is a 48-bit long bit string. The format of the User info ID parameter is out of scope of this specification.

If the length of application layer group info contents field is bigger than indicated in figure 5.3.2.13, receiving entity shall ignore any superfluous octets located at the end of the application layer group info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe identifiers contents |  |  |  |  |  |  |  | octet o3+1 octet o3+2 |
| ProSe identifier 1 |  |  |  |  |  |  |  | octet (o3+3)* octet o31* |
| ProSe identifier 2 |  |  |  |  |  |  |  | octet (o31+1)* octet o32* |
| ... |  |  |  |  |  |  |  | octet (o32+1)* octet o33* |
| ProSe identifier n |  |  |  |  |  |  |  | octet (o33+1)* octet o34* = octet o4* |

Figure 5.3.2.14: ProSe identifiers

Table 5.3.2.14: ProSe identifiers

ProSe identifier (NOTE 1, NOTE 2): The ProSe identifier field contains a sequence of a sixteen octet OS Id field, a one octet OS App Id length field, and an OS App Id field. The OS Id field shall be transmitted first. The OS Id field contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 9562 [12].

NOTE 1: Further definition of the format of OS App ID is beyond the scope of this specification.

NOTE 2: The default ProSe identifier for the ProSe services that do not have dedicated configurations is coded as a sequence of a seventeen octet of "0". The default ProSe identifier matches all the ProSe services.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules contents |  |  |  |  |  |  |  |  | octet o4+1 octet o4+2 |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule 1 |  |  |  |  |  |  |  |  | octet (o4+3)* octet o54* |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule 2 |  |  |  |  |  |  |  |  | octet (o54+1)* octet o55* |  |
| ... |  |  |  |  |  |  |  |  | octet (o55+1)* octet o56* |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule n |  |  |  |  |  |  |  |  | octet (o56+1)* octet l* |  |

Figure 5.3.2.15: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules

Table 5.3.2.15: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules

ProSe identifier to destination layer-2 ID for initial discovery signalling mapping rule: The ProSe identifier to destination layer-2 ID for initial discovery signalling mapping rule field is coded according to figure 5.3.2.16 and table 5.3.2.16.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule contents |  |  |  |  |  |  |  |  | octet o54+1 octet o54+2 |  |
| ProSe identifiers |  |  |  |  |  |  |  |  | octet o54+3 octet o154 |  |
| Destination layer-2 ID for initial discovery signalling |  |  |  |  |  |  |  |  | octet o154+1 octet (o154+3) = octet o55 |  |

Figure 5.3.2.16: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule

Table 5.3.2.16: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule

ProSe identifiers (octet o54+3 to o154): The ProSe identifiers field is coded according to figure 5.3.2.14 and table 5.3.2.14. In case of the default ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule, the ProSe identifier is coded as the default ProSe identifier (see table 5.4.2.14).

Destination layer-2 ID for initial discovery signalling (octet o154+1 to o55): The destination layer-2 ID for initial discovery signalling field is a binary coded layer-2 identifier.

If the length of ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule contents field is bigger than indicated in figure 5.3.2.16, receiving entity shall ignore any superfluous octets located at the end of the ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of HPLMN 5G DDNMF address information contents |  |  |  |  |  |  |  | octet l+2 |
| HPLMN 5G DDNMF address information contents |  |  |  |  |  |  |  | octet l+3 octet m |

Figure 5.3.2.17: HPLMN 5G DDNMF address information

Table 5.3.2.17: HPLMN 5G DDNMF address information

Length of HPLMN 5G DDNMF address information (octet l+2): When the H5DAI is set to "HPLMN 5G DDNMF FQDN is present", the value of the length is the length of the HPLMN 5G DDNMF FQDN. When the H5DAI is set to "HPLMN 5G DDNMF IPv4 address is present", the value of the length is 4. When the H5DAI is set to "HPLMN 5G DDNMF IPv6 address is present", the value of the length is 16. When the H5DAI is set to "HPLMN 5G DDNMF IPv4 address and IPv6 address are present", the value of the length is 20. HPLMN 5G DDNMF address information contents (octet l+3 to octet m): When the H5DAI is set to "HPLMN 5G DDNMF FQDN is present", HPLMN 5G DDNMF address information filed contains the HPLMN 5G DDNMF FQDN and shall be coded as defined in clause 28.3.2.11 in 3GPP TS 23.003 [10]. When the H5DAI is to "HPLMN 5G DDNMF IPv4 address is present", HPLMN 5G DDNMF address information filed contains an IPv4 address in 4 octets. When the H5DAI is set to "HPLMN 5G DDNMF IPv6 address is present", HPLMN 5G DDNMF address information filed contains an IPv6 address in 16 octets. When the H5DAI is set to "HPLMN 5G DDNMF IPv4 address and IPv6 address are present", HPLMN 5G DDNMF address information filed contains a sequence of an IPv4 address in 4 octets and an IPv6 address in 16 octets.

==============Next change==============

### 5.6.2	Information elements coding

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | NSII | PAI | ProSeP info type = {UE policies for 5G ProSe remote UE} |  |  |  | octet k |
| Spare |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  | octet k+1 octet k+2 |
| Validity timer |  |  |  |  |  |  |  | octet k+3 octet k+7 |
| Served by NG-RAN |  |  |  |  |  |  |  | octet k+8 octet o1 |
| Not served by NG-RAN |  |  |  |  |  |  |  | octet o1+1 octet o2 |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information |  |  |  |  |  |  |  | octet o2+1 octet o3 |
| User info ID for discovery |  |  |  |  |  |  |  | octet o3+1 octet o3+6 |
| RSC info list |  |  |  |  |  |  |  | octet o3+7 octet l |
| Privacy timer |  |  |  |  |  |  |  | octet l+1 octet l+2 |
| N3IWF selection information for 5G ProSe layer-3 remote UE |  |  |  |  |  |  |  | octet (l+3)* octet m* |
| 5G PKMF address information |  |  |  |  |  |  |  | octet q* (see NOTE) octet p* |
| Warning message broadcast |  |  |  |  |  |  |  | octet a octet b |

NOTE:	The field is placed immediately after the last present preceding field.

Figure 5.6.2.1: ProSeP Info = {UE policies for 5G ProSe remote UE}

Table 5.6.2.1: ProSeP Info = {UE policies for 5G ProSe remote UE}

ProSeP info type (bit 1 to 4 of octet k) shall be set to "0100" (UE policies for 5G ProSe remote UE)

PKMF address indication (PAI) (bit 5 of octet k)

The PAI indicates whether the 5G PKMF address information is included in the IE or not

Bit

5

0 5G PKMF address information is not included

1 5G PKMF address information is included N3IWF selection information indication (NSII) (bit 6 of octet k) The NSII indicates whether the N3IWF selection information for 5G ProSe layer-3 remote UE is included in the IE or not Bit 6 0 N3IWF selection information for 5G ProSe layer-3 remote UE is not included 1 N3IWF selection information for 5G ProSe layer-3 remote UE is included

Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents.

Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe remote UE. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.6.2.2 and table 5.6.2.2, and contains configuration parameters for 5G ProSe remote UE when the UE is served by NG-RAN.

Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.6.2.5 and table 5.6.2.5, and contains configuration parameters for 5G ProSe UE-to-network relay discovery and communication when the UE is not served by NG-RAN.

Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information (octet o2+1 to o3): The default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information is coded according to figure 5.6.2.11b and table 5.6.2.11b and contains a list of the default destination layer-2 IDs for the initial UE-to-network relay discovery signalling.

User info ID for discovery (octet o3+1 to o3+6): The value of the User info ID parameter is a 48-bit long bit string. The format of the User info ID parameter is out of scope of this specification.

RSC info list (octet o3+7 to l): The RSC info list field is coded according to figure 5.6.2.12 and table 5.6.2.12 and contains the RSCs related parameters.

Privacy timer (octet l+1 to +2): The privacy timer field contains binary encoded duration, in units of seconds, after which the UE shall change the source layer-2 ID self-assigned by the UE while performing transmission of 5G ProSe direct communication. N3IWF selection information for 5G ProSe layer-3 remote UE (octet l+3 to m): The N3IWF selection information for 5G ProSe layer-3 remote UE field is coded according to figure 5.6.2.17 and table 5.6.2.17, and contains two parts: 1) N3IWF identifier configuration (either FQDN or IP address) for 5G ProSe layer-3 remote UE; 2) 5G ProSe layer-3 UE-to-network relay access node selection information.

5G PKMF address information (octet q to p) The 5G PKMF address information contains the IPv4 address(es), IPv6 address(es) and/or FQDN of the 5G PKMF and is coded according to figure 5.5.2.21, figure 5.5.2.22, figure 5.5.2.23 and table 5.5.2.21. At least one of the address parameters (FQDN, IPv4 address list or IPv6 address list) shall be included.

Warning message broadcast (octet a to b): The warning message broadcast filed is coded according to figure 5.6.2.20 and table 5.6.2.20 and contains the parameters for broadcasting the warning messages.

If the length of ProSeP info contents field is bigger than indicated in figure 5.6.2.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | SNAI | L3RI | octet k+10 |
| Authorized PLMN list for layer-2 remote UE |  |  |  |  |  |  |  | octet k+11 octet o1 |

Figure 5.6.2.2: Served by NG-RAN

Table 5.6.2.2: Served by NG-RAN

| Layer-3 remote UE authorization indication (L3RI) (octet k+10, bit 1): The layer-3 remote UE authorization indication field indicates whether the UE is authorized to act as a layer-3 remote UE. Bits 1 0 Not authorized to act as a layer-3 remote UE 1 Authorized to act as a layer-3 remote UE |  |
| --- | --- |
| Authorized PLMN list for layer-2 remote UE (octet k+11 to o1): The authorized PLMN list for layer-2 remote UE field is coded according to figure 5.6.2.3 and table 5.6.2.3. |  |
| Subscribed SNPN authorization indicator (SNAI) (octet k+10, bit 2) |  |
| Bit |  |
| 2 |  |
| 0 | Subscribed SNPN is not authorized |
| 1 | Subscribed SNPN is authorized |
| If the length of Served by NG-RAN field is bigger than indicated in figure 5.6.2.2, receiving entity shall ignore any superfluous octets located at the end of the Served by NG-RAN. |  |

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorized PLMN list contents |  |  |  |  |  |  |  |  | octet k+11 octet k+12 |  |
| Authorized PLMN 1 |  |  |  |  |  |  |  |  | octet (k+13)* octet (k+15)* |  |
| Authorized PLMN 2 |  |  |  |  |  |  |  |  | octet (k+16)* octet (k+18)* |  |
| ... |  |  |  |  |  |  |  |  | octet (k+19)* octet (o50-3)* |  |
| Authorized PLMN n |  |  |  |  |  |  |  |  | octet (o50-2)* octet o50* |  |

Figure 5.6.2.3: Authorized PLMN list

Table 5.6.2.3: Authorized PLMN list

Authorized PLMN: The authorized PLMN field is coded according to figure 5.6.2.4 and table 5.6.2.4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet k+16 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet k+17 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet k+18 |

Figure 5.6.2.4: PLMN ID

Table 5.6.2.4: PLMN ID

Mobile country code (MCC) (octet k+16, octet k+17 bit 1 to 4): The MCC field is coded as in ITU-T Recommendation E.212 [5], annex A.

Mobile network code (MNC) (octet k+17 bit 5 to 8, octet k+18): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111".

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| NR radio parameters per geographical area list for UE-to-network relay discovery |  |  |  |  |  |  |  | octet o1+3 octet o51 |
| NR radio parameters per geographical area list for UE-to-network relay communication |  |  |  |  |  |  |  | octet o51+1 octet o10 |
| Default PC5 DRX configuration for UE-to-network relay discovery |  |  |  |  |  |  |  | octet o10+1 octet o2 |

Figure 5.6.2.5: Not served by NG-RAN

Table 5.6.2.5: Not served by NG-RAN

NR radio parameters per geographical area list for UE-to-network relay discovery (octet o1+3 to o51): The NR radio parameters per geographical area list for UE-to-network relay discovery field is coded according to figure 5.6.2.6 and table 5.6.2.6.

NR radio parameters per geographical area list for UE-to-network relay communication (octet o51+1 to o2): The NR radio parameters per geographical area list for UE-to-network relay communication field is coded according to figure 5.6.2.7 and table 5.6.2.7.

Default PC5 DRX configuration for UE-to-network relay discovery (octet o10+1 to o2): The default PC5 DRX configuration for UE-to-network relay discovery field is coded according to figure 5.6.2.11a and table 5.6.2.11a.

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.6.2.5, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o1+3 octet o1+4 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o1+5 octet o510 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o510+1)* octet o511* |
| ... |  |  |  |  |  |  |  | octet (o511+1)* octet o512* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o512+1)* octet o51* |

Figure 5.6.2.6: NR radio parameters per geographical area list for UE-to-network relay discovery

Table 5.6.2.6: NR radio parameters per geographical area list for UE-to-network relay discovery

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.6.2.8 and table 5.6.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for UE-to-network relay communication contents |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o51+3 octet o513 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o513+1)* octet o514* |
| ... |  |  |  |  |  |  |  | octet (o514+1)* octet o515* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o515+1)* octet o10* |

Figure 5.6.2.7: NR radio parameters per geographical area list for UE-to-network relay communication

Table 5.6.2.7: NR radio parameters per geographical area list for UE-to-network relay communication

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.6.2.8 and table 5.6.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o510+1 octet o510+2 |
| Geographical area |  |  |  |  |  |  |  | octet o510+3 octet o5100 |
| Radio parameters |  |  |  |  |  |  |  | octet o5100+1 octet o511-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o511 |

Figure 5.6.2.8: Radio parameters per geographical area info

Table 5.6.2.8: Radio parameters per geographical area info

Geographical area (octet o510+3 to o5100): The geographical area field is coded according to figure 5.6.2.9 and table 5.6.2.9.

Radio parameters (octet o5100+1 to o511-1): The radio parameters field is coded according to figure 5.3.2.11 and table 5.3.2.11, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o511 bit 8): The managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.6.2.8, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o510+3 octet o510+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o510+5)* octet (o510+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o510+11)* octet (o510+16)* |
| ... |  |  |  |  |  |  |  | octet (o510+17)* octet (o510-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o510-1+6*n)* octet (o510+4+6*n)* = octet o5100* |

Figure 5.6.2.9: Geographical area

Table 5.6.2.9: Geographical area

Coordinate: The coordinate field is coded according to figure 5.6.2.10 and table 5.6.2.10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o510+11 octet o510+13 |
| Longitude |  |  |  |  |  |  |  | octet o510+14 octet o510+17 |

Figure 5.6.2.10: Coordinate area

Table 5.6.2.10: Coordinate area

Latitude (octet o510+11 to o510+13): The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude (octet o510+14 to o510+17): The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o5100+1 octet o5100+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o5100+3 octet o511-1 |

Figure 5.6.2.11: Radio parameters

Table 5.6.2.11: Radio parameters

Radio parameters contents (octet o5100+3 to o511-1): Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default PC5 DRX configuration for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| Default PC5 DRX configuration for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+3 octet o2 |

Figure 5.6.2.11a: Default PC5 DRX configuration for UE-to-network relay discovery

Table 5.6.2.11a: Default PC5 DRX configuration for UE-to-network relay discovery

Default PC5 DRX configuration contents for UE-to-network relay discovery: Default PC5 DRX configuration for UE-to-network relay discovery field is coded as sl-DefaultDRX-GC-BC-r17 in clause 6.3.5 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information contents |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| Default destination layer-2 ID 1 |  |  |  |  |  |  |  | octet o2+3 octet o2+5 |
| Default destination layer-2 ID 2 |  |  |  |  |  |  |  | octet (o2+6)* octet (o2+8)* |
| ... |  |  |  |  |  |  |  | octet (o2+9)* octet (o3-3)* |
| Default destination layer-2 ID n |  |  |  |  |  |  |  | octet (o3-2)* octet o3* |

Figure 5.6.2.11b: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Table 5.6.2.11b: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Default destination layer-2 ID (octet o2+3 to o2+5): The default destination layer-2 ID is a 24-bit long bit string.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info list contents |  |  |  |  |  |  |  |  | octet o3+7 octet o3+8 |  |
| RSC info 1 |  |  |  |  |  |  |  |  | octet o3+9 octet o52 |  |
| RSC info 2 |  |  |  |  |  |  |  |  | octet (o52+1)* octet o53* |  |
| ... |  |  |  |  |  |  |  |  | octet (o53+1)* octet o54* |  |
| RSC info n |  |  |  |  |  |  |  |  | octet (o54+1)* octet o4* |  |

Figure 5.6.2.12: RSC info list

Table 5.6.2.12: RSC info list

RSC info: The RSC info field is coded according to figure 5.6.2.13 and table 5.6.2.13.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+1 octet o52+2 |  |
| RSC list |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+3 octet o520 |  |
| Security related parameters for UE-to-network relay discovery |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o520+1 octet o511 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | TDI |  | CPSI |  | NSI |  | LI |  |  | octet o511+1 |  |
| NR-PC5 UE-to-network relay security policies |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o511+2) octet o530 |  |
| PDU session parameters of layer-3 relay |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o530+1)* octet o516* |  |
| Traffic descriptor |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o516+1)* octet o53* |  |

Figure 5.6.2.13: RSC info

Table 5.6.2.13: RSC info

RSC list (octet o52+3 to o520): The RSC list field is coded according to figure 5.6.2.14 and table 5.6.2.14.

Security related parameters for UE-to-network relay discovery (octet o520+1 to o511): The security related parameters for UE-to-network relay discovery field contains the security related parameters for UE-to-network relay discovery used when the security procedure over control plane as specified in 3GPP TS 33.503 [13] is used and is coded according to figure 5.6.2.15 and table 5.6.2.15.

Layer indication (LI) (octet o511+1 bit 1 to 2): Bits 2 1 0 1 Layer 3 1 0 Layer 2 The other values are reserved.

If LI is set to "Layer 3" and NSI is set to "Using N3IWF access for the relayed traffic is not supported", the PDU session parameters of layer-3 relay is included in the RSC info, otherwise the PDU session parameters of layer-3 relay is not included.

N3IWF support indication (NSI) (octet o511+1 bit 3): Bit 3 0 Using N3IWF access for the relayed traffic is not supported 1 Using N3IWF access for the relayed traffic is supported The NSI is set to "Using N3IWF access for the relayed traffic is supported" only when the LI is set to "Layer 3".

Control plane security indication (CPSI) (octet o511+1): The control plane security indication field indicates whether to use the security procedure over control plane as specified in 3GPP TS 33.503 [13] or not.

Bit

4

0 security procedure over control plane is not used

1 security procedure over control plane is used

Traffic descriptor indication (TDI) (octet o511+1 bit 5): Bit 5 0 Traffic descriptor field is not included 1 Traffic descriptor field is included

NR-PC5 UE-to-network relay security policies (octet o511+2 to o530): The NR-PC5 UE-to-network relay security policies is coded as the NR-PC5 unicast security policies defined in figure 5.4.2.34 and table 5.4.2.34.

PDU session parameters of layer-3 relay (octet o530+1 to o516): The PDU session parameters of layer-3 relay field is coded according to figure 5.6.2.16 and table 5.6.2.16.

Traffic descriptor (octet o516+1 to o53): The traffic descriptor field is coded according to figure 5.6.2.16a and table 5.6.2.16a.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC list contents |  |  |  |  |  |  |  |  | octet o52+3 octet o52+4 |  |
| RSC 1 |  |  |  |  |  |  |  |  | octet o52+5 octet o52+7 |  |
| RSC 2 |  |  |  |  |  |  |  |  | octet (o52+8)* octet (o52+10)* |  |
| … |  |  |  |  |  |  |  |  | octet (o52+11)* octet (o520-3)* |  |
| RSC n |  |  |  |  |  |  |  |  | octet (o520-2)* octet o520* |  |

Figure 5.6.2.14: RSC list

Table 5.6.2.14: RSC list

RSC (octet o52+5 to o52+7): The RSC identifies a connectivity service that the remote UE wants. The value of the RSC is a 24-bit long bit string. The values of the RSCs from "000001" to "00000A" in hexadecimal representation indicate the RSCs for emergency services if the LI is set to "Layer-3" and the values of the RSCs from "00000B" to "00000D" in hexadecimal representation indicate the RSCs for emergency services if the LI is set to "Layer-2". The values of the RSCs from "00000E" to "00000F" in hexadecimal representation are spare and shall not be used in this release of specification. The UE shall ignore the spare value of the RSC in this release of specification. For all other values, the format of the RSC is out of scope of this specification.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security related parameters validity timer |  |  |  |  |  |  |  |  | octet o520+1 octet o520+5 |  |
| Code-sending security parameters |  |  |  |  |  |  |  |  | octet (o520+6)* octet o524* |  |
| Code-receiving security parameters |  |  |  |  |  |  |  |  | octet (o524+1)* octet o511* |  |

Figure  5.6.2.15: Security related parameters for UE-to-network relay discovery

| 8 |  | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spare |  |  |  |  |  | PDUCK |  | PDUIK |  | PDUSK |  | octet o520+6 |  |
| DUSK |  |  |  |  |  |  |  |  |  |  |  | octet (o520+7)* octet o521* |  |
| DUIK |  |  |  |  |  |  |  |  |  |  |  | octet (o521+1)* octet o522* |  |
| DUCK |  |  |  |  |  |  |  |  |  |  |  | octet (o522+1)* octet o523* |  |
| Encrypted bitmask |  |  |  |  |  |  |  |  |  |  |  | octet (o523+1)* octet o524* |  |

Figure 5.6.2.15a: Code-sending security parameters

| 8 |  | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spare |  |  |  |  |  | PDUCK |  | PDUIK |  | PDUSK |  | octet o524+1 |  |
| DUSK |  |  |  |  |  |  |  |  |  |  |  | octet (o524+2)* octet o525* |  |
| DUIK |  |  |  |  |  |  |  |  |  |  |  | octet (o525+1)* octet o526* |  |
| DUCK |  |  |  |  |  |  |  |  |  |  |  | octet (o526+1)* octet o527* |  |
| Encrypted bitmask |  |  |  |  |  |  |  |  |  |  |  | octet (o527+1)* octet o511* |  |

Figure 5.6.2.15b: Code-receiving security parameters

Table 5.6.2.15: Security related parameters for UE-to-network relay discovery

| Security related parameters validity timer: |  |
| --- | --- |
| The security related parameters validity timer field provides the expiration time of validity of the security related parameters for UE-to-network relay discovery. The security related parameters validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) (NOTE 1) (NOTE 2). |  |
| Code-sending security parameters: |  |
| The code-sending security parameters field contains the security parameters needed by a sending UE to protect a 5G ProSe direct discovery message over PC5 interface as specified in 3GPP TS 33.503 [13]. If the security related parameters validity timer field is set to zero, then the code-sending security parameters field is not present, otherwise the code-sending security parameters field is present. |  |
| Code-receiving security parameters |  |
| The code-receiving security parameters field contains the security parameters needed by a receiving UE to process a 5G ProSe direct discovery message over PC5 interface as specified in 3GPP TS 33.503 [13]. If the security related parameters validity timer field is set to zero, then the code-receiving security parameters field is not present, otherwise the code-receiving security parameters field is present. |  |
| Presence of DUSK (PDUSK): |  |
| PDUSK indicates whether the DUSK field is present or not. |  |
| Bit |  |
| 1 |  |
| 0 | DUSK field is not included |
| 1 | DUSK field is included |
| Presence of DUIK (PDUIK): |  |
| PDUIK indicates whether the DUIK field is present or not. |  |
| Bit |  |
| 2 |  |
| 0 | DUIK field is not included |
| 1 | DUIK field is included |
| Presence of DUCK (PDUCK): |  |
| PDUCK indicates whether the DUCK field and the encrypted bitmask field are present or not. |  |
| Bit |  |
| 3 |  |
| 0 | DUCK and encrypted bitmask fields are not included |
| 1 | DUCK and encrypted bitmask fields are included |
| DUSK: |  |
| The DUSK field contains the value of the DUSK. The use of the DUSK is defined in 3GPP TS 33.503 [13]. |  |
| DUIK: |  |
| The DUIK field contains the value of the DUIK. The use of the DUIK is defined in 3GPP TS 33.503 [13]. |  |
| DUCK: |  |
| The DUCK field contains the value of the DUCK. The use of the DUCK is defined in 3GPP TS 33.503 [13]. |  |
| Encrypted bitmask: |  |
| The encrypted bitmask field contains the value of the encrypted bitmask, which is a 184-bit bitmask which uses bit "1" to mark the positions of the bits for which the DUCK encryption is applied. |  |
| NOTE 1: If the network does not have security related parameters for discovery in the UE policies or the security procedure over user plane as specified in 3GPP TS 33.503 [13] is used, the network shall set the security related parameters validity timer field (i.e. octet o520+1 to octet o520+5) to zero value. If the security related parameters validity timer field is set to zero value, the UE shall ignore the security related parameters validity timer. |  |
| NOTE 2: The network shall set the security related parameters validity timer field (i.e. octet o520+1 to octet o520+5) to zero value if the RSCs are for emergency services and there is no discovery security requirement based on regulation and the operator policy. |  |  |

| 8 |  | 7 |  | 6 |  |  | 5 |  | 4 |  | 3 |  |  | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PDU session parameters of layer-3 relay contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o530+1 octet o530+2 |  |
| Spare |  | PATP |  | PSSCM |  |  | PSNSSAI |  | PDNN |  | PDU session type |  |  |  |  | octet o530+3 |  |
| DNN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (octet o530+4)* octet o531* |  |
| S-NSSAI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o531+1)* octet (o516-1)* |  |
| Spare |  |  |  |  | Access type preference |  |  |  |  |  |  | SSC mode |  |  |  | octet o516* |  |

Figure 5.6.2.16: PDU session parameters of layer-3 relay

Table 5.6.2.16: PDU session parameters for layer-3 relay

| PDU session type (bits 3 to 1 of octet o530+3): The PDU session type field shall be encoded as the PDU session type value part of the PDU session type information element defined in clause 9.11.4.11 of 3GPP TS 24.501 [4]. |  |
| --- | --- |
| Presence of DNN (PDNN) (bit 4 of octet o530+3) |  |
| PDNN indicates whether the DNN field is present or not (NOTE 1) |  |
| Bit |  |
| 4 |  |
| 0 | DNN field is not included |
| 1 | DNN field is included |
| Presence of S-NSSAI (PSNSSAI) (bit 5 of octet o530+3) |  |
| PSNSSAI indicates whether the S-NSSAI field is present or not. |  |
| Bit |  |
| 5 |  |
| 0 | S-NSSAI field is not included |
| 1 | S-NSSAI field is included |
| Presence of SSC mode (PSSCM) (bit 6 of octet o530+3) |  |
| PSSCM indicates whether the SSC mode field is present or not. |  |
| Bit |  |
| 6 |  |
| 0 | SSC mode field is not included (NOTE 2) |
| 1 | SSC mode field is included |
| Presence of access type preference (PATP) (bit 7 of octet o530+3) |  |
| PATP indicates whether the access type preference mode field is present or not. |  |
| Bit |  |
| 7 |  |
| 0 | Access type preference field is not included (NOTE 2) |
| 1 | Access type preference field is included |
| DNN (octet o530+4 to o531): The DNN field shall be encoded as a sequence of a one octet DNN length field and a DNN value field of a variable size. The DNN value contains an APN as defined in 3GPP TS 23.003 [10]. |  |
| S-NSSAI (octet o531+1 to o516-1): The S-NSSAI field shall be encoded as a sequence of a one octet S-NSSAI length field and an S-NSSAI value field of a variable size. The S-NSSAI value shall be encoded as the value part of the S-NSSAI information element defined in clause 9.11.2.8 of 3GPP TS 24.501 [4]. |  |
| SSC mode (bits 3 to 1 of octet o516): The SSC mode field shall be encoded as the value part of the SSC mode information element defined in clause 9.11.4.16 of 3GPP TS 24.501 [4]. |  |
| Access type preference (bits 5 to 4 of octet o516): The access type preference field shall be encoded as the value part of the access type information element defined in clause 9.11.2.1A of 3GPP TS 24.501 [4]. |  |
| NOTE 1: PDNN shall be set to 1 if the 5G ProSe UE-to-network relay communication is not for emergency services. NOTE 2: Since SSC mode field and access type preference field are coded in the same octet, this octet is not included only when both PSSCM and PATP are set to 0. |  |

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of traffic descriptor contents |  |  |  |  |  |  |  |  | octet o516+1 octet o516+2 |  |
| Traffic descriptor |  |  |  |  |  |  |  |  | octet o516+3 octet o53 |  |

Figure 5.6.2.16a: Traffic descriptor

Table 5.6.2.16a: Traffic descriptor

Traffic descriptor (octet o516+3 to o53): The traffic descriptor field is coded according to figure 5.2.2 and table 5.2.1 in clause 5.2 of 3GPP TS 24.526 [11].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of N3IWF selection information for 5G ProSe layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+3)* octet (l+4)* |  |
| N3IWF identifier configuration for 5G ProSe layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+5)* octet l0* |  |
| 5G ProSe layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+1)* octet m* |  |

Figure 5.6.2.17: N3IWF selection information for 5G ProSe layer-3 remote UE

Table 5.6.2.17: N3IWF selection information for 5G ProSe layer-3 remote UE

N3IWF identifier configuration for 5G ProSe layer-3 remote UE (octet l+5 to l0): The N3IWF identifier configuration for 5G ProSe layer-3 remote UE contains a list of home N3IWF identifier entries and is coded according to figure 5.6.2.18 and table 5.6.2.18. 5G ProSe layer-3 UE-to-network relays access node selection information (octet l0+1 to m):

The 5G ProSe layer-3 UE-to-network relays access node selection information contains a sequence of the N3AN node selection information entries and is coded according to figure 5.6.2.19 and table 5.6.2.19.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of N3IWF identifier configuration for 5G ProSe layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+5)* octet (l+6)* |  |
| Contents of N3IWF identifier configuration for 5G ProSe layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+7)* octet l0* |  |

Figure 5.6.2.18: N3IWF identifier configuration for 5G ProSe layer-3 remote UE

Table 5.6.2.18: N3IWF identifier configuration for 5G ProSe layer-3 remote UE

Contents of N3IWF identifier configuration for 5G ProSe layer-3 remote UE (octet l+7 to l01): The contents of N3IWF identifier configuration for 5G ProSe layer-3 remote UE shall be encoded as the encoding of home N3IWF identifier configuration defined in clause 5.3.3.3 of 3GPP TS 24.526 [11].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 5G ProSe layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+1)* octet (l0+2)* |  |
| Contents of 5G ProSe layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+3)* octet m* |  |

Figure 5.6.2.19: 5G ProSe layer-3 UE-to-network relays access node selection information

Table 5.6.2.19: 5G ProSe layer-3 UE-to-network relays access node selection information

Contents of 5G ProSe layer-3 UE-to-network relays access node selection information (octet l0+3 to m): The contents of 5G ProSe layer-3 UE-to-network relays access node selection information shall be encoded as the encoding of N3AN node selection information defined in clause 5.3.3.2 of 3GPP TS 24.526 [11].

NOTE: In this release of specification, the "preference" bit (as shown in figure 5.3.3.2.2 of 3GPP TS 24.526 [11]) is always set to "0".

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of warning message broadcast |  |  |  |  |  |  |  |  | octet a octet a+1 |  |
| Destination layer-2 ID list |  |  |  |  |  |  |  |  | octet a+2 octet a1 |  |
| PQI |  |  |  |  |  |  |  |  | octet a1+1 octet a1+2 |  |
| PDB adjustment factor |  |  |  |  |  |  |  |  |  |  |
| NR Tx profile |  |  |  |  |  |  |  |  | octet a1+3 octet b |  |

Figure 5.6.2.20: Warning message broadcast

Table 5.6.2.20: Warning message broadcast

Destination layer-2 ID list (octet a+2 to a1): The destination layer-2 ID list field is coded according to figure 5.6.2.21 and table 5.6.2.21 and contains the list of destination layer-2 IDs.

PQI (octet a1+1): The PQI field is coded according to figure 5.5.2.18 and table 5.5.2.18 and contains the PQI for broadcasting the warning messages.

PDB adjustment factor (octet a1+2): The PDB adjustment factor field is codeded according to figure 5.5.2.18 and table 5.5.2.18 and contains the PDB adjustment factor for broadcasting the warning messages.

NR Tx profile (octet a1+3 to b): The NR Tx profile field is coded as SL-TxProfile-r17 in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of destination layer-2 ID list |  |  |  |  |  |  |  | octet a+2 octet a+3 |
| Destination layer-2 ID 1 |  |  |  |  |  |  |  | octet a+4 octet a+6 |
| Destination layer-2 ID 2 |  |  |  |  |  |  |  | octet (a+7)* octet (a+9)* |
| ... |  |  |  |  |  |  |  | octet (a+10)* octet (a1-3)* |
| Destination layer-2 ID n |  |  |  |  |  |  |  | octet (a1-2)* octet a1* |

Figure 5.6.2.21: Destination layer-2 ID list

Table 5.6.2.21: Destination layer-2 ID

Destination layer-2 ID (octet a+4 to a+6): The destination layer-2 ID is a 24-bit long bit string.

==============Next change==============

### 5.11.2	Information elements coding

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | PAI | ProSeP info type = {UE policies for 5G ProSe intermediate UE-to-network relay UE} |  |  |  | octet k |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  | octet k+1 octet k+2 |  |  |
| Validity timer |  |  |  |  |  |  |  | octet k+3 octet k+7 |  |  |
| Served by NG-RAN |  |  |  |  |  |  |  | octet k+8 octet o1 |  |  |
| Not served by NG-RAN |  |  |  |  |  |  |  | octet o1+1 octet o2 |  |  |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information |  |  |  |  |  |  |  | octet o2+1 octet o3 |  |  |
| User info ID for discovery |  |  |  |  |  |  |  | octet o3+1 octet o3+6 |  |  |
| RSC info list |  |  |  |  |  |  |  | octet o3+7 octet l |  |  |
| Privacy timer |  |  |  |  |  |  |  | octet l+1 octet l+2 |  |  |
| 5G PKMF address information |  |  |  |  |  |  |  | octet (l+3)* octet b* |  |
| ProSe PC5 broadcast for public warning notification |  |  |  |  |  |  |  | octet p octet q |

Figure 5.11.2.1: ProSeP Info = {UE policies for 5G ProSe intermediate UE-to-network relay UE}

Table 5.11.2.1: ProSeP Info = {UE policies for 5G ProSe intermediate UE-to-network relay UE}

| ProSeP info type (bit 1 to 4 of octet k) shall be set to "1001" (UE policies for 5G ProSe intermediate UE-to-network relay UE) |  |  |
| --- | --- | --- |
| PKMF address indication (PAI) (bit 5 of octet k) |  |  |
| The PAI indicates whether the 5G PKMF address information is included in the IE or not |  |  |
| Bit 5 |  |  |
| 0 5G PKMF address information is not included |  |  |
| 1 5G PKMF address information is included |  |  |
| Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents. |  |  |
| Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe remote UE. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |
| Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.11.2.2 and table 5.11.2.2, and contains configuration parameters for 5G ProSe intermediate UE-to-network relay UE when the UE is served by NG-RAN. |  |  |
| Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.6.2.5 and table 5.6.2.5, and contains configuration parameters for 5G ProSe multi-hop UE-to-network relay discovery and communication when the UE is not served by NG-RAN. |  |  |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information (octet o2+1 to o3): The default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information is coded according to figure 5.11.2.11b and table 5.11.2.11b and contains a list of the default destination layer-2 IDs for the initial multi-hop UE-to-network relay discovery signalling. |  |  |
| User info ID for discovery (octet o3+1 to o3+6): The value of the User info ID parameter is a 48-bit long bit string. The format of the User info ID parameter is out of scope of this specification. |  |  |
| RSC info list (octet o3+7 to l): The RSC info list field is coded according to figure 5.11.2.12 and table 5.11.2.12 and contains the RSCs related parameters. |  |  |
| Privacy timer (octet l+1 to l+2): The privacy timer field contains binary encoded duration, in units of seconds, after which the UE shall change the source layer-2 ID self-assigned by the UE while performing transmission of 5G ProSe direct communication. |  |  |
| 5G PKMF address information (octet l+3 to b): The 5G PKMF address information contains the IPv4 address(es), IPv6 address(es) and/or FQDN of the 5G PKMF and is coded according to figure 5.5.2.21, figure 5.5.2.22, figure 5.5.2.23 and table 5.5.2.21. At least one of the address parameters (FQDN, IPv4 address list or IPv6 address list) shall be included. ProSe PC5 broadcast for public warning notification (octet p to q): The ProSe PC5 broadcast for public warning notification filed is coded according to figure 5.5.2.24 and table 5.5.2.24 and contains the parameters for broadcasting ProSe PC5 broadcast message for warning notification. |  |  |
| If the length of ProSeP info contents field is bigger than indicated in figure 5.6.2.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents. |  |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | SNAI | L3IUNRI | octet k+10 |
| Authorized PLMN list for multi-hop layer-2 intermediate UE-to-network relay UE |  |  |  |  |  |  |  | octet k+11 octet v |

Figure 5.11.2.2: Served by NG-RAN

Table 5.11.2.2: Served by NG-RAN

Layer-3 intermediate UE-to-network relay UE authorization indication (L3IUNRI) (octet k+10, bit 1): The layer-3 remote UE authorization indication field indicates whether the UE is authorized to act as a layer-3 intermediate UE-to-network relay UE. Bits 1 0 Not authorized to act as a layer-3 intermediate UE-to-network relay UE. 1 Authorized to act as a layer-3 intermediate UE-to-network relay UE. Subscribed SNPN authorization indicator (SNAI) (octet k+10, bit 2) Bit 2 0 Subscribed SNPN is not authorized 1 Subscribed SNPN is authorized Authorized PLMN list for multi-hop layer-2 intermediate UE-to-network relay UE (octet k+11 to v): The authorized PLMN list for multi-hop layer-2 intermediate UE-to-network relay UE field is coded according to figure 5.10.2.3 and table 5.10.2.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet k+16 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet k+17 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet k+18 |

Figure 5.11.2.3: PLMN ID

Table 5.11.2.3: PLMN ID

Mobile country code (MCC) (octet k+16, octet k+17 bit 1 to 4): The MCC field is coded as in ITU-T Recommendation E.212 [5], annex A.

Mobile network code (MNC) (octet k+17 bit 5 to 8, octet k+18): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111".

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| NR radio parameters per geographical area list for UE-to-network relay discovery |  |  |  |  |  |  |  | octet o1+3 octet o51 |
| NR radio parameters per geographical area list for UE-to-network relay communication |  |  |  |  |  |  |  | octet o51+1 octet o10 |
| Default PC5 DRX configuration for UE-to-network relay discovery |  |  |  |  |  |  |  | octet o10+1 octet o2 |

Figure 5.11.2.4: Not served by NG-RAN

Table 5.11.2.4: Not served by NG-RAN

NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery (octet o1+3 to o51): The NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery field is coded according to figure 5.11.2.5 and table 5.11.2.5.

NR radio parameters per geographical area list for multi-hop UE-to-network relay communication (octet o51+1 to o2): The NR radio parameters per geographical area list for multi-hop UE-to-network relay communication field is coded according to figure 5.11.2.6 and table 5.11.2.6.

Default PC5 DRX configuration for UE-to-network relay discovery (octet o10+1 to o2): The default PC5 DRX configuration for UE-to-network relay discovery field is coded according to figure 5.11.2.11 and table 5.11.2.11.

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.11.2.4, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o1+3 octet o1+4 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o1+5 octet o510 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o510+1)* octet o511* |
| ... |  |  |  |  |  |  |  | octet (o511+1)* octet o512* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o512+1)* octet o51* |

Figure 5.11.2.5: NR radio parameters per geographical area list for UE-to-network relay discovery

Table 5.11.2.5: NR radio parameters per geographical area list for UE-to-network relay discovery

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.11.2.7 and table 5.11.2.7.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for multi-hop UE-to-network relay communication contents |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o51+3 octet o513 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o513+1)* octet o514* |
| ... |  |  |  |  |  |  |  | octet (o514+1)* octet o515* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o515+1)* octet o10* |

Figure 5.11.2.6: NR radio parameters per geographical area list for UE-to-network relay communication

Table 5.11.2.6: NR radio parameters per geographical area list for UE-to-network relay communication

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.11.2.7 and table 5.11.2.7.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o510+1 octet o510+2 |
| Geographical area |  |  |  |  |  |  |  | octet o510+3 octet o5100 |
| Radio parameters |  |  |  |  |  |  |  | octet o5100+1 octet o511-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o511 |

Figure 5.11.2.7: Radio parameters per geographical area info

Table 5.11.2.7: Radio parameters per geographical area info

Geographical area (octet o510+3 to o5100): The geographical area field is coded according to figure 5.11.2.8 and table 5.11.2.8.

Radio parameters (octet o5100+1 to o511-1): The radio parameters field is coded according to figure 5.3.2.10 and table 5.3.2.10, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o511 bit 8): The managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.11.2.7, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o510+3 octet o510+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o510+5)* octet (o510+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o510+11)* octet (o510+16)* |
| ... |  |  |  |  |  |  |  | octet (o510+17)* octet (o510-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o510-1+6*n)* octet (o510+4+6*n)* = octet o5100* |

Figure 5.11.2.8: Geographical area

Table 5.11.2.8: Geographical area

Coordinate: The coordinate field is coded according to figure 5.6.2.10 and table 5.6.2.10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o510+11 octet o510+13 |
| Longitude |  |  |  |  |  |  |  | octet o510+14 octet o510+17 |

Figure 5.11.2.9: Coordinate area

Table 5.11.2.9: Coordinate area

Latitude (octet o510+11 to o510+13): The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude (octet o510+14 to o510+17): The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o5100+1 octet o5100+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o5100+3 octet o511-1 |

Figure 5.11.2.10: Radio parameters

Table 5.11.2.10: Radio parameters

Radio parameters contents (octet o5100+3 to o511-1): Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default PC5 DRX configuration for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| Default PC5 DRX configuration for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+3 octet o2 |

Figure 5.11.2.11: Default PC5 DRX configuration for UE-to-network relay discovery

Table 5.11.2.11: Default PC5 DRX configuration for UE-to-network relay discovery

Default PC5 DRX configuration contents for UE-to-network relay discovery: Default PC5 DRX configuration for UE-to-network relay discovery field is coded as sl-DefaultDRX-GC-BC-r17 in clause 6.3.5 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information contents |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| Default destination layer-2 ID 1 |  |  |  |  |  |  |  | octet o2+3 octet o2+5 |
| Default destination layer-2 ID 2 |  |  |  |  |  |  |  | octet (o2+6)* octet (o2+8)* |
| ... |  |  |  |  |  |  |  | octet (o2+9)* octet (o3-3)* |
| Default destination layer-2 ID n |  |  |  |  |  |  |  | octet (o3-2)* octet o3* |

Figure 5.6.2.11a: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Table 5.6.2.11a: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Default destination layer-2 ID (octet o2+3 to o2+5): The default destination layer-2 ID is a 24-bit long bit string.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info list contents |  |  |  |  |  |  |  |  | octet o3+7 octet o3+8 |  |
| RSC info 1 |  |  |  |  |  |  |  |  | octet o3+9 octet o52 |  |
| RSC info 2 |  |  |  |  |  |  |  |  | octet (o52+1)* octet o53* |  |
| ... |  |  |  |  |  |  |  |  | octet (o53+1)* octet o54* |  |
| RSC info n |  |  |  |  |  |  |  |  | octet (o54+1)* octet o4* |  |

Figure 5.6.2.12: RSC info list

Table 5.6.2.12: RSC info list

RSC info: The RSC info field is coded according to figure 5.11.2.13 and table 5.11.2.13.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+1 octet o52+2 |  |  |
| RSC list |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+3 octet o520 |  |  |
| Security related parameters for multi-hop UE-to-network relay discovery |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o520+1 octet o511 |  |  |
| 0 Spare |  | CPSI |  | MNHI |  | TT |  |  | MHI |  | LI |  |  |  | octet o511+1 |  |  |
| NR-PC5 multi-hop UE-to-network relay security policies |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o511+2) octet o530 |  |  |
| Maximum number of hops |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o516+1)* |  |  |

Figure 5.11.2.13: RSC info

Table 5.11.2.13: RSC info

| RSC list (octet o52+3 to o520): The RSC list field is coded according to figure 5.11.2.14 and table 5.11.2.14. Security related parameters for multi-hop UE-to-network relay discovery (octet o520+1 to o511): The security related parameters for multi-hop UE-to-network relay discovery field contains the security related parameters for UE-to-network relay discovery used when the security procedure over control plane as specified in 3GPP TS 33.503 [13] is used and is coded according to figure 5.10.2.15 and table 5.10.2.15. |  |
| --- | --- |
| Layer indication (LI) (octet o511+1 bit 1 to 2): Bits 2 1 0 1 Layer 3 1 0 Layer 2 The other values are reserved. |  |
| Multi-hop indicator (MHI) (octet o511+1 bit 3): Bits 3 0 Multi-hop relay is not allowed 1 Multi-hop relay is allowed If MHI is set to "Multi-hop relay is allowed", the Maximum number of hops may be included in the RSC info. |  |
| Traffic type (TT) (octet o511+1 bit 4 to 5): The traffic type field indicates the traffic type of the relayed service and exists when the LI is set to "Layer 3". Bits 4 3 0 0 No information 0 1 IP 1 0 Ethernet 1 1 Unstructured |  |
| Maximum number of hops indicator (MNHI) (octet o511+1 bit 6): Bits 6 0 Maximum number of hops is not indicated 1 Maximum number of hops is indicated |  |
| Control plane security indication (CPSI) (octet o511+1 bit 7): The control plane security indication field indicates whether to use the security procedure over control plane as specified in 3GPP TS 33.503 [13] or not. Bit 7 0 security procedure over control plane is not used 1 security procedure over control plane is used |  |
| NR-PC5 multi-hop UE-to-network relay security policies (octet o511+2 to o530): The NR-PC5 multi-hop UE-to-network relay security policies is coded as the NR-PC5 unicast security policies defined in figure 5.4.2.34 and table 5.4.2.34. |  |
| Maximum number of hops (octet o516+1) Maximum number of hops is a binary coded representation of the maximum number of hops can be supported for the associated RSC. |  |

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC list contents |  |  |  |  |  |  |  |  | octet o52+3 octet o52+4 |  |
| RSC 1 |  |  |  |  |  |  |  |  | octet o52+5 octet o52+7 |  |
| RSC 2 |  |  |  |  |  |  |  |  | octet (o52+8)* octet (o52+10)* |  |
| … |  |  |  |  |  |  |  |  | octet (o52+11)* octet (o520-3)* |  |
| RSC n |  |  |  |  |  |  |  |  | octet (o520-2)* octet o520* |  |

Figure 5.11.2.14: RSC list

Table 5.11.2.14: RSC list

RSC (octet o52+5 to o52+7): The RSC identifies a connectivity service that the layer3 intermediate relay UE wants. The value of the RSC is a 24-bit long bit string. The values of the RSCs from "000001" to "00000A" in hexadecimal representation indicate the RSCs for emergency services if the LI is set to "Layer-3". The values of the RSCs from "00000E" to "00000F" in hexadecimal representation are spare and shall not be used in this release of specification. The UE shall ignore the spare value of the RSC in this release of specification. For all other values, the format of the RSC is out of scope of this specification.

==============Next change==============

### 5.12.2	Information elements coding

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | PAI | NSII | ProSeP info type = {UE policies for 5G ProSe multi-hop remote UE} |  |  |  | octet k |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  | octet k+1 octet k+2 |  |  |
| Validity timer |  |  |  |  |  |  |  | octet k+3 octet k+7 |  |  |
| Served by NG-RAN |  |  |  |  |  |  |  | octet k+8 octet o1 |  |  |
| Not served by NG-RAN |  |  |  |  |  |  |  | octet o1+1 octet o2 |  |  |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information |  |  |  |  |  |  |  | octet o2+1 octet o3 |  |  |
| User info ID for discovery |  |  |  |  |  |  |  | octet o3+1 octet o3+6 |  |  |
| RSC info list |  |  |  |  |  |  |  | octet o3+7 octet l |  |  |
| Privacy timer |  |  |  |  |  |  |  | octet l+1 octet l+2 |  |  |
| N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE |  |  |  |  |  |  |  | octet (l+3)* octet m* |  |  |
| 5G PKMF address information |  |  |  |  |  |  |  | octet (m+1)* octet n* |  |
| ProSe PC5 broadcast for public warning notification |  |  |  |  |  |  |  | octet p octet q |

Figure 5.12.2.1: ProSeP Info = {UE policies for 5G ProSe multi-hop remote UE}

Table 5.10.2.1: ProSeP Info = {UE policies for 5G ProSe multi-hop remote UE}

ProSeP info type (bit 1 to 4 of octet k) shall be set to "1010" (UE policies for 5G ProSe multi-hop remote UE)

N3IWF selection information indication (NSII) (bit 5 of octet k) The NSII indicates whether the N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE is included in the IE or not Bit 5 0 N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE is not included 1 N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE is included

Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents.

Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe multi-hop remote UE. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.12.2.2 and table 5.12.2.2, and contains configuration parameters for 5G ProSe multi-hop remote UE when the UE is served by NG-RAN.

Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.12.2.5 and table 5.12.2.5, and contains configuration parameters for 5G ProSe multi-hop UE-to-network relay discovery and communication when the UE is not served by NG-RAN.

Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information (octet o2+1 to o3): The default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information is coded according to figure 5.12.2.11b and table 5.12.2.11b and contains a list of the default destination layer-2 IDs for the initial multi-hop UE-to-network relay discovery signalling.

User info ID for discovery (octet o3+1 to o3+6): The value of the User info ID parameter is a 48-bit long bit string. The format of the User info ID parameter is out of scope of this specification.

RSC info list (octet o3+7 to l): The RSC info list field is coded according to figure 5.12.2.12 and table 5.12.2.12 and contains the RSCs related parameters.

Privacy timer (octet l+1 to l+2): The privacy timer field contains binary encoded duration, in units of seconds, after which the UE shall change the source layer-2 ID self-assigned by the UE while performing transmission of 5G ProSe direct communication. N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE (octet l+3 to m): The N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE field is coded according to figure 5.12.2.17 and table 5.12.2.17, and contains two parts: 1) N3IWF identifier configuration (either FQDN or IP address) for 5G ProSe multi-hop layer-3 remote UE; 2) 5G ProSe multi-hop layer-3 UE-to-network relay access node selection information.

5G PKMF address information (octet m+1 to n): The 5G PKMF address information contains the IPv4 address(es), IPv6 address(es) and/or FQDN of the 5G PKMF and is coded according to figure 5.5.2.21, figure 5.5.2.22, figure 5.5.2.23 and table 5.5.2.21. At least one of the address parameters (FQDN, IPv4 address list or IPv6 address list) shall be included. ProSe PC5 broadcast for public warning notification (octet p to q): The ProSe PC5 broadcast for public warning notification filed is coded according to figure 5.5.2.24 and table 5.5.2.24 and contains the parameters for broadcasting ProSe PC5 broadcast message for public warning notification.

If the length of ProSeP info contents field is bigger than indicated in figure 5.12.2.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | SNAI | M3RMI | octet k+10 |
| Authorized PLMN list for multi-hop layer-2 remote UE |  |  |  |  |  |  |  | octet k+11 octet v |

Figure 5.12.2.2: Served by NG-RAN

Table 5.12.2.2: Served by NG-RAN

Multi-hop layer-3 remote UE authorization indication (M3RMI) (octet k+10, bit 1): The multi-hop layer-3 remote UE authorization indication field indicates whether the UE is authorized to act as a multi-hop layer-3 remote UE. Bits 1 0 Not authorized to act as a multi-hop layer-3 remote UE 1 Authorized to act as a multi-hop layer-3 remote UE Subscribed SNPN authorization indicator (SNAI) (octet k+10, bit 2): Bit 2 0 Subscribed SNPN is not authorized 1 Subscribed SNPN is authorized Authorized PLMN list for multi-hop layer-2 remote UE (octet k+11 to v): The authorized PLMN list for multi-hop layer-2 remote UE field is coded according to figure 5.10.2.3 and table 5.10.2.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery |  |  |  |  |  |  |  | octet o1+3 octet o51 |
| NR radio parameters per geographical area list for multi-hop UE-to-network relay communication |  |  |  |  |  |  |  | octet o51+1 octet o10 |
| Default PC5 DRX configuration for multi-hop UE-to-network relay discovery |  |  |  |  |  |  |  | octet o10+1 octet o2 |

Figure 5.12.2.3: Not served by NG-RAN

Table 5.12.2.3: Not served by NG-RAN

NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery (octet o1+3 to o51): The NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery field is coded according to figure 5.12.2.4 and table 5.12.2.4.

NR radio parameters per geographical area list for multi-hop UE-to-network relay communication (octet o51+1 to o2): The NR radio parameters per geographical area list for UE-to-network relay communication field is coded according to figure 5.12.2.5 and table 5.12.2.5.

Default PC5 DRX configuration for UE-to-network relay discovery (octet o10+1 to o2): The default PC5 DRX configuration for UE-to-network relay discovery field is coded according to figure 5.6.2.10 and table 5.6.2.10.

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.6.2.5, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o1+3 octet o1+4 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o1+5 octet o510 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o510+1)* octet o511* |
| ... |  |  |  |  |  |  |  | octet (o511+1)* octet o512* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o512+1)* octet o51* |

Figure 5.12.2.4: NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery

Table 5.12.2.4: NR radio parameters per geographical area list for multi-hop UE-to-network relay discovery

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.12.2.8 and table 5.12.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for multi-hop UE-to-network relay communication contents |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o51+3 octet o513 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o513+1)* octet o514* |
| ... |  |  |  |  |  |  |  | octet (o514+1)* octet o515* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o515+1)* octet o10* |

Figure 5.12.2.5: NR radio parameters per geographical area list for multi-hop UE-to-network relay communication

Table 5.12.2.5: NR radio parameters per geographical area list for multi-hop UE-to-network relay communication

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.12.2.6 and table 5.12.2.6.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o510+1 octet o510+2 |
| Geographical area |  |  |  |  |  |  |  | octet o510+3 octet o5100 |
| Radio parameters |  |  |  |  |  |  |  | octet o5100+1 octet o511-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o511 |

Figure 5.12.2.6: Radio parameters per geographical area info

Table 5.12.2.6: Radio parameters per geographical area info

Geographical area (octet o510+3 to o5100): The geographical area field is coded according to figure 5.12.2.7 and table 5.12.2.7.

Radio parameters (octet o5100+1 to o511-1): The radio parameters field is coded according to figure 5.3.2.9 and table 5.3.2.9, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o511 bit 8): The managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.12.2.6, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o510+3 octet o510+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o510+5)* octet (o510+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o510+11)* octet (o510+16)* |
| ... |  |  |  |  |  |  |  | octet (o510+17)* octet (o510-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o510-1+6*n)* octet (o510+4+6*n)* = octet o5100* |

Figure 5.12.2.7: Geographical area

Table 5.12.2.7: Geographical area

Coordinate: The coordinate field is coded according to figure 5.12.2.8 and table 5.12.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o510+11 octet o510+13 |
| Longitude |  |  |  |  |  |  |  | octet o510+14 octet o510+17 |

Figure 5.12.2.8: Coordinate area

Table 5.12.2.8: Coordinate area

Latitude (octet o510+11 to o510+13): The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude (octet o510+14 to o510+17): The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o5100+1 octet o5100+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o5100+3 octet o511-1 |

Figure 5.12.2.9: Radio parameters

Table 5.12.2.9: Radio parameters

Radio parameters contents (octet o5100+3 to o511-1): Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default PC5 DRX configuration for multi-hop UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| Default PC5 DRX configuration for multi-hop UE-to-network relay discovery contents |  |  |  |  |  |  |  | octet o10+3 octet o2 |

Figure 5.12.2.10: Default PC5 DRX configuration for multi-hop UE-to-network relay discovery

Table 5.12.2.10: Default PC5 DRX configuration for multi-hop UE-to-network relay discovery

Default PC5 DRX configuration contents for UE-to-network relay discovery: Default PC5 DRX configuration for multi-hop UE-to-network relay discovery field is coded as sl-DefaultDRX-GC-BC-r17 in clause 6.3.5 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information contents |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| Default destination layer-2 ID 1 |  |  |  |  |  |  |  | octet o2+3 octet o2+5 |
| Default destination layer-2 ID 2 |  |  |  |  |  |  |  | octet (o2+6)* octet (o2+8)* |
| ... |  |  |  |  |  |  |  | octet (o2+9)* octet (o3-3)* |
| Default destination layer-2 ID n |  |  |  |  |  |  |  | octet (o3-2)* octet o3* |

Figure 5.12.2.11: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Table 5.12.2.11: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Default destination layer-2 ID (octet o2+3 to o2+5): The default destination layer-2 ID is a 24-bit long bit string.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info list contents |  |  |  |  |  |  |  |  | octet o3+7 octet o3+8 |  |
| RSC info 1 |  |  |  |  |  |  |  |  | octet o3+9 octet o52 |  |
| RSC info 2 |  |  |  |  |  |  |  |  | octet (o52+1)* octet o53* |  |
| ... |  |  |  |  |  |  |  |  | octet (o53+1)* octet o54* |  |
| RSC info n |  |  |  |  |  |  |  |  | octet (o54+1)* octet o4* |  |

Figure 5.12.2.12: RSC info list

Table 5.12.2.12: RSC info list

RSC info: The RSC info field is coded according to figure 5.12.2.13 and table 5.12.2.13.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+1 octet o52+2 |  |  |
| RSC list |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o52+3 octet o520 |  |  |
| Security related parameters for multi-hop UE-to-network relay discovery |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o520+1 octet o511 |  |  |
| 0 Spare |  | CPSI |  | MNHI |  | TDI |  | MHI |  | NSI |  | LI |  |  |  | octet o511+1 |  |  |
| NR-PC5 multi-hop UE-to-network relay security policies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o511+2) octet o530 |  |  |
| PDU session parameters of multi-hop layer-3 relay |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o530+1)* octet o516* |  |  |
| Traffic descriptor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o516+1)* octet o53* |  |  |
| Maximum number of hops |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o53+1)* |  |  |

Figure 5.12.2.13: RSC info

Table 5.12.2.13: RSC info

| RSC list (octet o52+3 to o520): The RSC list field is coded according to figure 5.12.2.14 and table 5.12.2.14. Security related parameters for multi-hop UE-to-network relay discovery (octet o520+1 to o511): The security related parameters for multi-hop UE-to-network relay discovery field contains the security related parameters for UE-to-network relay discovery used when the security procedure over control plane as specified in 3GPP TS 33.503 [13] is used and is coded according to figure 5.10.2.15 and table 5.10.2.15. |  |
| --- | --- |
| Layer indication (LI) (octet o511+1 bit 1 to 2): Bits 2 1 0 1 Layer 3 1 0 Layer 2 The other values are reserved. |  |
| If LI is set to "Layer 3" and NSI is set to "Using N3IWF access for the relayed traffic is not supported", the PDU session parameters of multi-hop layer-3 relay is included in the RSC info, otherwise the PDU session parameters of multi-hop layer-3 relay is not included. |  |
| N3IWF support indication (NSI) (octet o511+1 bit 3): Bit 3 0 Using N3IWF access for the relayed traffic is not supported 1 Using N3IWF access for the relayed traffic is supported The NSI is set to "Using N3IWF access for the relayed traffic is supported" only when the LI is set to "Layer 3". |  |
| Multi-hop indicator (MHI) (octet o511+1 bit 4): Bits 3 0 Multi-hop relay is not allowed 1 Multi-hop relay is allowed If MHI is set to "Multi-hop relay is allowed", the Maximum number of hops may be included in the RSC info. |  |
| Traffic descriptor indication (TDI) (octet o511+1 bit 5): Bit 5 0 Traffic descriptor field is not included 1 Traffic descriptor field is included |  |
| Maximum number of hops indicator (MNHI) (octet o511+1 bit 6): Bits 6 0 Maximum number of hops is not indicated 1 Maximum number of hops is indicated |  |
| Control plane security indication (CPSI) (octet o511+1 bit 7): The control plane security indication field indicates whether to use the security procedure over control plane as specified in 3GPP TS 33.503 [13] or not. Bit 7 0 security procedure over control plane is not used 1 security procedure over control plane is used |  |
| NR-PC5 multi-hop UE-to-network relay security policies (octet o511+2 to o530): The NR-PC5 multi-hop UE-to-network relay security policies is coded as the NR-PC5 unicast security policies defined in figure 5.4.2.34 and table 5.4.2.34. |  |
| PDU session parameters of multi-hop layer-3 relay (octet o530+1 to o516): The PDU session parameters of multi-hop layer-3 relay field is coded according to figure 5.12.2.16 and table 5.12.2.16. |  |
| Traffic descriptor (octet o516+1 to o53): The traffic descriptor field is coded according to figure 5.12.2.17 and table 5.12.2.17. |  |
| Maximum number of hops (octet o53+1) Maximum number of hops is a binary coded representation of the maximum number of hops can be supported for the associated RSC. |  |

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC list contents |  |  |  |  |  |  |  |  | octet o52+3 octet o52+4 |  |
| RSC 1 |  |  |  |  |  |  |  |  | octet o52+5 octet o52+7 |  |
| RSC 2 |  |  |  |  |  |  |  |  | octet (o52+8)* octet (o52+10)* |  |
| … |  |  |  |  |  |  |  |  | octet (o52+11)* octet (o520-3)* |  |
| RSC n |  |  |  |  |  |  |  |  | octet (o520-2)* octet o520* |  |

Figure 5.12.2.14: RSC list

Table 5.12.2.14: RSC list

RSC (octet o52+5 to o52+7): The RSC identifies a connectivity service that the multi-hop remote UE wants. The value of the RSC is a 24-bit long bit string. The values of the RSCs from "000001" to "00000A" in hexadecimal representation indicate the RSCs for emergency services if the LI is set to "Layer-3". The values of the RSCs from "00000E" to "00000F" in hexadecimal representation are spare and shall not be used in this release of specification. The UE shall ignore the spare value of the RSC in this release of specification. For all other values, the format of the RSC is out of scope of this specification.

| 8 |  | 7 |  | 6 |  |  | 5 |  | 4 |  | 3 |  |  | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PDU session parameters of multi-hop layer-3 relay contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o530+1 octet o530+2 |  |
| Spare |  | PATP |  | PSSCM |  |  | PSNSSAI |  | PDNN |  | PDU session type |  |  |  |  | octet o530+3 |  |
| DNN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (octet o530+4)* octet o531* |  |
| S-NSSAI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o531+1)* octet (o516-1)* |  |
| Spare |  |  |  |  | Access type preference |  |  |  |  |  |  | SSC mode |  |  |  | octet o516* |  |

Figure 5.12.2.15: PDU session parameters of multi-hop layer-3 relay

Table 5.12.2.15: PDU session parameters for multi-hop layer-3 relay

| PDU session type (bits 3 to 1 of octet o530+3): The PDU session type field shall be encoded as the PDU session type value part of the PDU session type information element defined in clause 9.11.4.11 of 3GPP TS 24.501 [4]. |  |
| --- | --- |
| Presence of DNN (PDNN) (bit 4 of octet o530+3) |  |
| PDNN indicates whether the DNN field is present or not (NOTE 1) |  |
| Bit |  |
| 4 |  |
| 0 | DNN field is not included |
| 1 | DNN field is included |
| Presence of S-NSSAI (PSNSSAI) (bit 5 of octet o530+3) |  |
| PSNSSAI indicates whether the S-NSSAI field is present or not. |  |
| Bit |  |
| 5 |  |
| 0 | S-NSSAI field is not included |
| 1 | S-NSSAI field is included |
| Presence of SSC mode (PSSCM) (bit 6 of octet o530+3) |  |
| PSSCM indicates whether the SSC mode field is present or not. |  |
| Bit |  |
| 6 |  |
| 0 | SSC mode field is not included (NOTE 2) |
| 1 | SSC mode field is included |
| Presence of access type preference (PATP) (bit 7 of octet o530+3) |  |
| PATP indicates whether the access type preference mode field is present or not. |  |
| Bit |  |
| 7 |  |
| 0 | Access type preference field is not included (NOTE 2) |
| 1 | Access type preference field is included |
| DNN (octet o530+4 to o531): The DNN field shall be encoded as a sequence of a one octet DNN length field and a DNN value field of a variable size. The DNN value contains an APN as defined in 3GPP TS 23.003 [10]. |  |
| S-NSSAI (octet o531+1 to o516-1): The S-NSSAI field shall be encoded as a sequence of a one octet S-NSSAI length field and an S-NSSAI value field of a variable size. The S-NSSAI value shall be encoded as the value part of the S-NSSAI information element defined in clause 9.11.2.8 of 3GPP TS 24.501 [4]. |  |
| SSC mode (bits 3 to 1 of octet o516): The SSC mode field shall be encoded as the value part of the SSC mode information element defined in clause 9.11.4.16 of 3GPP TS 24.501 [4]. |  |
| Access type preference (bits 5 to 4 of octet o516): The access type preference field shall be encoded as the value part of the access type information element defined in clause 9.11.2.1A of 3GPP TS 24.501 [4]. |  |
| NOTE 1: PDNN shall be set to 1 if the 5G ProSe UE-to-network relay communication is not for emergency services. NOTE 2: Since SSC mode field and access type preference field are coded in the same octet, this octet is not included only when both PSSCM and PATP are set to 0. |  |

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of traffic descriptor contents |  |  |  |  |  |  |  |  | octet o516+1 octet o516+2 |  |
| Traffic descriptor |  |  |  |  |  |  |  |  | octet o516+3 octet o53 |  |

Figure 5.12.2.16: Traffic descriptor

Table 5.12.2.16: Traffic descriptor

Traffic descriptor (octet o516+3 to o53): The traffic descriptor field is coded according to figure 5.2.2 and table 5.2.1 in clause 5.2 of 3GPP TS 24.526 [11].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+3)* octet (l+4)* |  |
| N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+5)* octet l0* |  |
| 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+1)* octet m* |  |

Figure 5.12.2.17: N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE

Table 5.12.2.17: N3IWF selection information for 5G ProSe multi-hop layer-3 remote UE

N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE (octet l+5 to l0): The N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE contains a list of home N3IWF identifier entries and is coded according to figure 5.12.2.19 and table 5.12.2.19. 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information (octet l0+1 to m):

The 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information contains a sequence of the N3AN node selection information entries and is coded according to figure 5.12.2.20 and table 5.12.2.20.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+5)* octet (l+6)* |  |
| Contents of N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE |  |  |  |  |  |  |  |  | octet (l+7)* octet l0* |  |

Figure 5.12.2.18: N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE

Table 5.12.2.18: N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE

Contents of N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE (octet l+7 to l01): The contents of N3IWF identifier configuration for 5G ProSe multi-hop layer-3 remote UE shall be encoded as the encoding of home N3IWF identifier configuration defined in clause 5.3.3.3 of 3GPP TS 24.526 [11].

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+1)* octet (l0+2)* |  |
| Contents of 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information |  |  |  |  |  |  |  |  | octet (l0+3)* octet m* |  |

Figure 5.12.2.19: 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information

Table 5.12.2.19: 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information

Contents of 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information (octet l0+3 to m): The contents of 5G ProSe multi-hop layer-3 UE-to-network relays access node selection information shall be encoded as the encoding of N3AN node selection information defined in clause 5.3.3.2 of 3GPP TS 24.526 [11].

NOTE: In this release of specification, the "preference" bit (as shown in figure 5.3.3.2.2 of 3GPP TS 24.526 [11]) is always set to "0".

==============Next change==============

### 5.14.2	Information elements coding

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 | 0 | PAI | ProSeP info type = {UE policies for 5G ProSe multi-hop end UE} |  |  |  |  | octet k |  |
| Spare |  |  |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  |  |  | octet k+1 octet k+2 |  |
| Validity timer |  |  |  |  |  |  |  |  |  | octet k+3 octet k+7 |  |
| Served by NG-RAN |  |  |  |  |  |  |  |  |  | octet k+8 octet o1 |  |
| Not served by NG-RAN |  |  |  |  |  |  |  |  |  | octet o1+1 octet o2 |  |
| Default destination layer-2 IDs for sending and receiving the discovery signalling for solicitation and for receiving the discovery signalling for announcement |  |  |  |  |  |  |  |  |  | octet o2+1 octet o3 |  |
| RSC info list |  |  |  |  |  |  |  |  |  | octet o3+1 octet l |  |
| 5G PKMF address information |  |  |  |  |  |  |  |  |  | octet (l+1)* octet o6* |  |

Figure 5.14.2.1: ProSeP Info = {UE policies for 5G ProSe multi-hop end UE}

Table 5.14.2.1: ProSeP Info = {UE policies for 5G ProSe multi-hop end UE}

ProSeP info type (bit 1 to 4 of octet k) shall be set to “1100” (UE policies for 5G ProSe multi-hop end UE)

PKMF address indication (PAI) (bit 5 of octet k)

The PAI indicates whether the 5G PKMF address information is included in the IE or not

Bit 5

0 5G PKMF address information is not included

1 5G PKMF address information is included

Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents.

Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe multi-hop end UE. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.14.2.2 and table 5.14.2.2, and contains configuration parameters for 5G ProSe multi-hop end UE when the UE is served by NG-RAN.

Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.14.2.3 and table 5.14.2.3, and contains configuration parameters for 5G ProSe multi-hop UE-to-UE relay discovery and communication when the UE is not served by NG-RAN. Default destination layer-2 IDs for sending and receiving the discovery signalling for solicitation and for receiving the discovery signalling for announcement (octet o2+1 to o3): The default destination layer-2 IDs for sending and receiving the discovery signalling for solicitation and for receiving the discovery signalling for announcement is coded according to figure 5.14.2.9b and table 5.14.2.9b and contains a list of the default destination layer-2 IDs for the initial UE-to-UE relay discovery signalling.

RSC info list (octet o3+1 to l): The RSC info list field is coded according to figure 5.14.2.10 and table 5.14.2.10 and contains the RSCs related paramters.

5G PKMF address information (octet l+1 to o6): The 5G PKMF address information contains the IPv4 address(es), IPv6 address(es) and/or FQDN of the 5G PKMF and is coded according to figure 5.5.2.21, figure 5.5.2.22, figure 5.5.2.23 and table 5.5.2.21. At least one of the address parameters (FQDN, IPv4 address list or IPv6 address list) shall be included.

If the length of ProSeP info contents field is bigger than indicated in figure 5.14.2.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| Authorized PLMN list for layer-3 end UE |  |  |  |  |  |  |  | octet k+10 octet o1 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | SNAI | octet o1+1 |

Figure 5.14.2.2: Served by NG-RAN

Table 5.14.2.2: Served by NG-RAN

| Authorized PLMN list for layer-3 end UE: The authorized PLMN list for layer-3 end UE field is coded according to figure 5.13.2.3 and table 5.13.2.3. |  |
| --- | --- |
| If the length of Served by NG-RAN field is bigger than indicated in figure 5.5.2.2, receiving entity shall ignore any superfluous octets located at the end of the Served by NG-RAN. |  |
| Subscribed SNPN authorization indicator (SNAI) (octet o1+1 bit 1) |  |  |
| Bit |  |  |
| 1 |  |  |
| 0 |  | Subscribed SNPN is not authorized |
| 1 |  | Subscribed SNPN is authorized |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery |  |  |  |  |  |  |  | octet o1+3 octet o51 |
| NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication |  |  |  |  |  |  |  | octet o51+1 octet o10 |
| Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery |  |  |  |  |  |  |  | octet o10+1 octet o2-1 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | L3EUI | octet o2 |

Figure 5.14.2.3: Not served by NG-RAN

Table 5.14.2.3: Not served by NG-RAN

NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery (octet o1+3 to o51): The NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery field is coded according to figure 5.14.2.4 and table 5.14.2.4.

NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication (octet o51+1 to o2): The NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication field is coded according to figure 5.14.2.5 and table 5.14.2.5.

Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery (octet o10+1 to o2-1): The default PC5 DRX configuration for multi-hop UE-to-UE relay discovery field is coded according to figure 5.14.2.9a and table 5.14.2.9a.

5G ProSe multi-hop layer-3 end UE when not served by NG-RAN indicator (L3EUI) (octet o2 bit 1): The L3EUI bit indicates whether the UE is authorized to use a 5G ProSe layer-3 UE-to-UE relay UE when not served by NG-RAN. Bit 1 0 Not authorized 1 Authorized

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.14.2.3, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery contents |  |  |  |  |  |  |  | octet o1+3 octet o1+4 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o1+5 octet o510 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o510+1)* octet o511* |
| ... |  |  |  |  |  |  |  | octet (o511+1)* octet o512* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o512+1)* octet o51* |

Figure 5.14.2.4: NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery

Table 5.14.2.4: NR radio parameters per geographical area list for multi-hop UE-to-UE relay discovery

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.14.2.6 and table 5.14.2.6.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication contents |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet o51+3 octet o513 |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o513+1)* octet o514* |
| ... |  |  |  |  |  |  |  | octet (o514+1)* octet o515* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o515+1)* octet o10* |

Figure 5.14.2.5: NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication

Table 5.14.2.5: NR radio parameters per geographical area list for multi-hop UE-to-UE relay communication

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.14.2.6 and table 5.14.2.6.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o510+1 octet o510+2 |
| Geographical area |  |  |  |  |  |  |  | octet o510+3 octet o5100 |
| Radio parameters |  |  |  |  |  |  |  | octet o5100+1 octet o511-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o511 |

Figure 5.14.2.6: Radio parameters per geographical area info

Table 5.14.2.6: Radio parameters per geographical area info

Geographical area (octet o510+3 to o5100): The geographical area field is coded according to figure 5.14.2.7 and table 5.14.2.7.

Radio parameters (octet o5100+1 to o511-1): The radio parameters field is coded according to figure 5.3.2.11 and table 5.3.2.11, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o511 bit 8): The managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.14.2.6, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o510+3 octet o510+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o510+5)* octet (o510+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o510+11)* octet (o510+16)* |
| ... |  |  |  |  |  |  |  | octet (o510+17)* octet (o510-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o510-1+6*n)* octet (o510+4+6*n)* = octet o5100* |

Figure 5.14.2.7: Geographical area

Table 5.14.2.7: Geographical area

Coordinate: The coordinate field is coded according to figure 5.14.2.8 and table 5.14.2.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o510+11 octet o510+13 |
| Longitude |  |  |  |  |  |  |  | octet o510+14 octet o510+17 |

Figure 5.14.2.8: Coordinate area

Table 5.14.2.8: Coordinate area

Latitude (octet o510+11 to o510+13): The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude (octet o510+14 to o510+17): The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o5100+1 octet o5100+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o5100+3 octet o511-1 |

Figure 5.14.2.9: Radio parameters

Table 5.14.2.9: Radio parameters

Radio parameters contents (octet o5100+3 to o511-1): Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default PC5 DRX configuration for multi-hop UE-to-UE relay discovery contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery contents |  |  |  |  |  |  |  | octet o10+3 octet o2-1 |

Figure 5.14.2.9a: Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery

Table 5.14.2.9a: Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery

Default PC5 DRX configuration contents for multi-hop UE-to-UE relay discovery: Default PC5 DRX configuration for multi-hop UE-to-UE relay discovery field is coded as sl-DefaultDRX-GC-BC-r17 in clause 6.3.5 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of default destination layer-2 IDs for sending and receiving the discovery signalling for solicitation and for receiving the discovery signalling for announcement |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| Default destination layer-2 ID 1 |  |  |  |  |  |  |  | octet o2+3 octet o2+5 |
| Default destination layer-2 ID 2 |  |  |  |  |  |  |  | octet (o2+6)* octet (o2+8)* |
| ... |  |  |  |  |  |  |  | octet (o2+9)* octet (o3-3)* |
| Default destination layer-2 ID n |  |  |  |  |  |  |  | octet (o3-2)* octet o3* |

Figure 5.14.2.9b: Default destination layer-2 IDs for sending and receiving the discovery signalling for solicitation and for receiving the discovery signalling for announcement

Table 5.14.2.9b: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement

Default destination layer-2 ID (octet o2+3 to o2+5): The default destination layer-2 ID is a 24-bit long bit string.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info list contents |  |  |  |  |  |  |  |  | octet o3+7 octet o3+8 |  |
| RSC info 1 |  |  |  |  |  |  |  |  | octet o3+9 octet o6 |  |
| RSC info 2 |  |  |  |  |  |  |  |  | octet (o6+1)* octet o7* |  |
| … |  |  |  |  |  |  |  |  | octet (o7+1)* octet o8* |  |
| RSC info n |  |  |  |  |  |  |  |  | octet (o8+1)* octet o4* |  |

Figure 5.14.2.10: RSC info list

Table 5.14.2.10: RSC info list

RSC info: The RSC info field is coded according to figure 5.14.2.11 and table 5.14.2.11.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC info contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o30 octet o30+1 |  |
| CPSI |  | SLPI |  | MNHI |  | MHI |  | TT |  |  | LI |  |  | octet o30+2 |  |
| RSC list |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o30+3 octet o31 |  |
| Maximum number of hops |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o31+1)* octet o32* |  |
| Security related parameters for multi-hop UE-to-UE relay discovery |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o32+1)* octet o33* |  |
| NR-PC5 multi-hop UE-to-UE relay security policies |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o33+1)* octet o34* |  |

Figure 5.14.2.11: RSC info

Table 5.14.2.11: RSC info

Layer indication (LI) (octet o30+2 bit 1 to 2): Bits 2 1 0 1 Layer 3 (NOTE 1) 1 0 Layer 2

Traffic type (TT) (octet o30+2 bit 3 to 4): The traffic type field indicates the traffic type of the relayed service and exists when the LI is set to "Layer 3". Bits 4 3 0 0 No information 0 1 IP 1 0 Ethernet 1 1 Unstructured Multi-hop indicator (MHI) (octet o30+2 bit 5): Bits 5 0 Multi-hop relay is not allowed (NOTE 2) 1 Multi-hop relay is allowed Maximum number of hops indicator (MNHI) (octet o30+2 bit 6): Bits 6 0 Maximum number of hops is not indicated 1 Maximum number of hops is indicated

Security related parameters indication (SLPI) (octet o30+2 bit 7): The Security related parameters indication indicates whether the security related parameters for UE-to-UE relay discovery field is included or not. Bits 7 0 The security related parameters for multi-hop UE-to-UE relay discovery not included 1 The security related parameters for multi-hop UE-to-UE relay discovery included

Control plane security indication (CPSI) (octet o30+2 bit 8): The control plane security indication field indicates whether to use the security procedure over control plane as specified in 3GPP TS 33.503 [13], if the network assistance security indication indicates the security procedure with network assistance is to be used. Bit 6 0 security procedure over control plane is not used 1 security procedure over control plane is used

RSC list (octet o30+3 to o31): The RSC list field is coded according to figure 5.14.2.12 and table 5.14.2.12.

Maximum number of hops (octet o31+1 to octet o32) Maximum number of hops is a binary coded representation of the maximum number of hops can be supported for the associated RSC.

Security related parameters for multi-hop UE-to-UE relay discovery (octet o32+1 to o33): The security related parameters for multi-hop UE-to-UE relay discovery field contains the security related parameters for multi-hop UE-to-UE relay discovery used when the security procedure over control plane as specified in 3GPP TS 33.503 [13] is used and is coded as security related parameters for discovery defined in figure 5.5.2.15 and table 5.5.2.15. NR-PC5 multi-hop UE-to-UE relay security policies (octet o33+1 to o34): The NR-PC5 multi-hop UE-to-UE relay security policies are coded as the NR-PC5 unicast security policies defined in figure 5.4.2.36 and table 5.4.2.36.

NOTE 1: In this release, LI bit shall be set to “Layer3”. NOTE 2 In this release, MHI bit shall be set to “Multi-hop relay is allowed”.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RSC list contents |  |  |  |  |  |  |  |  | octet o52+3 octet o52+4 |  |
| RSC 1 |  |  |  |  |  |  |  |  | octet o52+5 octet o52+7 |  |
| RSC 2 |  |  |  |  |  |  |  |  | octet (o52+8)* octet (o52+10)* |  |
| … |  |  |  |  |  |  |  |  | octet (o52+11)* octet (o520-3)* |  |
| RSC n |  |  |  |  |  |  |  |  | octet (o520-2)* octet o520* |  |

Figure 5.14.2.12: RSC list

Table 5.14.2.12: RSC list

RSC (octet o52+5 to o52+7): The RSC identifies a connectivity service the UE-to-UE relay provides. The value of the RSC is a 24-bit long bit string. The values of the RSC from "000001" to "00000F" in hexadecimal representation are spare and shall not be used in this release of specification. The UE shall ignore the spare value of the RSC in this release of specification. For all other values, the format of the RSC is out of scope of this specification.

==============End of change==============