3GPP TSG-CT WG1 Meeting #159	C1-260232

Goa, India, 9th – 13th February 2026

Source:	vivo

Title:	Discussion on Rel-20 work on 5GSAT_Ph4-ARC

Document for:	INFORMATION

Agenda item:	20.2.2

Abstract of the contribution: This contribution summarizes the progress of 5GSAT_Ph4-ARC for Rel-20 in SA2, analyses the possible stage 3 work needed in CT WGs and provides our suggestion on how to proceed related CT work.

1 Introduction

SA2 has studied Integration of satellite components in the 5G architecture Phase 4 (5GSAT_Ph4-ARC) for Rel-20 and the corresponding study item can be found in SP-250400. TR 23.700-19 specifies a total 29 solutions and the 85% of this study item has been done after SA#110 (Dec. 2025).

Work item 5GSAT_Ph4-ARC for SA2 normative work was approved in SA#110 (see SP-251703).

This paper analyses the possible stage 3 work needed in CT WGs and provides our suggestion on how to proceed related CT work.

2 Possible stage 3 work needed in CT WGs

2.1 Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

This key issue focuses on the system enhancements and QoS enhancements to support IMS voice service over NB-IoT NTN via GEO satellite connecting to EPC. Conclusions for Key Issue #1 captured in TR 23.700-19 include both voice packets and IMS signalling shall be transported over the NB-IoT (GEO) user plane using a single PDN connection. Two main options are identified for transporting voice packets: as IP data or as non-IP data. The below table provide the corresponding CT impacts for each option, as well as the common CT impacts.

NOTE:	The green highlight represents the impact on CT1, the blue highlight represents the impact on CT3, the yellow highlight represents the impact on CT4.

Table 1: The possible impacts on CT WGs based on the conclusions for KI#1

| Conclusions in SA2 TR 23.700-19 | SA2 scheme options | Corresponding CT aspects |  | TS/CT WG(s) |
| --- | --- | --- | --- | --- |
| The following conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made: - The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U. - The IMS signalling shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U. - A single PDN connection shall be used to transport both IMS signalling and IMS voice. | 1. Transporting voice media traffic with UDP/IP header | Capability negotiation | Potential capability negotiation between the UE and the MME, e.g., for IMS voice over NB-IoT during the Attach and TAU procedure. | CT1: TS 24.301 |
|  |  | Subscription check | Potential subscription check for IMS voice over NB-IoT (GEO) supported. | CT4: TS 29.272 |
|  |  | Support dedicated EPS bearer for NB-IOT | Potential update on PGW and PCRF to support the pre-establishment of a dedicated EPS bearer for IMS voice. | CT3: TS 29.212 CT4: TS 29.274 |
|  | 2. Transporting voice media traffic without UDP/IP header | Capability negotiation | Potential capability negotiation between the UE and the network for non-IP packets transport via NAS or IMS procedure. Potential interaction between NAS/IMS and AS layer for non-IP packets transport. | CT1: TS 24.301/TS 24.008/TS 24.229 |
|  |  | Voice packets transmission | The handling for UE and PGW to associate the non-IP data with the dedicated bearer: - Potential update on TFT to enable the UE to associate the non-IP data with the dedicated bearer. - Potential update on the UE and PGW handling for adding/removing UDP/IP header when receiving and sending data for dedicated bearer of IP type PDN Connection. | CT1: TS 24.008 CT1: TS 24.229/TS 24.301 CT4: TS 29.281 |
|  |  |  | The RTP header handling at UE and AGW. | SA4: It depends on the RTP payload format for a new codec to be defined in SA4. |
|  | 3. Common aspects | QoS enhancement | Potential new QCI for the dedicated EPS bearer used for voice traffic over NB-IoT NTN. | CT1: TS 24.301 CT3: TS 29.212 |
|  |  | IMS timers | Potential update on SIP timers. | CT1 TS 24.229 |
|  |  | Clarification on restrictions for NB-IoT | Increase maximum number of active user plane radio bearers for NB-S1 mode. | CT1: TS 24.301 |
|  |  |  | Clarify the dedicated EPS bearer contexts activation procedure may be used for NB-S1 mode. |  |

Observation 1: It is foreseen that CT1, CT3, and CT4 WG are impacted by KI#1 and the CT impacts relate to EPS enhancements for transporting IMS signalling and voice packets over user plane.

2.2 Key Issue #2: IMS enhancement for GEO NB-IoT NTN access

This key issue focuses on IMS enhancements to enable the usage of NB-IoT as the IP-CAN to meet the SA1 performance requirements for IMS voice call setup time using GEO satellite. Since no conclusion has yet been reached for Key Issue #2, the potential CT impact are listed based on analysis of all 13 solutions documented in the TR 23.700-19 and discussion progress of SA2 meetings. The IMS enhancements mainly include two aspects:

-	IMS signalling optimization based on text-encoded protocol (i.e., SIP), summarized in Table 2-1; and

-	IMS signalling optimization based on binary-encoded protocol, summarized in Table 2-2.

Table 2-1: IMS signalling optimization based on text-encoded protocol

| SA2 schemes | Details | Associated solutions | Corresponding CT aspects | TS/CT WG(s) |
| --- | --- | --- | --- | --- |
| SIP message size reduction | Fixed SIP fields may be omitted by the UE and completed by the P-CSCF | Solution#12, 15 | The mandatory IEs defined by RFC 3261 may be omitted and shall be supplied by the P-CSCF. Note: Any additional IEs that may be omitted could be evaluated by CT1 and specified as necessary. | CT1: TS 24.229 |
|  | Support SigComp | Solution#12, 13, 17 | Potential clarification on that the UE shall enable SigComp negotiation when it is accessing network via NB-IoT (GEO). | CT1: TS 24.229 |
| SIP message number reduction | Support SIP INVITE with no pre-condition | Solution#12, 13, 15, 17 | The UE and the P-CSCF shall support handling of SIP INVITE messages without precondition if pre-condition is disabled. | CT1: TS 24.229 |
|  | Early configuration of SDP parameters | Solution#16 | Potential pre-defined SDP parameters defined for NB-IoT (GEO). | CT1: TS 24.229 |
| Potential IMS interaction procedure for the NB-IOT(GEO) |  | Solution#12, 14, 15, 16, 17 | Potential stage 3 work based on SA2 progress. | CT1: TS 24.229 CT4: TS 29.175 /TS 29.176 |

Table 2-2: IMS signalling optimization based on binary-encoded protocol

| SA2 schemes | Associated solutions | Corresponding CT aspects | TS/CT WG(s) |
| --- | --- | --- | --- |
| Binary-encoded protocol between UE and P-CSCF for call setup signalling | Solution#4, 18, 19, 20 | Potential support of SIP session management using binary-encoded protocol, e.g., new Call Setup procedure and new message need to be design, P-CSCF need to support translating between text-encoded message and binary-encoded message. | CT1: new spec for IMS signalling optimization based on binary-encoded protocol |

In addition, the below impacts are common to both options and may bring changes to TS 24.229:

-	Potential capability negotiation of using binary-encoded protocol during SIP REGISTER procedure.

-	The handling of ALG in the P-CSCF/AGW/MRF for perform codec transcoding.

Observation 2: It is foreseen that CT1 and CT4 WG are impacted by KI#2 and the CT impacts relate to IMS signalling optimization for reducing IMS call setup time.

Besides, IMS signalling optimization involves the design of binary-encoded IMS protocol. Given that IMS has historically been designed using text-encoded and binary-encoded IMS protocol design is a brand new concept, that is expected to require a significantly higher standardization effort for the definition of new procedures and message formats.

2.3 Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC

This key issue focuses on how to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

Since no conclusion has yet been reached for Key Issue #3, the potential CT impact are listed based on five solutions documented in the TR 23.700-19.

Table 3: The possible impacts on CT based on the solutions for KI#3

| SA2 schemes | Associated solutions | Corresponding CT aspects | TS/CT WG(s) |
| --- | --- | --- | --- |
| Support emergency call based on text-encoded protocol | Solution#4, 21, 22, 23, 25 | Potential handling of UE and P-CSCF for IMS emergency call setup without IMS emergency registration if RAT type is NB-IoT (GEO). Potential handling of P-CSCF to support continuation of emergency call setup when initiated as a normal call over NB-IoT (GEO) access in non-UE detectable emergency session. Potential emergency support indication from the MME to the UE accessing over NB-IoT NTN within Attach and TAU Accept. | CT1: TS 24.229 CT1: TS 24.229 CT1: TS 24.301 |
| Support emergency call based on binary-encoded protocol | Solution#24 | Potential support of emergency-related fields in the binary-encoded protocol. | CT1: new spec for IMS signalling optimization based on binary-encoded protocol |

Observation 3: It is foreseen that CT1 is impacted by KI#3 and the CT impacts relate to IMS signalling optimization for emergency call to reduce IMS call setup time.

  - Key Issue #4: Location service for IMS emergency call and regulatory services over NB-IoT NTN

This key issue focuses on whether existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements. According to three solutions documented in the TR 23.700-19, the E-CSCF can obtain the UE location using existing procedure.

Observation 4: No fundamental changes to the CT specifications are required. Potential changes are expected to be limited to the addition of clarification specific to NB-IoT (GEO).

2.5 Key Issue #5: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

This key issue studies how to support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services (e.g., 5G-LAN services with IP and Ethernet PDU Session type). According to agreed principles for KI#5 documented in the TR 23.700-19, existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused to trigger UE-SAT-UE communication.

Observation 5: No fundamental changes to the specifications are required. Potential changes are expected to be limited to the addition of clarification specific to NB-IoT (GEO).

3 Suggestion on how to proceed related CT work

Based on the above analyses in section 2, the stage 3 work of 5GSAT Ph4-ARC comprises two distinct aspects: (1) EPS enhancements for transporting IMS signalling and voice packets over user plane; (2) IMS signalling optimizations for reducing IMS call setup time.

Considering most of EPS/IMS (SIP/binary-encoded protocol) work need to be carried out in CT1, it is suitable for CT1 to take the lead of stage 3 work on 5GSAT_Ph4-ARC.

The EPS enhancements mainly focus on the user plane transport of IMS signalling and voice packets, while IMS signalling optimizations aim to minimize IMS call setup latency. These two technical aspects serve fundamentally different purposes, maintain a clear and well-defined standardization boundary as defined in 3GPP specifications.

According to the CT1 working procedures, the contributions related to EPS enhancements and IMS signalling optimizations are discussed in separate sessions, as the NAS and SIP protocols always need delegates with different expertise to do the standardized work.

Observation 6: The stage 3 work on EPS enhancements and IMS signalling optimizations are two distinct technical aspects and are discussed in separate sessions, with no dependency between them.

Based on the observations from the SA2 discussion, the total message size for the IMS call setup procedure can be reduced from approximately 11000 bytes to around 2000 bytes by applying different IMS signalling simplification approaches. In particular, when a binary-encoded IMS signalling optimization approach is used, the total message size for the IMS call setup procedure can be further reduced to below 1000 octets.

The large overhead of IMS signalling degrades the transport efficiency of voice packets and poses more challenges to radio link quality and transmission delay over the radio interface, especially in high-delay satellite environments. Reducing IMS signalling overhead is therefore important to ensure voice quality.

Observation 7: IMS signalling optimizations can significantly reduce the total message size of the IMS call setup procedure, and can consequently reduce the transmission delay and improve voice quality.

Besides, IMS signalling optimizations can be applicable to broader scenarios (e.g., TN scenario), as their benefits are generic rather than scenario-specific. In addition, some existing features (e.g., CIoT EPS optimization, NAS overhead reduction) were initially specified for particular scenario but the applicability were extended during the normative work phase. In this respect, it is reasonable to consider IMS signalling optimizations as applicable to broader scenarios as well.

Observation 8: IMS signalling optimizations are applicable to broader scenarios (e.g., TN scenario) not just NB-IoT GEO voice and are future-proof.

To facilitate and ensure the progress and successful completion of the stage 3 normative work, it proposed to undertake the stage 3 work in two WIDs.

Proposal 1: It proposes to undertake the stage 3 work of 5GSAT Ph4-ARC in two CT WG WIDs.

The SA2 SID on 5GSAT_Ph4-ARC is not stable enough and is expected to be completed in SA#111 (Mar. 2026). After the completion of SA2#173, the related CT WG WID can be discussed and approved in Apr. 2026 (CT1#160) and in Jun. 2026 (CT#112), then the stage 3 work of CT WG may be started accordingly.

Proposal 2: It proposes to follow SA2 progress for 5GSAT_Ph4-ARC closely and the related CT WG WIDs can be discussed and approved in Apr. 2026 and CT plenary in Jun. 2026, then the stage 3 work of CT WG can start accordingly.

4 Conclusion

Based on the above discussion, following observations are provided:

Observation 1: It is foreseen that CT1, CT3, and CT4 WG are impacted by KI#1 and the CT impacts relate to EPS enhancements for transporting IMS signalling and voice packets over user plane.

Observation 2: It is foreseen that CT1 and CT4 WG are impacted by KI#2 and the CT impacts relate to IMS signalling optimization for reducing IMS call setup time.

Observation 3: It is foreseen that CT1 is impacted by KI#3 and the CT impacts relate to IMS signalling optimization for emergency call to reduce IMS call setup time.

Observation 4: No fundamental changes to the CT specifications are required. Potential changes are expected to be limited to the addition of clarification specific to NB-IoT (GEO).

Observation 5: No fundamental changes to the specifications are required. Potential changes are expected to be limited to the addition of clarification specific to NB-IoT (GEO).

Observation 6: The stage 3 work on EPS enhancements and IMS signalling optimizations are two distinct technical aspects and are discussed in separate sessions, with no dependency between them.

Observation 7: IMS signalling optimizations can significantly reduce the total message size of the IMS call setup procedure, and can consequently reduce the transmission delay and improve voice quality.

Observation 8: IMS signalling optimizations are applicable to broader scenarios (e.g., TN scenario) not just NB-IoT GEO voice and are future-proof.

Based on above observations, the following proposals are provided:

Proposal 1: It proposes to undertake the stage 3 work of 5GSAT Ph4-ARC in two CT WG WIDs.

Proposal 2: It proposes to follow SA2 progress for 5GSAT_Ph4-ARC closely and the related CT WG WID can be discussed and approved in Apr. 2026 and CT plenary in Jun. 2026, then the stage 3 work of CT WG can start accordingly.