3GPP TSG-CT WG1 Meeting #157	C1-256705

Sophia Antipolis, France, 13-17 October 2025                                      was C1-2563136

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.283 | CR | 0003 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN on X-3GPP-P-Asserted-Identity. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMCLoc |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Request to the LMS may come from a LMC or an network entity such as another LMS or an MC service server. The change makes it possible to for a network entity to be auhorized to send a request to the LMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change of x-3gpp-asserted-identity header field name to lower case, since HTTP/2 requires that header fields are in lower case. Adding a requirement on how to identity the sender of the request if the request comes from a network entity. The above changes are applicable in both 6.3.2.3.2 and 6.3.2.5.2. Fixing a style issue in heading 6.3.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be clear on how to authorize a network entitty to send requests to LMS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.2, 6.3.2.5.2, 6.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Clause 6.3.2.6 includes only unchanged sub clauses. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.3.2.3.2	Client request subscription using LocationInfoSubscribe operation

To request a subscription for location information, the client shall send an HTTP POST message to the LMS on LMS_Information collection resource (i.e, Location information subscription) to create the resources associated with the subscription in the LMS.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE 1:	The LocationInfoSubscribe operation is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an x-3gpp-asserted-identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the x-3gpp-asserted-identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the LocationSubscriptionRequest as specified in clause 7.2.6.2.5.

The LocationSubscriptionRequest data object shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location notifications shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; or

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested; and

4)	an expTime attribute containing the time and date for the subscription expiry;

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LMS_LocationInformation_request;

3)	if the received HTTP request does not contain an Authorization header field with the "Bearer" authentication scheme and a bearer access token and the received HTTP request contains an x-3gpp-asserted-identity header field, the LMS shall consider the URI in the x-3gpp-asserted-identity header field as the identity of the sender of the HTTP request.

4)	if the received HTTP request does contain an Authorization header field with the "Bearer" authentication scheme and bearer access token then:

a)	if the received request also includes an x-3gpp-asserted-identity header field, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps; and

b)		if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

5)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid subscription request target client list" in a Warning header field, and shall not continue with the rest of the steps;

6)	if the MC user's location user configuration data document does not include a <rule> element including a <action> element including a <allow-subscription-on-location-information> element set to true the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Unauthorized for location subscription" in a Warning header field, and shall not continue with the rest of the steps;

7)	generate a unique subscription identifier;

8)	create a new resource with the location subscription request as specified in clause 7.2.3.3.3; and

9)	send a HTTP 201 Created response message to the LMC and include the subscription identifier in the Location header;

The LMS shall identify all MC service IDs for which the subscription request applies according to step 6-11 in the procedure for a one-time location information request as specified in clause 6.3.2.2.2.

The LMS shall create location information notifications and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message shall contain location information for the MC UEs of the identified MC service IDs.

The identified MC service IDs may change during the subscription time if the MC service IDs are determine based on affiliated group members or activated functional aliases.

The notifications shall be sent repeatedly until the subscription expires based on the attribute expTime or until a unsubscribe message is received.

* * * Next Change * * * *

##### 6.3.2.5.2	Client requests update of subscription using LocationInfoSubscribeUpdate operation

To update an existing subscription the client shall send an HTTP PUT message to the LMS on the resource URI identifying the individual LMC subscription resource representation as specified in clause 7.2.3.4.2. The HTTP PUT message shall replace all properties of the existing resource with the LMS Location information subscribe request.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE 1:	The LocationInfoSubscribeUpdate operation is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X- x-3gpp-asserted-identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the x-3gpp-asserted-identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP PUT message shall include the LocationSubscriptionRequest datatype as specified in clause 7.2.6.2.5.

The LocationSubscriptionRequest data object shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location notifications shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; or

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested; and

4)	an expTime attribute containing the time and date for the subscription expiry;

Upon receiving the HTTP PUT message from the LMC, the LMS shall:

1)	validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LMS_LocationInformation_request;

3)	if the received HTTP request does not contain an Authorization header field with the "Bearer" authentication scheme and a bearer access token and the received HTTP request contains an x-3gpp-asserted-identity header field, the LMS shall consider the URI in the x-3gpp-asserted-identity header field as the identity of the sender of the HTTP request.

4)	if the received HTTP request does contain an Authorization header field with the "Bearer" authentication scheme and bearer access token then:

a)	if the received request also includes an x-3gpp-asserted-identity header field, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps; and

b)

if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

5)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid subscription request target client list" in a Warning header field, and shall not continue with the rest of the steps;

6)	if the MC user's location user configuration data document does not include a <rule> element including a <action> element including a <allow-subscription-on-location-information> element set to true the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Unauthorized for location subscription" in a Warning header field, and shall not continue with the rest of the steps;

7)	update the complete subscription resource with the location subscription information as specified in clause 7.2.3.3.3; and

8)	send a HTTP 204 No Content response message to the LMC.

The LMS shall identify all MC service IDs for which the subscription request applies according to step 6-11 in the procedure for a one-time location information request as specified in clause 6.3.2.2.2.

The LMS shall create location information notifications and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message shall contain location information for the MC UEs of the identified MC service IDs.

The identified MC service IDs may change during the subscription time if the MC service IDs are determine based on affiliated group members or activated functional aliases.

#### The notifications shall be sent repeatedly until the subscription expires based on the attribute expTime or until a unsubscribe message is received.

#### 6.3.2.6	LocationInfoSubscribeDelete