3GPP TSG-CT WG1 Meeting #157	C1-256468

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on AIoT NAS message ciphering updates

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

As stage 2 security requirements have progressed and captured in 33.369 v1.0.0, corresponding updates can be done related to ciphering of AIoT NAS messages on 24.369.

For the second editor’s note in clause 4.2.5, on which part of AIoT NAS messages are ciphered, it is proposed to align to the proposal adding a Security header IE and a MAC IE as leading contents of security protected AIoT NAS messages. These security parameters are unciphered and ciphering starts with the next IE until the end of the message, i.e. from Message type IE until the end of the message. It is proposed to clarify this, and delete the related Editor’s note.

3. Conclusions

Clause 4.2.5 can be updated and aligned to stage 2 and a AIoT NAS message coding proposal for security parameters in AIoT NAS messages.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 4.2.5	Ciphering of AIoT NAS signalling messages

The use of ciphering in a network is an operator option subject to AIOTF configuration. When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0.

All AIoT NAS messages, except the INVENTORY REPORT message, shall be sent ciphered. If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered.

Editor's note: It is FFS whether other AIoT NAS messages than INVENTORY REPORT message can be sent unciphered.

The algorithm to calculate the ciphering of AIoT NAS messages is described in annex D.2 in 3GPP TS 33.501 [9], and the ciphered data shall include octets 6 to n of the AIoT NAS messages. In addition to the data that is to be ciphered, the constant BEARER, DIRECTION bit, COUNT, KEY and LENGTH are input to the ciphering algorithm. These parameters are described in 3GPP TS 33.369 [6].

* * * End of Changes * * *