3GPP TSG-CT WG1 Meeting #157	C1-256618

# Sophia Antipolis, France, 13-17 October 2025

Title:	LS on information provided by the lower layers for AIoT device

Response to:	-

Release:	Release19

Work Item:	AmbientIoT-CT

Source:	CT1

To:	RAN2, RAN3

Cc:	SA2, SA3

Contact Person:

Name:	Yizhong Zhang

Tel. Number:	-

E-mail Address:	yizhong.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

CT1 finds the following information is used by the AIoT NAS layer, but is provided to the AIoT NAS layer by the lower layers:

1)	the AIoT identification information

According to TS 23.369 v19.1.0 and TS 33.369 v19.0.0, the AIoT identification information is used to determine whether the AIoT device is matched or not by the AIoT NAS layer. The AIoT identification information can include an AIoT device permanent identity, a T-ID, or a filtering information, see follows:

<clause 6.2.2 of TS 23.369>

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

<clause 5.4.1 of TS 33.369>

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

<clause 5.4.3 of TS 33.369>

In step 2, 3 and 4, the T-ID shall be used as a device identification information.

Besides, the type of AIoT identification information is needed for AIoT NAS layer as the AIoT NAS layer needs to distinguish which information is included in the AIoT identification information

2)	the RANDAIOT_n

According to TS 33.369 v19.0.0, the RANDAIOT_n  is used for the authentication procedure, see follows:

<clause 5.2.2 of TS 33.369>

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

3. NG-RAN shall include RANDAIOT_n in  the paging request message to the AIoT device in addition to other device identification information.

3)	the T-ID handling indication

According to TS 33.369 v19.0.0, the T-ID is used to protect the privacy of AIoT device during the inventory procedure, see follows:

<clause 5.4.3 of TS 33.369>

-	In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message.

4)	the ciphering algorithms

According to TS 33.369 v19.0.0, the ciphering is optionally supported for the AIoT command messages. When the cyphering is activated, the AIoT NAS layer can obtain an indication from the lower layers on the ciphering algorithms, see follows:

<clause 5.3.2 of TS 33.369>

3.	The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN.

<clause 4.2.5 of TS 24.369>

If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered.

CT1 kindly asks RAN2 and RAN3 to check if they can provide above information to the AIoT NAS layer.

2. Actions:

To RAN2 and RAN3 group.

ACTION:	CT1 kindly asks RAN2 and RAN3 to check if they can provide such information to the AIoT NAS layer.

3. Date of Next CT1 Meetings:

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2026..................India