3GPP TSG-CT WG1 Meeting #158	C1-257495

Dallas, US , 17-21 November 2025	Revision of C1-257199

Source:	China Mobile, Huawei, HiSilicon, OPPO, Ericsson

Title:	pCR on the EN in 6.2.2

Spec:	3GPP TS 24.369 v1.1.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to solve the EN in 6.2.2 according to Reply LS on the maximum supported AIoT NAS container length(C1-257038).

- Reason for Change

6.2.2	Message too long

Editor's note:	Maximum AIoT NAS message length is FFS and to be defined by RAN3.

In (TP to TS 38.413 BL CR) Detailed IE encoding and miscellaneous, RAN3 defines AIoT NAS PDU as OCTET STRING, without Semantics description. It is suggested to consider the maximum AIoT NAS message length according to RAN2 reply LS.

Reply LS on the maximum supported AIoT NAS container length:

...

In conclusion, as to the maximum supported AIoT NAS container length mentioned in the LS, RAN2 understands

For R2D upper layer data for a given command, the maximum size of one R2D NAS container is 119 bytes

For the response to a given command in D2R, the maximum size of one D2R NAS container is 125 bytes considering SA1 requirement

The above conclusion is only for Rel-19 Ambient IoT.

...

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.1.0.

* * * First Change * * * *

6.2.2	Message too long

The AIoT device and the AIOTF shall not send an AIoT NAS message larger than the maximum size to prevent exceeding the value range for upper layer data specified in 3GPP TS 38.391 [8].

NOTE:	In this release of the specification, the maximum size of an AIoT NAS message allowed to be sent by the AIoT device is 125 octets, while the maximum size of an AIoT NAS message allowed to be sent by the AIOTF is 119 octets.

* * * End of Changes * * * *