3GPP TSG-CT WG1 Meeting #159	C1-260559

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.571 | CR | 0109 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support for ExtendedFacility IE for 5G LCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicion |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The Facility information element is a type 4 element with a length field of 1 byte. The maximum size it can carry is 255 octets. This has been the size of the IE from rel-8. For 4G this size seemed enough. But for 5G LCS, the component of the LCS operation can carry a much higher size and for that it is required to extend the Facility information element. According to ASN.1 encoding in 24080-i50-ASN1, example IEs with large size as below: 1. referenceNumberExt, areaEventReporting, eventReportAllowedArea, multiplePositioningProtocolPDUs and userPlaneReportAFAddr IEs can exceed 255 octets. Additonally,there is no up limit for h-gmlc-callBackUri. Therefore, the component of the lcs-MOLR, lcs-PeriodicTriggeredInvoke, lcs-CancelDeferredLocation, lcs-MSCancelDeferredLocation and lcs-EventReport operations may exceed 255 octets that cannot be carried in a type 4 Facility IE and lead to the corresponding 5G LCS procedures failed. 2. rangingSLPPList IE in can go over 255 octets according to 24.080 and 38.355, and the relatedUEInfo IE can exceed 255 octets according to applicationLayerID definition in 23.304. Hence, the component of the lcs-SLMOLR, lcs-SLMTLR, lcs-DLRSPPTransport and lcs-ULRSPPTransport operations may exceed 255 octets that cannot be carried in a type 4 Facility IE and lead to the corresponding 5G LCS procedures failed. Therefore, it is proposed to enable the usage of ExtendedFacility IE to carry the lenght of component exceeding 255 octets for 5G LCS operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the support to use ExtendedFacility IE for 5G LCS operations when the UE and the network (all LMFs and the AMF) support the ExtendedFacility IE for 5G LCS and the length of the component exceeds 255 octets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The component of the 5G LCS operations can exceed 255 octets and lead to the corresponding procedure failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.2.1, 4.2.2, 5.2.1.1.1, 5.2.1.1.2, 5.2.1.3.2, 5.2.1.4.1, 5.2.1.4.2, 5.2.1.5.2, 5.2.1.7.2, 5.2.1.8.2, 5.2.2.1.1, 5.2.2.1.2, 5.2.2.2.2, 5.2.2.4.2, 5.2.2.5.2, 5.2.2.7.2, 5.2.2.8.2, 5.2.2.9.1, 5.2.2.9.2, 5.2.2.11.2, 5.3.1. |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 24.080 CR 0130 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The UE and NW capability negotaition is specified in TS 24.501 CR 6999 . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - updated 5GS to 5G in the title; - added co-signer and updated the linked CR#; - updated the term from “ExtendedFacility IE for 5GS LCS” to “ExtendedFacility IE for 5G LCS”; - reworded a bit in 4.2.1, 4.2.2 and 5.3.1. Rev 2: - Removed the linkage to CT1 CR and reflected it in "Other comments" |  |  |  |  |  |  |  |  |

==============First change==============

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ExtendedFacility IE for 5G LCS: Usage of ExtendedFacility information element in the LCS operations as specified in clause 5.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.261 [8] apply:

satellite NG-RAN

PRU ON/OFF state: This state is defined for a UE associated as a PRU with an LMF (i.e. after the successful completion of PRU association procedure). This state indicates whether the PRU functionality of the UE is activated (i.e. ON state) or deactivated (i.e. OFF state) while it is associated with an LMF.

UE operating as PRU: From the UE perspective, a UE which supports PRU functionality and operates as PRU. From the network perspective, the UE allowed to operate as a PRU based on the UE subscription data and the PRU state is ON.

==============Next change==============

### 4.2.1	UE support of LCS

The UE announces to the network its ability to support:

a)	LCS notification mechanism

b)	LPP messages;

c)	ExtendedFacility IE for 5G LCS; or

d)	any combination of a, b and c;

using the 5GMM capability IE defined in 3GPP TS 24.501 [3].

The UE supporting LCS notification mechanism and LPP messages may support PRU (defined in 3GPP TS 38.305 [7]). The UE’s ability to support PRU is stored in the UDM (see 3GPP TS 23.273 [2]) without any indication in 5GMM capability IE defined in 3GPP TS 24.501 [3].

A UE accessing 5G network via satellite NG-RAN shall not operate as a PRU.

==============Next change==============

### 4.2.2	Network support of LCS

The network announces to the UE its ability to support LCS in 5GC using the 5GS network feature support IE defined in 3GPP TS 24.501 [3]. The information is taken into account by the UE, in addition to UE's LCS capabilities, for the UE to determine whether to initiate MO-LR procedure in NG-RAN as specified in 3GPP TS 23.273 [3], clause 6.2.

If the ExtendedFacility IE for 5G LCS is supported homogeneously across all LMFs and is supported by the AMF, the network announces to the UE its ability to support ExtendedFacility IE for 5G LCS using the 5GS network feature support IE defined in 3GPP TS 24.501 [3].

NOTE:	An SNPN can provide Location Services except for the cases of interworking with EPC, and roaming, which are not supported in an SNPN.

==============Next change==============

##### 5.2.1.1.1	General

The supplementary services Location Notification operation enables the AMF to trigger the end-user notification verification process on the UE using NAS signalling. The supplementary services messages are transported using the DL NAS TRANSPORT message and the UL NAS TRANSPORT message defined in 3GPP TS 24.501 [3]. UE Location Privacy Indication information may be included in Location Notification Return Result to indicate whether subsequent LCS requests or ranging and sidelink positioning requests will be allowed or disallowed by the UE. Figure 5.2.1.1.1.1 illustrates an example of the NAS signalling transport applicable to a 5GC-MT-LR procedure for immediate or deferred location or to an SL-MT-LR procedure for immediate or deferred location.

Figure 5.2.1.1.1.1: NAS signalling transport for Supplementary Services Location Notification

NOTE:	The optional Additional information IE of the DL NAS TRANSPORT message or UL NAS TRANSPORT message is not included when the LCS Location Notification signalling is transported in the Payload container.

==============Next change==============

##### 5.2.1.1.2	Normal operation

The network invokes a location notification procedure by sending a REGISTER message containing an lcs-LocationNotification invoke component to the UE as defined in 3GPP TS 24.080 [5]. This may be sent either to request verification for a 5GC-MT-LR or to notify the UE about an already authorized 5GC-MT-LR.

In case of privacy verification for LCS or ranging and sidelink positioning, the UE shall respond to the request by sending a RELEASE COMPLETE message containing the user's response and optionally UE Location Privacy Indication information in a lcs-LocationNotification return result component (see figure 5.2.1.1.2.1). If rangingSlExt is included, the privacy verification is for ranging and sidelink positioning and the corresponding verification result for ranging and sidelink positioning is included in UE Location Privacy Indication information.

During the supplementary services Location Notification operation, the network shall run the timer T(LCSN) defined in 3GPP TS 24.080 [5]. This timer is started when the operation is sent, and stopped when a response is received from the UE. If the timer T(LCSN) expires in the network before any response from the UE (e.g. due to no response from the user), the network shall interpret this by applying the default treatment defined in 3GPP TS 23.273 [2] (i.e. disallow location if barred by subscription or allow location if allowed by subscription). The network may also terminate the dialogue by sending a RELEASE COMPLETE message.

In the case of location notification, the UE shall terminate the dialogue immediately by sending a RELEASE COMPLETE message containing a lcs-LocationNotification return result.

If the UE is unable to process the request received from the network, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

The use of Facility or ExtendedFacility IE in figure 5.2.1.1.2.1 is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke =lcs-LocationNotification (notificationType, locationType, lcsClientExternalID, lcsClientName, lcsRequestorID, lcsCodeword, lcsServiceTypeId, deferredLocationExt, rangingSlExt))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result = lcs-LocationNotification (verificationResponse, locationPrivacyIndication, validTimePeriod))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.1.2.1: Location Notification

==============Next change==============

##### 5.2.1.3.2	Normal operation

The LMF sends a REGISTER message to the UE containing lcs-PeriodicTriggeredInvoke invoke component as defined in 3GPP TS 24.080 [5]. The REGISTER message is transported to the UE via the serving AMF as described in figure 5.2.1.3.1.1.

If the UE can support the periodic or triggered location, the UE returns a RELEASE COMPLETE message to the LMF containing an lcs-PeriodicTriggeredInvoke return result component. The RELEASE COMPLETE message is transported to the LMF via the serving AMF as described in figure 5.2.1.3.1.1.

For supporting of the location events reporting over user plane connection, as described in clause 6.16.1 of 3GPP TS 23.273 [2], the LMF provides the UE with the following:

a)	the endpoint address for the location reporting over user plane connection;

b)	the security information for the location reporting over user plane connection;

c)	the cumulative event report timer, if available; or

d)	the maximum number of events reporting counter, if available.

If the UE receives the cumulative event report timer or the maximum number of events reporting counter from the LMF, the UE shall perform the UE initiated Event Reporting procedure as defined in subclause 5.2.2.4 with including the indication for the cumulative event report.

If the UE is unable to process or support the request received from the network, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services periodic or triggered location operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

Figure 5.2.1.3.2.1 illustrates the signalling for normal operation between the UE and the network. The use of Facility or ExtendedFacility IE is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke = lcs-PeriodicTriggeredInvoke (referenceNumber, qoS, mapped-Qos, reportingPLMNList, periodicLocation, areaEventReporting, eventReportAllowedArea, motionEventReporting, referenceNumberExt, h-gmlc-callBackUri, supportedGADShapes, deferredRoutingIdentifier, reportingAccessTypes, multiplePositioningProtocolPDUs, controlPlane-CIoT-5GS-Optimisation, scheduledLocTime, userPlaneReportAFAddr, cumulativeReportCriteria, h-gmlc-address, reportingInd))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result =lcs-PeriodicTriggeredInvoke)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.3.2.1: Periodic or Triggered Location Invocation

NOTE 1:	The mappedLcs-QoS is obtained by the UE in 5GS and used for location service continuity from 5GS to EPS for periodic or triggered location event reporting.

NOTE 2:	If h-gmlc-address as defined in 3GPP TS 24.080 [5] is received in the lcs-PeriodicTriggeredInvoke invoke component, it is used for a EPC-(H)GLMC during the procedure of location service continuity between EPS and 5GS.

NOTE 3:		If reportingIndication as defined in 3GPP TS 24.080 [5] is received in the lcs-PeriodicTriggeredInvoke invoke component, the reportingIndication is used for determining whether the Event Reporting procedure is triggered when the UE is inside or outside the eventReportAllowedArea.

==============Next change==============

##### 5.2.1.4.1	General

The supplementary services Cancel Deferred Location operation enables the AMF to cancel ongoing periodic or triggered location in a target UE using NAS signalling as described in 3GPP TS 23.273 [2] clause 6.3.3. The lcs-CancelDeferredLocation invoke component, lcs-CancelDeferredLocation return result component, related return error component and related reject component are transported using the DL NAS TRANSPORT message and the UL NAS TRANSPORT message defined in 3GPP TS 24.501 [3]. Figure 5.2.1.4.1-1 illustrates an example of the NAS signalling transport.

Figure 5.2.1.4.1.1: NAS signalling transport for Cancel Deferred Location

NOTE:	The optional Additional information IE of the DL NAS TRANSPORT message or UL NAS TRANSPORT message is not included when the lcs-CancelDeferredLocation invoke component, lcs-CancelDeferredLocation return result component, related return error component or related reject component is transported in the Payload container.

==============Next change==============

##### 5.2.1.4.2	Normal operation

The AMF invokes a cancel deferred location procedure by sending a REGISTER message containing an lcs-CancelDeferredLocation invoke component to the UE as defined in 3GPP TS 24.080 [5].

The UE shall terminate the ongoing periodic or triggered location if this can be identified from the information in the lcs-CancelDeferredLocation invoke component.

The UE shall then return a RELEASE COMPLETE message containing an lcs-CancelDeferredLocation return result component (see figure 5.2.1.4.2.1).

If the UE is unable to process the request received from the network or cannot identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the supplementary services cancel deferred location operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

The use of Facility or ExtendedFacility IE in figure 5.2.1.4.2.1 is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke = lcs-CancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result = lcs-CancelDeferredLocation)

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.4.2.1: Cancel Deferred Location

==============Next change==============

##### 5.2.1.5.2	Normal operation

The LMF invokes a PRU disassociation procedure by sending lcs-PruDisassociation invoke component to the associated UE operating as PRU as defined in 3GPP TS 24.080 [5].

The UE operating as PRU shall terminate the PRU association with the serving PLMN if this can be identified from the information in the lcs-PruDisassociation invoke component. The UE shall then return a RELEASE COMPLETE message containing a lcs-PruDisassociation return result component (see figure 5.2.1.5.2.1). If the UE operating as PRU receives a new routing ID for a new serving LMF in the lcs-PruDisassociation invoke component from the LMF, the UE operating as PRU may perform a PRU association procedure with the new serving LMF as described in clause 5.2.2.7.

The use of Facility or ExtendedFacility IE in figure 5.2.1.5.2.1 is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke = lcs-PruDisassociation(newLmfRoutingId, ackIndication))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result = lcs-PruDisassociation)

Figure 5.2.1.5.2.1: Network initiated PRU disassociation procedure

NOTE 1:	The following IE defined in lcs-PruDisassociation operation in 3GPP TS 24.080 [5] is used for NG-RAN LCS:

-	newLmfRoutingId

==============Next change==============

##### 5.2.1.7.2	Normal operation

In order to initate the network initiated RSPP supplementary information transport procedure, the LMF shall send a REGISTER message to the UE containing the lcs-DLRSPPTransport invoke component as defined in 3GPP TS 24.080 [5].

In the lcs-DLRSPPTransport invoke component included in the REGISTER message, the LMF shall include rangingSLPPList IE. In the rangingSLPPList, the LMF:

a)	may include the embedded SLPP message as defined in 3GPP TS 38.355 [4a] for the receiving UE;

b)	shall include the embedded SLPP message(s) as defined in 3GPP TS 38.355 [4a] for the related SL reference UE(s) or the related located UE(s) with the associated application layer ID(s);

NOTE:	The SLPP message(s) sent by the LMF include the SLPP message(s) to request capability, to provide assistance data or to request location information as defined in 3GPP TS 38.355 [4a].

c)	may include the indication of network assisted sidelink positioning if LMF determines to apply UE based sidelink positioning;

d)	may include the scheduled location time; and

e)	may include the relatedUEInfo IE indicating information of candidate located UE(s) when the required location result is absolute location.

The receiving UE shall then return a RELEASE COMPLETE message containing a lcs-DLRSPPTransport return result component (see Figure 5.2.1.7.2-1) if the receiving UE accepts the DLRSPPTransport invoke component.

If the receiving UE does not accept the DLRSPPTransport invoke component, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the network initiated RSPP supplementary information transport operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

Figure 5.2.1.7.2.1 illustrates the signalling for normal operation between the UE and the network. The use of Facility or ExtendedFacility IE is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke = lcs-DLRSPPTransport (rangingSLPPList, scheduledLocTime, ueBased, relatedUEInfo))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result = lcs-DLRSPPTransport ())

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.7.2.1: Network initiated RSPP supplementary information transport procedure

NOTE 1:	Only the following IEs defined in DLRSPPTransport operation in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	rangingSLPPList

-	scheduledLocTime

-	ueBased

-	relatedUEInfo

NOTE 2:	The rangingSLPPList IE is added to DLRSPPTransport to allow for passing multiple SLPP messages for the UE connecting with the LMF and/or the other related UE. Its ASN.1 description is given in 3GPP TS 24.080 [5].

==============Next change==============

##### 5.2.1.8.2	Normal operation

In order to initate the sidelink mobile terminating location request procedure, the LMF shall send a REGISTER message to the UE containing the lcs-SLMTLR invoke component as defined in 3GPP TS 24.080 [5] as described in clause 6.20.2, clause 6.20.3, clause 6.20.4 or clause 6.20.5 of 3GPP TS 23.273 [2]. In the lcs-SLMTLR invoke component, the LMF:

-	shall include the relatedUEInfo IE indicating information of candidate located UE(s) when the required location result is absolute location or indicating information of candidate SL reference UE(s) when the required location result is relative location;

-	shall include the locatedUEselect IE indicating whether the target UE or the LMF is to select the located UE(s) when the required location result is absolute location; and

-	may include the coordinateID IE indicating local coordinate(s) when the required location result is absolute location. The coordinateID IE is used to determine the target UE location in local coordinates.

When the lcs-SLMTLR invoke component is applied for periodic-triggered location events as described in clause 6.20.4 of 3GPP TS 23.273 [2], the LMF additionally:

-	shall include the requestedRangingResult IE indicating information of the requested location results (e.g. absolute locations, relative locations, distances, directions, velocities and relative velocities) for the target UE;

-	shall include the h-gmlc-address IE indicating the address for the (H)GMLC;

-	shall include the referenceNumber IE indicating the LDR reference number;

-	shall include periodicLocation IE, areaEventReporting IE, or motionEventReporting IE indicating the reporting criteria;

-	shall include the deferredRoutingIdentifier IE indicating identification of a new LMF, if any,which is to serve the UE; and

-	may include the lcs-QoS IE indicating the QoS for the requested location results.

If the UE accepts the lcs-SLMTLR invoke component, the UE shall send a RELEASE COMPLETE message to the LMF containing the lcs-SLMTLR return result component. In the lcs-SLMTLR return result component, the UE:

-	shall include the relatedUEInfo IE containing information of the related UEs for the ranging and sidelink positioning discovered by the UE, if the lcs-SLMTLR invoke component includes the locatedUEselect IE indicating that the LMF is to select the located UE(s);

-	shall include the relatedUEInfo IE containing information of the related UEs for the ranging and sidelink positioning selected by the UE, if the lcs-SLMTLR invoke component includes the locatedUEselect IE indicating that target UE is to select the located UE(s); and

-	may include the rangingSLPPList IE containing the embedded SLPP message providing sidelink positioning capabilities of the related UEs as defined in 3GPP TS 38.355 [4a].

If the UE is unable to process or support the request received from the network, the UE shall send a RELEASE COMPLETE message containing a return error component or reject component. Error values are specified in 3GPP TS 24.080 [5].

During the sidelink mobile terminating location request operation, the network shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the UE. If this timer expires the network shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message.

Figure 5.2.1.8.2.1 illustrates the signalling for normal operation between the UE and the network. The use of Facility or ExtendedFacility IE is specified in subclause 5.3.1.

UE	Network

REGISTER

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Invoke = lcs-SLMTLR (slmtlr-Type, supportedGADShapes, relatedUEInfo, locatedUEselect, coordinateID, lcs-QoS, requestedRangingResult, referenceNumber, h-gmlc-address, periodicLocation, areaEventReporting, motionEventReporting, deferredRoutingIdentifier))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Return result = lcs-SLMTLR(relatedUEInfo, rangingSLPPList))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Figure 5.2.1.8.2.1: Sidelink mobile terminating location request procedure

NOTE 1:	Only the following IEs defined in lcs-SLMTLR operation in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	slmtlr-Type

-	supportedGADShapes

-	relatedUEInfo

-	locatedUEselect

-	coordinateID

-	rangingSLPPList

-	lcs-QoS

-	requestedRangingResult

-	referenceNumber

-	h-gmlc-address

-	periodicLocation

-	areaEventReporting

-	motionEventReporting

-	deferredRoutingIdentifier

NOTE 2:	The rangingSLPPList IE is added to the lcs-SLMTLR return result component to allow for passing multiple UE positioning information SLPP messages (e.g. UE capabilities) to the LMF. Its ASN.1 description is given in 3GPP TS 24.080 [5].

==============Next change==============

##### 5.2.2.1.1	General

The supplementary services MO-LR operation enables the UE to launch MO positioning session or request location assistance data using NAS signaling. The NAS signaling are transported using the DL NAS TRANSPORT message and the UL NAS TRANSPORT message defined in 3GPP TS 24.501 [3]. Figure 5.2.2.1.1-1 illustrates an example of the NAS signaling transport for an MO-LR session.

Figure 5.2.2.1.1-1: NAS signaling transport for MO-LR

NOTE:	The optional Additional information IE of the UL NAS TRANSPORT message or DL NAS TRANSPORT message is not included when the lcs-MOLR invoke component, lcs-MOLR return result component, related return error component or related reject component is transported in the Payload container.

==============Next change==============

##### 5.2.2.1.2	Normal operation

The UE invokes an MO-LR by sending a REGISTER message to the network containing an lcs-MOLR invoke component. SS Version Indicator value 1 or above shall be used.

The receiving network entity shall initiate the handling of location request in the network. The network shall pass the result of the location procedure to the UE by sending a FACILITY message to the UE containing an lcs-MOLR return result component. When location estimate is kept in the network entity and this information satisfies the requested accuracy and the requested maximum age of location, then the network may reuse this information and the positioning measurement procedure may be skipped.

The network shall pass the result of the location procedure to the UE only if the location estimate is given in a format that the UE supports, as indicated by either the presence (and content) or the absence of the parameter supportedGADShapes, which may be sent by the UE in the lcs-MOLR invoke component.

The UE may terminate the dialogue by sending a RELEASE COMPLETE message in the case of single location request (see figure 5.2.2.1.1-1). The UE may also initiate another location request operation by sending a FACILITY message to the network containing an lcs-MOLR invoke component (see figure 5.2.2.1.1-2). After the last location request operation the UE shall terminate the dialogue by sending a RELEASE COMPLETE message.

If the network is unable to successfully fulfil the request received from the UE (e.g. to provide a location estimate or location assistance information), it shall clear the transaction by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5]. If the network is unable to provide a location estimate due to lack of support in the UE for the type of shape of the location estimate, then it shall use the error Facility Not Supported.

If the network has returned a result to the UE in a FACILITY message but, after some PLMN administered time period has elapsed, has not received either a new location request operation in a FACILITY message or a RELEASE COMPLETE message from the UE, the network may clear the transaction by sending a RELEASE COMPLETE message.

During the MO-LR operation the UE shall run a timer T(LCSL). This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and shall inform the user of the failure.

The use of Facility or ExtendedFacility IE in figure 5.2.2.1.1-1 and figure 5.2.2.1.1-2 are specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.1.2-1: Single mobile originated location request

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo, scheduledLocTime))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

FACILITY

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-MOLR (molr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, gpsAssistanceData, supportedGADShapes, lcsServiceTypeID, ageOfLocationInfo, locationType, pseudonymIndicator, h-gmlc-address, locationEstimate, velocityEstimate, referenceNumber, periodicLDRInfo, locationUpdateRequest, sequenceNumber, terminationCause, mo-lrShortCircuit, ganssAssistanceData, multiplePositioningProtocolPDUs, locationInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

(Return result = lcs-MOLR (locationEstimate, velocityEstimate, add-LocationEstimate, decipheringKeys, referenceNumber, h-gmlc-address, mo-lrShortCircuit, reportingPLMNList, timestampOfLocationEstimate))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.1.2-2: Multiple mobile originated location requests

NOTE 1:	Only the following IEs defined in lcs-MOLR operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	molr-Type

-	lcs-QoS

-	lcsServiceTypeID

-	ageOfLocationInfo

-	locationType

-	mlc-Number

-	lcsClientExternalID

-	pseudonymIndicator

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	locationEstimate

-	add-LocationEstimate

-	h-gmlc-address

-	decipheringKeys

-	scheduledLocTime

-	velocityEstimate

NOTE 2:	The multiplePositioningProtocolPDUs IE is added to the lcs-MOLR invoke component to allow for passing multiple UE positioning information LPP messages (e.g. UE location measurements or UE capabilities) to the LMF for NG-RAN LCS. Its ASN.1 description is given in 3GPP TS 24.080 [5], where the maximum number of LPP messages is specified. There is one or more types of multiple LPP messages can be encapsulated in one multiplePositioningProtocolPDUs IE.

==============Next change==============

##### 5.2.2.2.2	Normal operation

The UE invokes a cancel deferred location procedure by sending a REGISTER message containing an lcs-MSCancelDeferredLocation invoke component to the LMF as defined in 3GPP TS 24.080 [5].

The LMF shall terminate the ongoing periodic or triggered location if this can be identified from the information in the lcs-MSCancelDeferredLocation invoke component.

The LMF shall then return a RELEASE COMPLETE message containing an lcs-MSCancelDeferredLocation return result component (see Figure 5.2.2.2.2-1).

NOTE:	If the UE has requested the cancellation of the deferred location procedure for the location events reporting over user plane connection and the UE receives the acknowledgment of the cancellation from the LMF, the UE releases the user plane connection to the LCS Client or AF if the release has not been yet initiated.

If the LMF is unable to process the request received from the UE or can not identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated cancel deferred location operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

The use of Facility or ExtendedFacility IE in figure 5.2.2.2.2-1 is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-MSCancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-MSCancelDeferredLocation)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.2.2-1: Cancel Deferred Location

NOTE:	Only the following IEs defined in lcs-MSCancelDeferredLocation operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	referenceNumberExt

-	h-gmlc-callBackUri

==============Next change==============

##### 5.2.2.4.2	Normal operation

The UE invokes an EventReport procedure by sending a REGISTER message containing an lcs-EventReport invoke component to the LMF as defined in 3GPP TS 24.080 [5].

If the LMF can handle this event report, the LMF shall return a RELEASE COMPLETE message containing an lcs-EventReport return result component (see Figure 5.2.2.4.2-1).

If the LMF is unable to process the request received from the UE or cannot identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated event reporting procedure operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed. The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

The use of Facility or ExtendedFacility IE in figure 5.2.2.4.2-1 is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-EventReport (eventType, referenceNumberExt, h-gmlc-callBackUri, lcs-QoS, locationInfo, supportedGADShapes, multiplePositioningProtocolPDUs, terminationCause, cumulativeEventReport))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-EventReport(deferredRoutingIdentifier, terminationCause))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.4.2-1: EventReport

NOTE:	Only the following identifiers defined in lcs-EventReport operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	eventType

-	referenceNumberExt

-	h-gmlc-callBackUri

-	lcs-QoS

-	locationInfo

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	terminationCause

-	cumulativeEventReport

==============Next change==============

##### 5.2.2.5.2	Normal operation

The UE invokes a Location Privacy Setting procedure by sending a REGISTER message containing an lcs-LocationPrivacySetting invoke component as defined in 3GPP TS 24.080 [5]. UE's Location Privacy Indication information is included in lcs-LocationPrivacySetting invoke component.

The AMF shall transfer the UE Location Privacy Indication information in lcs-LocationPrivacySetting invoke component to UDM.

The AMF shall return a RELEASE COMPLETE message containing a LocationPrivacySetting Acknowledgement component (see figure 5.2.2.5.2-1) if the acknowledgement message is received from UDM.

If the AMF is unable to process the lcs-LocationPrivacySetting invoke component received from the UE or cannot identify that provisioning the parameters of UE Location Privacy Setting information to UDM is executed successfully, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated location privacy setting procedure operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed. The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

The use of Facility or ExtendedFacility IE in figure 5.2.2.5.2-1 is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-LocationPrivacySetting (locationPrivacyIndication, validTimePeriod, eventReportExpectedArea, areaUsageIndication)

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-LocationPrivacySetting)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.5.2-1: LocationPrivacySetting

NOTE:	Only the following identifiers defined in lcs-LocationPrivacySetting operation in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	locationPrivacyIndication

-	validTimePeriod

-	eventReportExpectedArea

-	areaUsageIndication

==============Next change==============

##### 5.2.2.7.2	Normal operation

The UE operating as PRU invokes a PRU association procedure by sending a REGISTER message containing a lcs-PruAssociation invoke component to the serving LMF as defined in 3GPP TS 24.080 [5]. The lcs-PruAssociation invoke component shall include a type for the PRU association request whether it is initial PRU association request or PRU association update, the positioning capabilities of the UE operating as PRU, PRU ON/OFF state, and optionally the location information if available. The UE operating as PRU may include the pre-configured routing ID for an initial association, or the routing ID received from the previous PRU association procedure in the UL NAS TRANSPORT message.

The LMF shall return a RELEASE COMPLETE message containing a lcs-PruAssociation return result component if the LMF accepts the PRU association. The lcs-PruAssociation return result component shall indicate the conditions for performing PRU association updates with the serving LMF. The conditions for performing PRU association update may include a periodic PRU association update timer and PRU association update based on a change of PRU location, change of PRU TAI, change of serving AMF.

If the LMF is unable to process the lcs-PruAssociation invoke component received from the UE operating as PRU as specified in clause 6.17.1 of 3GPP TS 23.273 [2], the LMF shall return a RELEASE COMPLETE message containing reject component. The reject component may include routing ID of the serving LMF if new LMF is selected to serve the UE operating as PRU.

The use of Facility or ExtendedFacility IE in figure 5.2.2.7.2.1 is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (invoke = lcs-PruAssociation (associationType, positioningCapabilities, stateOfPRU, locationOfPRU)

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (return result = lcs-PruAssociation (periodicUpdateTimer, updateTrigger)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (newLMFroutingId))

Figure 5.2.2.7.2.1: PRU association procedure

==============Next change==============

##### 5.2.2.8.2	Normal operation

The UE operating as PRU invokes a PRU disassociation procedure by sending lcs-PruDisassociation invoke component to the serving LMF as defined in 3GPP TS 24.080 [5]. The lcs-PruDisassociation invoke component shall include an indication whether an acknowledgement from the LMF is expected or not. The UE operating as PRU shall include the routing ID received from the previous PRU association procedure in the UL NAS TRANSPORT message.

The LMF shall terminate the PRU association with the associated UE operating as PRU if this can be identified from the information in the lcs-PruDisassociation invoke component. The LMF shall then return a RELEASE COMPLETE message containing a lcs-PruDisassociation return result component (see figure 5.2.2.8.2-1). If the UE operating as PRU has not indicated that an acknowledgement is expected, the LMF may disassociate the UE operating as PRU locally without return the RELEASE COMPLETE message.

The use of Facility or ExtendedFacility IE in figure 5.2.2.8.2-1 is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (invoke = lcs-PruDisassociation (AckIndication, newLmfRoutingId))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (return result =lcs-PruDisassociation)

Figure 5.2.2.8.2-1: UE initiated PRU disassociation procedure

==============Next change==============

##### 5.2.2.9.1	General

The supplementary services SL-MO-LR operation enables the target UE to launch the ranging and sidelink MO positioning session or request location assistance data using NAS signaling. The NAS signaling is transported using the DL NAS TRANSPORT message and the UL NAS TRANSPORT message defined in 3GPP TS 24.501 [3]. Figure 5.2.2.9.1-1 illustrates an example of the NAS signaling transport for an SL-MO-LR.

The SL-MO-LR operation can be triggered by a target UE as specified in clause 6.20.1 of 3GPP TS 23.273 [2].

Figure 5.2.2.9.1-1: NAS signalling transport for successful SL-MO-LR

NOTE:	The Additional information IE of the UL NAS TRANSPORT message or DL NAS TRANSPORT message is not included when the lcs-SLMOLR invoke component, lcs-SLMOLR return result component, related return error component or related reject component is transported in the Payload container.

==============Next change==============

##### 5.2.2.9.2	Normal operation

The UE invokes an SL-MO-LR by sending a REGISTER message to the network containing an lcs-SLMOLR invoke component. SS Version Indicator value 1 or above shall be used.

The receiving network entity shall initiate the handling of location request in the network. The network shall pass the result of the location procedure to the UE by sending a FACILITY message to the UE containing an lcs-SLMOLR return result component. When location estimate is kept in the network entity and this information satisfies the requested accuracy and the requested maximum age of location, then the network may reuse this information and the positioning measurement procedure may be skipped.

The network shall pass the result of the location procedure to the UE only if the location estimate is given in a format that the UE supports, as indicated by either the presence (and content) or the absence of the parameter supportedGADShapes, which may be sent by the UE in the lcs-SLMOLR invoke component operation.

The UE may terminate the dialogue by sending a RELEASE COMPLETE message in the case of single location request (see figure 5.2.2.9.2-1). The UE may also initiate another location request operation by sending a FACILITY message to the network containing an lcs-SLMOLR invoke component (see figure 5.2.2.9.2-2). After the last location request operation the UE shall terminate the dialogue by sending a RELEASE COMPLETE message.

When the network is congested and if the network decides not to perform the location procedure, the network may allow UE only ranging and sidelink positioning for a given time duration by sending a RELEASE COMPLETE message to the UE containing an lcs-SLMOLR return result component including a ueOnlyRSLPosAllowed IE. The network shall provide the time duration in the ueOnlyRSLPosAllowed IE.

Upon receiving the lcs-SLMOLR return result component including a ueOnlyRSLPosAllowed IE, the UE may perform the UE only sidelink positioning procedure as specified in clause 6.6 of 3GPP TS 23.586 [10] within the time duration provided in the ueOnlyRSLPosAllowed IE.

If the network is unable to successfully fulfil the request received from the UE (e.g. to provide a location estimate or location assistance information), it shall clear the transaction by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5]. If the network is unable to provide a location estimate due to lack of support in the UE for the type of shape of the location estimate, then it shall use the error Facility Not Supported.

If the network has returned a result to the UE in a FACILITY message but, after some PLMN administered time period has elapsed, has not received either a new location request operation in a FACILITY message or a RELEASE COMPLETE message from the UE, the network may clear the transaction by sending a RELEASE COMPLETE message.

During the lcs-SLMOLR operation, the UE shall run a timer T5101. This timer is started when the lcs-SLMOLR invoke component is sent, and stopped when a lcs-SLMOLR return result component, return error component or reject component is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and shall inform the user of the failure.

In the lcs-SLMOLR invoke component included in the REGISTER or FACILITY message, the UE:

a)	shall include the slmolr-Type IE set to rangingSidelink;

b)	shall include the relatedUEInfo IE containing one entry for each of the one or more SL reference UEs or located UEs, containing application layer ID of the SL reference UE and the slReferenceUE role or application layer ID of the located UE and the locatedUE role;

c)	shall include the calculationAssistIndicator IE, if the UE needs the location calculation assistance;

d)	shall include the preferredRangingResult IE set to absoluteLocationIndicator, absoluteVelocityIndicator, relativeLocationIndicator, rangeDirection, relativeVelocityIndicator or any combination of them;

e)	shall include the lcs-QoS IE; and

g)	may include the lcsClientExternalID IE indicating identity of the LCS client or the AF. If the lcsClientExternalID is included, the UE may include the mlc-Number IE indicating the GMLC via which the LCS client or the AF is to be be accessed, may include the lcsServiceTypeID IE and may include pseudonymIndicator to indicate that the pseudonym is needed.

When the network receives the lcs-SLMOLR invoke component included in the REGISTER or FACILITY message, the network shall perform the "procedures of SL-MO-LR involving LMF" as specified in 3GPP TS 23.273 [2].

If the network accepts the lcs-SLMOLR invoke component, the network shall return a FACILITY message to the UE containing lcs-SLMOLR return result component (see figure 5.2.2.9.2-1 and figure 5.2.2.9.2-2). In the lcs-SLMOLR return result component, the network:

a)	may include the absoluteLocation IE, if the absoluteLocationIndicator is requested in the preferredRangingResult IE;

b)	may include the absoluteVelocity IE, if the absoluteVelocityIndicator is requested in the preferredRangingResult IE;

c)	may include the relativeResult IE, if the relativeLocationIndicator, rangeDirection, relativeVelocityIndicator or any combination of them are requested in the preferredRangingResult IE; and

d)	may include the timestamp IE.

The use of Facility or ExtendedFacility IE in figure 5.2.2.9.2-1 and figure 5.2.2.9.2-2 are specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-SLMOLR (slmolr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, supportedGADShapes, lcsServiceTypeID, pseudonymIndicator, h-gmlc-address, calculationAssistIndicator, preferredRangingResult, relatedUEInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-SLMOLR (absoluteLocation, absoluteVelocity, relativeResult, timestamp))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return result = lcs-SLMOLR (ueOnlyRSLPosAllowed))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.9.2-1: Single sidelink mobile originated location request

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-SLMOLR (slmolr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, supportedGADShapes, lcsServiceTypeID, pseudonymIndicator, h-gmlc-address, calculationAssistIndicator, preferredRangingResult, relatedUEInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-SLMOLR (absoluteLocation, absoluteVelocity, relativeResult, timestamp))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

FACILITY

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-SLMOLR (slmolr-Type, lcs-QoS, lcsClientExternalID, mlc-Number, supportedGADShapes, lcsServiceTypeID, pseudonymIndicator, h-gmlc-address, calculationAssistIndicator, preferredRangingResult, relatedUEInfo))

FACILITY

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-SLMOLR (absoluteLocation, absoluteVelocity, relativeResult, timestamp))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return result (ueOnlyRSLPosAllowed))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

------------------------------------------------------------------------------------------------------------------------>

Figure 5.2.2.9.2-2: Multiple sidelink mobile originated location requests

NOTE:	Only the following IEs defined in lcs-SLMOLR operation in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	slmolr-Type

-	lcs-QoS

-	lcsClientExternalID

-	mlc-Number

-	supportedGADShapes

-	lcsServiceTypeID

-	pseudonymIndicator

-	h-gmlc-address

-	calculationAssistIndicator

-	preferredRangingResult

-	relatedUEInfo

-	absoluteLocation

-	absoluteVelocity

-	relativeResult

-	ueOnlyRSLPosAllowed

-	timestamp

==============Next change==============

##### 5.2.2.11.2	Normal operation

In order to initiate the UE initiated RSPP supplementary information transport procedure, the initiating UE shall send a REGISTER message to the LMF containing the lcs-ULRSPPTransport invoke component as defined in 3GPP TS 24.080 [5].

In the lcs-ULRSPPTransport invoke component included in the REGISTER message, the UE shall include rangingSLPPList IE. In the rangingSLPPList, the UE:

a)	may include the embedded SLPP message as defined in 3GPP TS 38.355 [4a] for the initiating UE; and

b)	shall include the embedded SLPP message(s) as defined in 3GPP TS 38.355 [4a] for the related SL reference UE(s) or the related located UE(s) with the associated application layer ID(s).

NOTE:	The SLPP message(s) sent by the initiating UE include the SLPP message(s) to provide capability, to request assistance data or to provide location information as defined in 3GPP TS 38.355 [4a].

The LMF shall then return a RELEASE COMPLETE message containing an lcs-ULRSPPTransport return result component (see Figure 5.2.2.11.2-1) if the LMF accepts the lcs-ULRSPPTransport invoke component.

If the LMF does not accept the lcs-ULRSPPTransport invoke component, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated RSPP supplementary information transport operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed. The UE may terminate the dialogue by sending a RELEASE COMPLETE message, and informs the user of the failure.

Figure 5.2.2.11.2-1 illustrates the signalling for normal operation between the UE and the network. The use of Facility or ExtendedFacility IE is specified in subclause 5.3.1.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility or ExtendedFacility (Invoke = lcs-ULRSPPTransport (rangingSLPPList))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility or ExtendedFacility (Return result = lcs-ULRSPPTransport())

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility or ExtendedFacility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.11.2-1: UE initiated RSPP supplementary information transport

NOTE 1: Only the following IEs defined in lcs-ULRSPPTransport operations in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	rangingSLPPList

NOTE 2:	The rangingSLPPList IE is added to lcs-ULRSPPTransport invoke component to allow for passing multiple SLPP messages for the UE connecting with the LMF and/or the other related UE. Its ASN.1 description is given in 3GPP TS 24.080 [5].

==============Next change==============

### 5.3.1	Messages for Location services operations

The LCS message format and information elements coding for the lcs-MOLR, lcs-LocationNotification, lcs-EventReport, lcs-PeriodicTriggeredInvoke, lcs-CancelDeferredLocation, lcs-MSCancelDeferredLocation, lcs-LocationPrivacySetting, lcs-SLMOLR, lcs-SLMTLR, lcs-PruAssociation, lcs-PruDisassociation, lcs-DLRSPPTransport, and lcs-ULRSPPTransport operations (clause 5.2) are defined in 3GPP TS 24.080 [5] for the following messages:

-	REGISTER message;

-	FACILITY message; and

-	RELEASE COMPLETE message.

For a component of the location services operation in clause 5.2, when the length of the component is less than or equal to 255 octets, the component shall be carried in the Facility IE of the REGISTER message, FACILITY message or RELEASE COMPLETE message, as defined in 3GPP TS 24.080 [5].

For a component of the location services operation used in clause 5.2, when the length of the component cannot be fitted into the Facility IE, i.e. exceeds 255 octets, and the UE and the network support ExtendedFacility IE for 5G LCS as specified in clause 4.2, the component shall be carried in the ExtendedFacility IE of the REGISTER message, FACILITY message or RELEASE COMPLETE message. The Facility IE shall be empty and ignored when ExtendedFacility IE is present, as defined in 3GPP TS 24.080 [5].

==============End of changes==============