3GPP TSG-CT WG1 Meeting #157	C1-256520

Sophia Antipolis, France, 13-17 October 2025	was C1-256226

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7023 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Determine which PDU session to use for non-3GPP device traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in S2-2507791 (23.501 CR#6345) and captured in latest version of TS 23.501: “NOTE 1: To support QoS differentiation of traffic for Non-3GPP Device Identifier, how the UE determines which PDU Session to use for the non-3GPP device's traffic is not specified; the existing mechanisms (e.g., URSP) can be used to determine which PDU Session to use. It is up to UE implementation to determine when to initiate PDU Session Modification procedure for updating the Non-3GPP Device Connection Information.” In existing TS 24.501, it is clarified that: “NOTE 2: To support QoS differentiation of traffic for non-3GPP device(s) connecting through the UE, how the UE determines which PDU session to use for the non-3GPP device traffic is up to UE implementation.” However, it is not indicated that which mechanisms (e.g. URSP) can be used to determine which PDU session to use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align with stage 2 requirment to clarify that existing mechanisms (e.g. URSP rules if any or UE local configuration) can be used to determine which PDU session to use for the non-3GPP device traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2 requirement. Unclear which mechanisms can be used to determine which PDU session to use for the non-3GPP device traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.30	QoS differentiation of traffic for non-3GPP device identifier

A 5GS can support QoS differentiation of traffic for non-3GPP devices connecting through the UE, as specified in 3GPP TS 23.501 [8]. The UE and the network may support QoS differentiation of traffic for non-3GPP devices.

The UE shall bind a non-3GPP device identifier to a non-3GPP device, for the traffic of the non-3GPP device that requires differentiated QoS (see 3GPP TS 23.501 [8] and 3GPP TS 23.316 [6D]). A non-3GPP device identifier shall be bound to no more than one non-3GPP device simultaneously. This binding enables the 5GS to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	QoS differentiation of traffic for the non-3GPP device identifier also applies to the wireline access when a non-3GPP device is connecting to the 5G-RG.

NOTE 2:	To support QoS differentiation of traffic for non-3GPP device(s) connecting through the UE, how the UE determines which PDU session to use for the non-3GPP device traffic is up to UE implementation. Existing mechanisms (e.g. URSP rules if any or UE local configuration) can be used to determine which PDU session to use for the non-3GPP device traffic.

The UE enables QoS differentiation of traffic from one or more non-3GPP devices associated with non-3GPP device identifiers by providing each non-3GPP device identifier and the corresponding connection information to the SMF in the PDU session modification procedure. The UE shall provide the non-3GPP device connection information for each non-3GPP device depending on the PDU session type of the PDU session, as follows:

a)	for Ethernet PDU session type the UE shall include the non-3GPP device identifier and the Ethernet connection information;

b)	for IPv4 PDU session type the UE shall include the non-3GPP device identifier and the IPv4 connection information;

c)	for IPv6 PDU session type the UE shall include the non-3GPP device identifier and the IPv6 connection information; and

d)	for IPv4v6 PDU session type the UE shall include the non-3GPP device identifier and:

1)	the IPv4 connection information;

2)	the IPv6 connection information; or

3)	both 1) and 2).

NOTE 3:	For an IPv4v6 PDU session type, it is up to UE implementation to decide whether to use IPv4 connection information, IPv6 connection information, or a combination of both. This decision can be based on the traffic associated with the non-3GPP device and the IP address allocation by the network for that PDU session.

* * * End of Change * * * *