3GPP TSG-CT WG1 Meeting #157	C1-256563

# Sophia-Antipolis, France 13-17 October 2025	(was C1-256477)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4589 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MINT Ph2 Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several corrections for MINT_Ph2 are proposes: 1] There is a typo on clause 3.1 UE determined PLMN wth disaster condition 2] In clause 4.12 the acronym needs to be spelled out when used the first time. 3] A space was inadvertently inserted in figure 9.9.3.34 for MINT- EPS |  |  |  |  |  |  |  |  |
| Summary of change: |  | The editorials are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for MINT in EPS feature will be incomplete in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.12, 9.9.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The term "mobile station" (MS) in the present document is synonymous with the term "user equipment" (UE) as defined in 3GPP TR 21.905 [1].

1x CS fallback capable UE: A UE that uses a CS infrastructure for a voice call and other CS-domain services by falling back to cdma2000® 1x access network if the UE is served by E-UTRAN when a CS service is requested.

Aggregate maximum bit rate: The maximum bit rate that limits the aggregate bit rate of a set of non-GBR bearers of a UE. Definition derived from 3GPP TS 23.401 [10].

APN based congestion control: Congestion control in session management where the network can reject session management requests from UEs or deactivate PDN connections when the associated APN is congested.

Attached for emergency bearer services: A UE is attached for emergency bearer services if it has only a PDN connection for emergency bearer services established.

Attached for disaster roaming services: A UE is considered as "attached for disaster roaming services in EPS" when it has successfully completed attach or tracking area update for disaster roaming services.

Attached for access to RLOS: A UE is attached for access to RLOS if the UE requested access to RLOS during the attach procedure and has a PDN connection for RLOS established after completion of attach procedure.

Attached for EPS services with control plane CIoT EPS optimization: A UE supporting CIoT EPS optimizations is attached for EPS services, and control plane CIoT EPS optimization along with one or more other CIoT EPS optimizations have been accepted by the network.

Attached for EPS services with user plane CIoT EPS optimization: A UE supporting CIoT EPS optimizations is attached for EPS services, and user plane CIoT EPS optimization along with one or more other CIoT EPS optimizations have been accepted by the network.

Attached for EPS services with CIoT EPS optimization: A UE is attached for EPS services with control plane CIoT EPS optimization or attached for EPS services with user plane CIoT EPS optimization.

Chosen PLMN: The same as selected PLMN as specified in 3GPP TS 23.122 [6].

Control plane CIoT EPS optimization: signalling optimizations to enable efficient transport of user data (IP, non-IP, Ethernet or SMS) over control plane via the MME including optional header compression of IP data.

Control plane CIoT EPS optimization with overhead reduction: Control plane CIoT EPS optimization using EMM messages with reduced protocol overhead.

CS fallback cancellation request: A request received from the MM sublayer to cancel a mobile originating CS fallback.

CS fallback capable UE: A UE that uses a CS infrastructure for a voice call and other CS-domain services by falling back to A/Gb or Iu mode if the UE is served by E-UTRAN when a CS service is requested.

CSG cell: A cell in which only members of the CSG can get normal service. Depending on local regulation, the CSG cell can provide emergency bearer services also to subscribers who are not member of the CSG. Definition derived from 3GPP TS 23.401 [10].

CSG ID: A CSG ID is a unique identifier within the scope of one PLMN defined in 3GPP TS 23.003 [2] which identifies a Closed Subscriber Group (CSG) in the PLMN associated with a cell or group of cells to which access is restricted to members of the CSG.

CSG selection: A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs. Definition derived from 3GPP TS 23.122 [6].

Current TAI: A TAI of a selected PLMN broadcast in the cell on which the UE is camping. If the cell is a satellite E-UTRA cell broadcasting multiple TACs of the selected PLMN, the UE NAS layer selects the TAI from these multiple TACs of the selected PLMN as specified in clause 4.11.3.

NOTE 1:	For the purpose of this definition, the selected PLMN can either be the registered PLMN or a PLMN selected according to PLMN selection rules as specified in 3GPP TS 23.122 [6].

Dedicated bearer: An EPS bearer that is associated with uplink packet filters in the UE and downlink packet filters in the PDN GW where the filters only match certain packets. Definition derived from 3GPP TS 23.401 [10].

Default bearer: An EPS bearer that gets established with every new PDN connection. Its context remains established throughout the lifetime of that PDN connection. A default EPS bearer is a non-GBR bearer. Definition derived from 3GPP TS 23.401 [10].

Emergency EPS bearer context: A default EPS bearer context activated with request type "emergency" or "handover of emergency bearer services", or any dedicated EPS bearer context associated to this default EPS bearer context.

EMM context: An EMM context is established in the UE and the MME when an attach procedure is successfully completed.

EMM-CONNECTED mode: A UE is in EMM-CONNECTED mode when a NAS signalling connection between UE and network is established or after indication from the lower layers that the RRC connection has been resumed when the UE was in EMM-IDLE mode with suspend indication. The term EMM-CONNECTED mode used in the present document corresponds to the term ECM-CONNECTED state used in 3GPP TS 23.401 [10].

EMM-IDLE mode: A UE is in EMM-IDLE mode when no NAS signalling connection between UE and network exists or when RRC connection suspend has been indicated by lower layers. The term EMM-IDLE mode used in the present document corresponds to the term ECM-IDLE state used in 3GPP TS 23.401 [10].

EPS security context: In the present specification, EPS security context is used as a synonym for EPS NAS security context specified in 3GPP TS 33.401 [19].

EPS services: Services provided by PS domain. Within the context of this specification, EPS services is used as a synonym for GPRS services in 3GPP TS 24.008 [13].

Evolved packet core network: The successor to the 3GPP Release 7 packet-switched core network, developed by 3GPP within the framework of the 3GPP System Architecture Evolution (SAE).

Evolved packet system: The evolved packet system (EPS) or evolved 3GPP packet-switched domain consists of the evolved packet core network and the evolved universal terrestrial radio access network. Definition derived from 3GPP TS 23.401 [10].

E-UTRAN cell: A cell with E-UTRAN access technology or satellite E-UTRAN access technology.

GBR bearer: An EPS bearer that uses dedicated network resources related to a guaranteed bit rate (GBR) value, which are permanently allocated at EPS bearer establishment/modification. Definition derived from 3GPP TS 23.401 [10].

General NAS level mobility management congestion control: The type of congestion control that is applied at a general overload or congestion situation in the network, e.g., lack of processing resources.

Group specific session management congestion control: Type of congestion control at session management level that is applied to reject session management requests from UEs belonging to a particular group when one or more group congestion criteria as specified in 3GPP TS 23.401 [10] are met.

Highest ranked ACDC category: The ACDC category with the lowest value as defined in 3GPP TS 24.105 [35].

Initial NAS message: A NAS message is considered as an initial NAS message, if this NAS message can trigger the establishment of a NAS signalling connection. For instance, the ATTACH REQUEST message is an initial NAS message.

Initial attach for disaster roaming services: An attach procedure performed with EPS attach type "disaster roaming attach in EPS" in the ATTACH REQUEST message.

In NB-S1 mode: Indicates this paragraph applies only to a system which operates in NB-S1 mode. For a multi-access system this case applies if the current serving radio access network or the current serving satellite access network provides access to network services via E-UTRA by NB-IoT (see 3GPP TS 36.300 [20], 3GPP TS 36.331 [22], 3GPP TS 36.306 [44]).

In WB-S1 mode: Indicates this paragraph applies only to a system which operates in WB-S1 mode. For a multi-access system this case applies if the system operates in S1 mode, but not in NB-S1 mode. WB-S1 mode also includes satellite access.

In WB-S1/CE mode: Indicates this paragraph applies only when a UE, which is a CE mode B capable UE (see 3GPP TS 36.306 [44]), is operating in CE mode A or B in WB-S1 mode.

IPv4v6 capability: Capability of the IP stack associated with a UE to support a dual stack configuration with both an IPv4 address and an IPv6 address allocated.

Kilobit: 1000 bits.

Last Visited Registered TAI: A TAI which is contained in the TAI list that the UE registered to the network and which identifies the tracking area last visited by the UE. If the cell is a satellite cell broadcasting multiple TAIs, a TAI which is contained in the TAI list that the UE registered to the network and last selected by the UE as the current TAI.

Linked Bearer Identity: This identity indicates to which default bearer the additional bearer resource is linked.

LIPA PDN connection: A PDN connection, for which the default EPS bearer context or default PDP context was activated with an APN authorized to use LIPA. The network authorizes an APN for using LIPA based on the subscription profile (see 3GPP TS 29.272 [16C]) and subsequently the network considers this PDN connection a LIPA PDN connection.

Lower layer failure: A failure reported by the AS to the NAS that cannot be corrected on AS level. When the AS indicates a lower layer failure to NAS, the NAS signalling connection is not available.

Mapped EPS security context: A mapped security context to be used in EPS. Definition derived from 3GPP TS 33.401 [19].

Mapped GUTI: A GUTI which is mapped from a P-TMSI and an RAI allocated previously by an SGSN or a 5G-GUTI previously allocated by an AMF. Mapping rules are defined in 3GPP TS 23.003 [2]. Definition derived from 3GPP TS 23.401 [10].

Megabit: 1,000,000 bits.

Message header: A standard L3 message header as defined in 3GPP TS 24.007 [12].

MME area: An area containing tracking areas served by an MME.

MO MMTEL voice call is started: the MO-MMTEL-voice-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MO-MMTEL-voice-started indication, the MO-MMTEL-voice-ended indication has not been received.

MO MMTEL video call is started: the MO-MMTEL-video-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MO-MMTEL-video-started indication, the MO-MMTEL-video-ended indication has not been received.

MT MMTEL voice call is started: the MT-MMTEL-voice-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MT-MMTEL-voice-started indication, the MT-MMTEL-voice-ended indication has not been received.

MT MMTEL video call is started: the MT-MMTEL-video-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MT-MMTEL-video-started indication, the MT-MMTEL-video-ended indication has not been received.

MO SMSoIP is started: the MO-SMSoIP-attempt-started indication was received from upper layers (see 3GPP TS 24.341 [15D]) and after reception of the MO-SMSoIP-attempt-started indication, the MO-SMSoIP-attempt-ended indication has not been received.

MT SMSoIP is started: the MT-SMSoIP-attempt-started indication was received from upper layers (see 3GPP TS 24.341 [15D]) and after reception of the MT-SMSoIP-attempt-started indication, the MT-SMSoIP-attempt-ended indication has not been received.

MUSIM UE: A UE with multiple valid USIMs, capable of initiating and maintaining simultaneous separate registration states with PLMN(s) using identities and credentials associated with those USIMs and supporting one or more of the NAS signalling connection release, the paging indication for voice services, the reject paging request, the paging restriction and the paging timing collision control.

NAS level mobility management congestion control: Congestion control mechanism in the network in mobility management. "NAS level mobility management congestion control" consists of "subscribed APN based congestion control" and "general NAS level mobility management congestion control".

NAS signalling connection: A peer to peer S1 mode connection between UE and MME. A NAS signalling connection consists of the concatenation of an RRC connection via the "LTE-Uu" interface and an S1AP connection via the S1 interface. Additionally, for the purpose of optimized handover or idle mode mobility from cdma2000® HRPD access to E-UTRAN (see 3GPP TS 23.402 [11]), the NAS signalling connection can consist of a concatenation of an S101-AP connection and a signalling tunnel over a cdma2000® HRPD access network.

NOTE 2:	cdma2000® is a registered trademark of the Telecommunications Industry Association (TIA-USA).

NAS signalling connection recovery: A mechanism initiated by the NAS to restore the NAS signalling connection on indication of "RRC connection failure" by the lower layers.

Native GUTI: A GUTI previously allocated by an MME. Definition derived from 3GPP TS 23.401 [10].

Non-access stratum protocols: The protocols between UE and MSC or SGSN that are not terminated in the UTRAN, and the protocols between UE and MME that are not terminated in the E-UTRAN. Definition derived from 3GPP TR 21.905 [1].

Non-emergency EPS bearer context: Any EPS bearer context which is not an emergency EPS bearer context.

Non-EPS services: Services provided by CS domain. Within the context of this specification, non-EPS services is used as a synonym for non-GPRS services in 3GPP TS 24.008 [13]. A UE which camps on E-UTRAN can attach to both EPS services and non-EPS services.

Non-GBR bearer: An EPS bearer that uses network resources that are not related to a guaranteed bit rate (GBR) value. Definition derived from 3GPP TS 23.401 [10].

Non-satellite E-UTRA cell: A cell with E-UTRAN access technology.

PDN address: An IP address assigned to the UE by the Packet Data Network Gateway (PDN GW).

PDN connection for emergency bearer services: A PDN connection with an emergency EPS bearer context or with a default PDP context activated with request type "emergency" or "handover of emergency bearer services".

PDN connection for RLOS: A PDN connection for which the default EPS bearer context was activated with request type "RLOS".

Plain NAS message: A NAS message with a header including neither a message authentication code nor a sequence number.

Persistent EPS bearer context: either a non-emergency EPS bearer context representing a GBR bearer with QoS equivalent to QoS of teleservice 11 and where there is a radio bearer associated with that context, or an emergency EPS bearer context where there is a radio bearer associated with that context.

NOTE 3:	An example of a persistent EPS bearer context is a non-emergency EPS bearer context with QCI = 1 where there is a radio bearer associated with that context.

Procedure Transaction Identity: An identity which is dynamically allocated by the UE for the UE requested ESM procedures. The procedure transaction identity is released when the procedure is completed.

RAT-related TMSI: When the UE is camping on an E-UTRAN cell, the RAT-related TMSI is the GUTI; when it is camping on a GERAN or UTRAN cell, the RAT-related TMSI is the P-TMSI.

Registered PLMN: The PLMN on which the UE is registered. The identity of the registered PLMN is provided to the UE within the GUTI.

Relay node: A network element in the E-UTRAN, wirelessly connected to an eNode B and providing relaying function to UEs served by the E-UTRAN. Definition derived from 3GPP TS 23.401 [10].

Removal of eCall only mode restriction: All the limitations as described in 3GPP TS 22.101 [46] for the eCall only mode do not apply any more.

RLOS EPS bearer context: A default RLOS EPS bearer context which was activated with request type "RLOS", or any dedicated EPS bearer context associated to this default EPS bearer context.

The label (S1 mode only) indicates that this clause or paragraph applies only to a system which operates in S1 mode, i.e. with a functional division that is in accordance with the use of an S1 interface between the radio access network and the core network. The S1 mode includes WB-S1 mode and NB-S1 mode. In a multi-access system this case is determined by the current serving radio access network or the current serving satellite access network.

S&F Monitoring List: A list of satellite ID(s) for satellite(s) that belong to the same PLMN. This list indicates the satellite(s) that the UE may use to (re)attempt NAS procedures or may receive MT data from, when the satellites are operating in S&F mode.

Satellite E-UTRAN cell: A cell with satellite E-UTRAN access technology.

Satellite E-UTRAN RAT type: In case of satellite E-UTRAN access, RAT types are used to distinguish different types of satellite E-UTRAN access, as defined in 3GPP TS 38.413 [31]. In this version of the specification, the defined satellite E-UTRAN RAT types are "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "LTE-M(LEO)", "LTE-M(MEO)" and "LTE-M(GEO)".

SCEF PDN Connection: A PDN connection established between the UE and the Service Capability Exposure Function (SCEF) for transmitting the UE's non-IP data related to a specific application.

SGi PDN Connection: A PDN connection established between the UE and the Packet Gateway (P-GW) for transmitting the UE's IP, non-IP or Ethernet data related to a specific application.

S101 mode: Applies to a system that operates with a functional division that is in accordance with the use of an S101 interface. For the definition of the S101 reference point, see 3GPP TS 23.402 [11].

SIPTO at the local network PDN connection: A PDN connection, for which the default EPS bearer context or default PDP context was activated with an APN authorized to use SIPTO at the local network and it was activated such that the traffic of the PDN connection will be using an L-GW. The network authorizes an APN for using SIPTO at the local network based on the subscription profile (see 3GPP TS 29.272 [16C]) and subsequently the network considers this PDN connection a SIPTO at the local network PDN connection. SIPTO at the local network PDN connection can be of IP, non-IP or Ethernet PDN type.

SIPTO at the local network PDN connection with a collocated L-GW: A SIPTO at the local network PDN connection which is established to a L-GW function collocated with the (H)(e)NodeB. The core-network entity (i.e. the MME or the SGSN) can be aware of whether the SIPTO at the local network PDN connection with a collocated L-GW is used when the PDN connection is established.

SIPTO at the local network PDN connection with a stand-alone GW: A SIPTO at the local network PDN connection which is established to a stand-alone GW (with collocated L-GW and S-GW). The core-network entity (i.e. the MME or the SGSN) can be aware of whether the SIPTO at the local network PDN connection with a stand-alone GW is used when the PDN connection is established.

"SMS only": A subset of services which includes only Short Message Service. A UE camping on E-UTRAN can attach to both EPS services and "SMS only".

SMS over NAS: refers to SMS in MME or SMS over SGs.

SMS over S102: refers to SMS which uses 1xCS procedures in EPS as defined in 3GPP TS 23.272 [9].

Subscribed APN based congestion control: Congestion control in mobility management where the network can reject attach requests from UEs with a certain APN in the subscription.

S&F satellite E-UTRAN cell: A cell with satellite E-UTRAN access technology where the cell broadcasts an indication that the eNode-B is operating in S&F mode.

TAI list: A list of TAIs that identify the tracking areas that the UE can enter without performing a tracking area updating procedure. The TAIs in a TAI list assigned by an MME to a UE pertain to the same MME area.

Tracking area update for disaster roaming services: A tracking area update procedure performed with EPS attach type "disaster roaming tracking area updating in EPS" in the ATTACH REQUEST message.

Traffic flow aggregate: A temporary aggregate of packet filters that are included in a UE requested bearer resource allocation procedure or a UE requested bearer resource modification procedure and that is inserted into a traffic flow template (TFT) for an EPS bearer context by the network once the UE requested bearer resource allocation procedure or UE requested bearer resource modification procedure is completed.

UE configured for dual priority: A UE which provides dual priority support is configured for NAS signalling low priority and also configured to override the NAS signalling low priority indicator (see 3GPP TS 24.368 [15A], 3GPP TS 31.102 [17]).

UE configured to use AC11 – 15 in selected PLMN: A UE configured with at least one access class in the range 11-15 on the USIM, and the access class is applicable in the selected PLMN according to 3GPP TS 22.011 [1A].

UE's availability for voice calls in the IMS: The indication of this availability or non-availability is provided by the upper layers of the UE as specified in 3GPP TS 24.229 [13D] in the annex relevant to the IP-Connectivity Access Network in use or determined in the NAS layer, as specified in clause 4.3.1. If availability is indicated, the UE uses the IM CN Subsystem and can terminate or originate requests for SIP sessions including an audio component with codecs suited for voice.

UE's usage setting: This is a UE setting that indicates whether the UE has preference for voice services over data services or vice-versa. If a UE has preference for voice services, then the UE's usage setting is "voice centric". If a UE has preference for data services, then the UE's usage setting is "data centric". A UE whose setting is "data centric" may still require access to voice services. A UE whose setting is "voice centric" may still require access to data services. This definition is derived from 3GPP TS 23.221 [8A] and it applies to voice capable UEs. If the UE is capable of both S1 mode and N1 mode, there is a single UE's usage setting which applies to both 5GS and EPS (see 3GPP TS 24.501 [54]).

UE supporting CIoT EPS optimizations: A UE that supports control plane CIoT EPS optimization or user plane CIoT EPS optimization and one or more other CIoT EPS optimizations when the UE is in S1 mode.

UE using EPS services with control plane CIoT EPS optimization: A UE that is attached for EPS services with the control plane CIoT EPS optimization accepted by the network.

UE using EPS services with control plane CIoT EPS optimization with overhead reduction: A UE that is attached for EPS services with the control plane CIoT EPS optimization with overhead reduction accepted by the network.

User plane CIoT EPS optimization: Signalling optimizations to enable efficient transport of user data (IP, non-IP or Ethernet) over the user plane.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.221 [8A] apply:

Restricted local operator services

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.401 [10] apply:

APN rate control status

Cellular IoT (CIoT)

DCN-ID

Dedicated core network

eCall only mode

NarrowBand-IoT

PDN connection

S&F mode

Service Gap Control

Store and forward satellite operation

UE paging probability information

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.272 [9] apply:

CS fallback

SMS in MME

SMS over SGs

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.682 [11A] apply:

SCEF

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.008 [13] apply:

A/Gb mode

Access domain selection

Default PDP context

Extended idle-mode DRX cycle

Iu mode

Native P-TMSI

Power saving mode

PS signalling connection

RR connection

TFT

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.102 [18] apply:

UMTS security context

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.401 [19] apply:

Current EPS security context

Data via MME

Full native EPS security context

KASME

K'ASME

Mapped security context

Native EPS security context

Non-current EPS security context

Partial native EPS security context

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.122 [6] apply:

Access Technology

Country

Discontinuous Coverage

EHPLMN

HPLMN

Limited Service State

MINT-EPS

Shared Network

Suitable Cell

UE determined PLMN with disaster condition

VPLMN

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.216 [8] apply:

SRVCC

vSRVCC

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.011 [1A] apply:

Application specific Congestion control for Data Communication (ACDC)

Extended Access Barring

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.003 [10] apply:

Local Home Network Identifier

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.303 [31] apply:

ProSe direct communication

ProSe direct discovery

ProSe UE-to-Network Relay

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.161 [36] apply:

Multi-access PDN connection

NBIFOM

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.167 [45] apply:

eCall over IMS

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.501 [54] apply:

5G-EA

5G-IA

5GMM-CONNECTED mode

5GMM-DEREGISTERED

5GMM-DEREGISTERED-INITIATED

5GMM-IDLE mode

5GMM-NULL

5GMM-REGISTERED

5GMM-REGISTERED-INITIATED

5GMM-SERVICE-REQUEST-INITIATED

Applicable UE radio capability ID for the current UE radio configuration in the selected network

Control plane CIoT 5GS optimization

N1 mode

NB-N1 mode

Native 5G-GUTI

NG-RAN cell

Non-satellite NG-RAN cell

Service-level-AA

Satellite NG-RAN cell

UE operating in single-registration mode in a network supporting N26 interface

UE supporting UAS services

User plane CIoT 5GS optimization

For the purposes of the present document, the following terms and definitions given in 3GPP TS 36.413 [23] apply:

User Location Information

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.256 [60] apply:

3GPP UAV ID

CAA (Civil Aviation Administration)-Level UAV Identity

Command and Control (C2) Communication

Direct C2 communication

No-transmit zone (NTZ)

UAV controller (UAV-C)

UAS Services

UAS Service Supplier (USS)

Uncrewed Aerial System (UAS)

USS communication

UUAA

UUAA-SM

* * * Next Change * * * *

## 4.12	MINT-EPS for UE to obtain service in EPS under disaster condition

The UE and the network may support Minimization of service interruption (MINT-EPS).

If the UE supports MINT-EPS, the indication of whether disaster roaming in EPS is enabled in the UE, the one or more "list of PLMN(s) to be used in disaster condition", if available, are stored in the non-volatile memory in the ME as specified in annex C and are kept when the UE enters EMM-DEREGISTERED state. Annex C specifies the condition under which the indication of whether disaster roaming in EPS is enabled in the UE, the one or more "lists of PLMN(s) to be used in disaster condition", stored in the ME are deleted.

If the UE supports MINT-EPS, the UE shall indicate this capability to the network during the attach procedure or tracking area updating procedure. Upon selecting a PLMN for disaster roaming in EPS as specified in 3GPP TS 23.122 [6], the UE shall perform an attach procedure with the EPS attach type set to "disaster roaming attach", or a tracking area updating procedure with the EPS update type set to "disaster roaming update" for an inter-system change from 5GS to EPS, for disaster roaming services. When the UE is attached for disaster roaming services, the MME shall only include TAIs covering the area with the disaster condition.

Upon selecting a PLMN for disaster roaming as specified in 3GPP TS 23.122 [6]:

a)	if the UE does not have a stored disaster roaming wait range the UE shall perform an attach procedure for disaster roaming services on the selected PLMN as described in subclause 5.5.1; and

b)	if the UE has a stored disaster roaming wait range the UE shall generate a random number within the disaster roaming wait range and start a timer with the generated random number. While the timer is running, the UE shall not initiate attach on the selected PLMN except if the UE needs to request a PDN connection for emergency bearer services, in which case the UE shall initiate the attach procedure, set the EPS attach type IE to "EPS emergency attach" in the ATTACH REQUEST message and keep the timer running. Upon expiration of the timer, if the UE does not have a PDN connection for emergency bearer services, the UE shall perform an attach procedure for disaster roaming services as described in subclause 5.5.1 if still camped on the selected PLMN. If the UE has a PDN connection for emergency bearer services when the timer expires, the attach procedure for disaster roaming services as described in subclause 5.5.1 shall be performed after the release of the PDN connection for emergency bearer services, if the UE is still camped on the selected PLMN.

If the UE is switched off when the timer for disaster roaming wait range is running, the UE shall behave as follows when the UE is switched on, the USIM in the UE remains the same and the UE selects the PLMN for disaster roaming:

-	let t1 be the time remaining for the timer for disaster roaming wait range timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

Upon expiry of the timer for disaster roaming wait range, the UE shall not start a timer with the disaster roaming wait range value again until disaster condition has ended.

Upon determining that a disaster condition has ended as specified in 3GPP TS 23.122 [5]:

a)	the UE shall stop the timer started with a generated random number within the disaster roaming wait range, if running;

b)	the UE shall perform PLMN selection as specified in 3GPP TS 23.122 [5], except if the UE already selected an allowable PLMN as specified in 3GPP TS 23.122 [5]; and

c)	if the UE selects the UE determined PLMN with disaster condition and has a stored disaster return wait range, which is provided by the PLMN providing disaster roaming services or by the selected PLMN; and the UE shall generate a random number within the disaster return wait range and start a timer with the generated random number value. While the timer is running, the UE shall not initiate registration on the selected PLMN except if the UE needs to request an emergency PDU session, in which case the UE shall initiate the registration procedure, set the 5GS registration type IE to "emergency registration" in the REGISTRATION REQUEST message and keep the timer running. Upon expiration of the timer, if the UE does not have an emergency PDU session, the UE shall perform the registration procedure if still camped on the selected PLMN. If the UE has an emergency PDU session when the timer expires, the registration procedure shall be performed after the release of the emergency PDU session, if the UE is still camped on the selected PLMN.

Otherwise, the UE shall perform the registration procedure in the selected PLMN.

If the UE is switched off when the timer for disaster return wait range is running, the UE shall behave as follows when the UE is switched on, the USIM in the UE remains the same and the UE selects the UE determined PLMN with disaster condition:

-	let t1 be the time remaining for the timer for disaster return wait range timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

Upon expiry of the timer for disaster return wait range, the UE shall not start a timer with the disaster return wait range value again until the UE selects a PLMN for disaster roaming services.

When the MME assigns a tracking area to the UE attached for disaster roaming services, the MME shall only include TAIs covering the area with the disaster condition.

If the UE is attached for disaster roaming services and the registered PLMN is removed from forbidden PLMN lists due to reasons specified in subclause 4.4.6 or Annex C in 3GPP TS 23.122 [6], then UE shall initiate the detach procedure and perform PLMN selection as specified in 3GPP TS 23.122 [6].

* * * Next Change * * * *

#### 9.9.3.34	UE network capability

The purpose of the UE network capability information element is to provide the network with information concerning aspects of the UE related to EPS or interworking with GPRS and 5GS. The contents might affect the manner in which the network handles the operation of the UE. The UE network capability information indicates general UE characteristics, and it shall therefore, except for fields explicitly indicated, be independent of the frequency band of the channel it is sent on.

The UE network capability information element is coded as shown in figure 9.9.3.34.1 and table 9.9.3.34.1.

The UE network capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 15 octets.

NOTE:	The requirements for the support of UMTS security algorithms in the UE are specified in 3GPP TS 33.102 [18], and the requirements for the support of EPS security algorithms in 3GPP TS 33.401 [19].

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE network capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of UE network capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| EEA0 | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 3 |
| EIA0 | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EPS-UPIP |  | octet 4 |
| UEA0 | UEA1 |  | UEA2 |  | UEA3 |  | UEA4 |  | UEA5 |  | UEA6 |  | UEA7 |  | octet 5* |
| UCS2 | UIA1 |  | UIA2 |  | UIA3 |  | UIA4 |  | UIA5 |  | UIA6 |  | UIA7 |  | octet 6* |
| ProSe-dd | ProSe |  | H.245-ASH |  | ACC-CSFB |  | LPP |  | LCS |  | 1xSR VCC |  | NF |  | octet 7* |
| ePCO | HC-CP CIoT |  | ERw/oPDN |  | S1-U data |  | UP CIoT |  | CP CIoT |  | Prose-relay |  | ProSe-dc |  | octet 8* |
| 15 bearers | SGC |  | N1mode |  | DCNR |  | CP backoff |  | RestrictEC |  | V2X PC5 |  | multipleDRB |  | octet 9* |
| RPR | PIV |  | NCR |  | V2X NR-PC5 |  | UP-MT-EDT |  | CP-MT-EDT |  | WUSA |  | RACS |  | octet 10* |
| MINT-EPS | OHR-CP CIoT |  | SFSO |  | ATUC |  | RCLIN |  | EDC |  | PTCC |  | PR |  | octet 11* |
| 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 12* -15* |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.9.3.34.1: UE network capability information element

Table 9.9.3.34.1: UE network capability information element

| EPS encryption algorithms supported (octet 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPS encryption algorithm EEA0 supported (octet 3, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA0 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA0 supported |  |  |
| EPS encryption algorithm 128-EEA1 supported (octet 3, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA1 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA1 supported |  |  |
| EPS encryption algorithm 128-EEA2 supported (octet 3, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA2 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA2 supported |  |  |
| EPS encryption algorithm 128-EEA3 supported (octet 3, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA3 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm 128-EEA3 supported |  |  |
| EPS encryption algorithm EEA4 supported (octet 3, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA4 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA4 supported |  |  |
| EPS encryption algorithm EEA5 supported (octet 3, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA5 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA5 supported |  |  |
| EPS encryption algorithm EEA6 supported (octet 3, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA6 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA6 supported |  |  |
| EPS encryption algorithm EEA7 supported (octet 3, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA7 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS encryption algorithm EEA7 supported |  |  |
| EPS integrity algorithms supported (octet 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPS integrity algorithm EIA0 supported (octet 4, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA0 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA0 supported |  |  |
| EPS integrity algorithm 128-EIA1 supported (octet 4, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA1 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA1 supported |  |  |
| EPS integrity algorithm 128-EIA2 supported (octet 4, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA2 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA2 supported |  |  |
| EPS integrity algorithm 128-EIA3 supported (octet 4, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA3 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm 128-EIA3 supported |  |  |
| EPS integrity algorithm EIA4 supported (octet 4, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA4 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA4 supported |  |  |
| EPS integrity algorithm EIA5 supported (octet 4, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA5 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA5 supported |  |  |
| EPS integrity algorithm EIA6 supported (octet 4, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA6 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS integrity algorithm EIA6 supported |  |  |
| EPS-UPIP supported (octet 4, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EPS-UPIP not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EPS-UPIP supported |  |  |
| UMTS encryption algorithms supported (octet 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| UMTS encryption algorithm UEA0 supported (octet 5, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA0 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA0 supported |  |  |
| UMTS encryption algorithm UEA1 supported (octet 5, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA1 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA1 supported |  |  |
| UMTS encryption algorithm UEA2 supported (octet 5, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA2 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA2 supported |  |  |
| UMTS encryption algorithm UEA3 supported (octet 5, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA3 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA3 supported |  |  |
| UMTS encryption algorithm UEA4 supported (octet 5, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA4 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA4 supported |  |  |
| UMTS encryption algorithm UEA5 supported (octet 5, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA5 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA5 supported |  |  |
| UMTS encryption algorithm UEA6 supported (octet 5, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA6 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA6 supported |  |  |
| UMTS encryption algorithm UEA7 supported (octet 5, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA7 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS encryption algorithm UEA7 supported |  |  |
| UCS2 support (UCS2) (octet 6, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |
| This information field indicates the likely treatment of UCS2 encoded character strings by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | The UE has a preference for the default alphabet (defined in |  |  |
|  |  |  |  |  |  |  |  |  |  | 3GPP TS 23.038 [3]) over UCS2 (see ISO/IEC 10646 [29]). |  |  |
| 1 |  |  |  |  |  |  |  |  |  | The UE has no preference between the use of the default alphabet and |  |  |
|  |  |  |  |  |  |  |  |  |  | the use of UCS2. |  |  |
| UMTS integrity algorithms supported (octet 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| UMTS integrity algorithm UIA1 supported (octet 6, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA1 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA1 supported |  |  |
| UMTS integrity algorithm UIA2 supported (octet 6, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA2 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA2 supported |  |  |
| UMTS integrity algorithm UIA3 supported (octet 6, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA3 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA3 supported |  |  |
| UMTS integrity algorithm UIA4 supported (octet 6, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA4 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA4 supported |  |  |
| UMTS integrity algorithm UIA5 supported (octet 6, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA5 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA5 supported |  |  |
| UMTS integrity algorithm UIA6 supported (octet 6, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA6 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA6 supported |  |  |
| UMTS integrity algorithm UIA7 supported (octet 6, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA7 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | UMTS integrity algorithm UIA7 supported |  |  |
| NF capability (octet 7, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | notification procedure not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | notification procedure supported |  |  |
| 1xSRVCC capability (octet 7, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS supported |  |  |
|  |  |  |  |  |  |  |  |  |  | (see 3GPP TS 23.216 [8]) |  |  |
| Location services (LCS) notification mechanisms capability (octet 7, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | LCS notification mechanisms not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | LCS notification mechanisms supported (see 3GPP TS 24.171 [13C]) |  |  |
| LTE Positioning Protocol (LPP) capability (octet 7, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | LPP not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | LPP supported (see 3GPP TS 36.355 [22A]) |  |  |
| Access class control for CSFB (ACC-CSFB) capability (octet 7, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | eNodeB-based access class control for CSFB not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | eNodeB-based access class control for CSFB supported (see 3GPP TS 22.011 [1A]) |  |  |
| H.245 After SRVCC Handover capability (H.245-ASH) (octet 7, bit 6) This bit indicates the capability for H.245 with support and use of pre-defined codecs, and if needed, H.245 codec negotiation after SRVCC handover. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | H.245 after SRVCC handover capability not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | H.245 after SRVCC handover capability supported (see 3GPP TS 23.216 [8]) |  |  |
| ProSe (octet 7, bit 7) This bit indicates the capability for ProSe. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | ProSe not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | ProSe supported |  |  |
| ProSe direct discovery (ProSe-dd) (octet 7, bit 8) This bit indicates the capability for ProSe direct discovery. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | ProSe direct discovery not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | ProSe direct discovery supported |  |  |
| ProSe direct communication (ProSe-dc) (octet 8, bit 1) This bit indicates the capability for ProSe direct communication. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | ProSe direct communication not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | ProSe direct communication supported |  |  |
| ProSe UE-to-network-relay (ProSe-relay) (octet 8, bit 2) This bit indicates the capability to act as a ProSe UE-to-network relay |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Acting as a ProSe UE-to-network relay not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Acting as a ProSe UE-to-network relay supported |  |  |
| Control plane CIoT EPS optimization (CP CIoT) (octet 8, bit 3) This bit indicates the capability for control plane CIoT EPS optimization. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Control plane CIoT EPS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Control plane CIoT EPS optimization supported |  |  |
| User plane CIoT EPS optimization (UP CIoT) (octet 8, bit 4) This bit indicates the capability for user plane CIoT EPS optimization. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | User plane CIoT EPS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | User plane CIoT EPS optimization supported |  |  |
| S1-U data transfer (S1-U data) (octet 8, bit 5) This bit indicates the capability for S1-U data transfer. This bit shall be considered only if the Control plane CIoT EPS optimization (CP CIoT) bit (octet 8, bit 3) is set to 1. If the Control plane CIoT EPS optimization (CP CIoT) bit (octet 8, bit 3) is set to 0, the MME shall assume S1-U data transfer is supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | S1-U data transfer not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | S1-U data transfer supported |  |  |
| EMM-REGISTERED without PDN connection (ERw/oPDN) (octet 8, bit 6) This bit indicates the capability for EMM-REGISTERED without PDN connection. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | EMM-REGISTERED without PDN connection not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | EMM-REGISTERED without PDN connection supported |  |  |
| Header compression for control plane CIoT EPS optimization (HC-CP CIoT) (octet 8, bit 7) This bit indicates the capability for header compression for control plane CIoT EPS optimization. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Header compression for control plane CIoT EPS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Header compression for control plane CIoT EPS optimization supported |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) This bit indicates the support of the extended protocol configuration options IE. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Extended protocol configuration options IE not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Extended protocol configuration options IE supported |  |  |
| Multiple DRB support (multipleDRB) (octet 9, bit 1) This bit indicates the capability to support multiple user plane radio bearers (see 3GPP TS 36.306 [44], 3GPP TS 36.331 [22]) in NB-S1 mode. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Multiple DRB not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Multiple DRB supported |  |  |
| V2X communication over PC5 (V2X PC5) (octet 9, bit 2) This bit indicates the capability for V2X communication over E-UTRA-PC5. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 supported |  |  |
| Restriction on use of enhanced coverage support (RestrictEC) (octet 9, bit 3) This bit indicates the capability to support restriction on use of enhanced coverage. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage supported |  |  |
| Control plane data backoff support (CP backoff) (octet 9, bit 4) This bit indicates the support of back-off timer for transport of user data via the control plane. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | back-off timer for transport of user data via the control plane not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | back-off timer for transport of user data via the control plane supported |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) This bit indicates the capability for dual connectivity with NR. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | dual connectivity with NR not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | dual connectivity with NR supported |  |  |
| N1 mode supported (N1mode) (octet 9, bit 6) This bit indicates the capability for N1 mode for 3GPP access. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | N1 mode for 3GPP access not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | N1 mode for 3GPP access supported |
| Service gap control (SGC) (octet 9, bit 7) This bit indicates the capability for service gap control |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | service gap control not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | service gap control supported |
| Signalling for a maximum number of 15 EPS bearer contexts (15 bearers) (octet 9, bit 8) This bit indicates the support of signalling for a maximum number of 15 EPS bearer contexts |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | Signalling for a maximum number of 15 EPS bearer contexts not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | Signalling for a maximum number of 15 EPS bearer contexts supported |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) This bit indicates the capability for RACS. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  | RACS not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  | RACS supported |  |  |
| Wake-up signal (WUS) assistance (octet 10, bit 2) This bit indicates the support of wake-up signal assistance |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | WUS assistance not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | WUS assistance supported |
| Control plane Mobile Terminated-Early Data Transmission (CP-MT-EDT) (octet 10, bit 3) This bit indicates the support of control plane Mobile Terminated-Early Data Transmission |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Control plane Mobile Terminated-Early Data Transmission not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Control plane Mobile Terminated-Early Data Transmission supported |
| User plane Mobile Terminated-Early Data Transmission (UP-MT-EDT) (octet 10, bit 4) This bit indicates the support of user plane Mobile Terminated-Early Data Transmission |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | User plane Mobile Terminated-Early Data Transmission not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | User plane Mobile Terminated-Early Data Transmission supported |
| V2X communication over NR-PC5 (V2X NR-PC5) (octet 10, bit 5) This bit indicates the capability for V2X communication over NR-PC5. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 supported |
| NAS signalling connection release (NCR) (octet 10, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of NAS signalling connection release. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | NAS signalling connection release not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | NAS signalling connection release supported |
| Paging indication for voice services (PIV) (octet 10, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of paging indication for voice services. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services supported |
| Reject paging request (RPR) (octet 10, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of reject paging request. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | reject paging request not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | reject paging request supported |
| Paging restriction (PR) (octet 11, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of paging restriction. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | paging restriction not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | paging restriction supported |
| Paging timing collision control (PTCC) (octet 11, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of paging timing collision control. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | paging timing collision control not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | paging timing collision control supported |
| Enhanced Discontinuous Coverage (EDC) (octet 11, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of enhanced discontinuous coverage. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | enhanced discontinuous coverage not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | enhanced discontinuous coverage supported |
| Reporting coarse location information via NAS (RCLIN) (octet 11, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of reporting coarse location information via NAS when accessing satellite E-UTRAN in NB-S1 mode. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | reporting coarse location information via NAS not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | reporting coarse location information via NAS supported |
| Access technology utilization control (ATUC) (octet 11, bit 5) This bit indicates the support of Access technology utilization control. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Access technology utilization control not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Access technology utilization control supported |
| S&F satellite operation (SFSO) (octet 11, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of S&F satellite operation. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | S&F satellite operation not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  | S&F satellite operation supported |
| Data transport over control plane CIoT EPS optimization with overhead reduction (OHR-CP CIoT) (octet 11, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support for control plane CIoT optimization with overhead reduction. This bit shall be considered only if the Control plane CIoT EPS optimization (CP CIoT) bit (octet 8, bit 3) is set to 1. This bit shall be ignored if the Control plane CIoT EPS optimization (CP CIoT) bit (octet 8, bit 3) is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  | data transport over control plane CIoT EPS optimization with overhead reduction not supported |  |
| 1 |  |  |  |  |  |  |  |  |  |  | data transport over control plane CIoT EPS optimization with overhead reduction supported |  |
| Minimization of service interruption support in EPS (MINT-EPS) (octet 11, bit 8) This bit indicates the capability to support MINT-EPS |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  | MINT-EPS not supported |  |
| 1 |  |  |  |  |  |  |  |  |  |  | MINT-EPS supported |  |
| All other bits in octet 12 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for E-UTRAN (bits 8 to 5 of octet 3), it shall support the same algorithm for NR-PDCP as specified in 3GPP TS 33.401 [19]. The NR-PDCP is specified in 3GPP TS 38.323 [53]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity algorithms for E-UTRAN (bits 8 to 5 of octet 4), it shall support the same algorithm for NR-PDCP as specified in 3GPP TS 33.401 [19]. NOTE 3: In this release of the specification, the EPS-UPIP supported bit is only applicable for a UE supporting dual connectivity with NR. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *