3GPP TSG-WG CT1 Meeting #157	C1-256712

# Sophia-Antipolis, FR, 13th Oct – 17th Oct, 2025	 (revision of C1-256439)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0104 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current procedure of avatar communication, the procedure on how the avatar representation is downloaded using the avatar id is not clear enough. S2-2507606 clarifies how the avatar id is downloaded using the avatar id associated url. This CR is to add this procedure into the avatar communication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure on how the avatar respresentation is downloaded. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The avatar communication procedure is not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3.2.1.1; C.3.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

C.3.2.1.1	Avatar ID List Downloading

When the UE downloading the avatar communication application through the established bootstrap data channel as specified in clause 9.3.2.1.2 or clause 9.3.2.1.3.1, the UE may fetch the list of avatar ID as specified in 3GPP TS 23.228 [3] through bootstrap data channel or by pre-configuration.

NOTE 1:	The list of avatar ID can be downloaded to the UE through application data channel. This procedure is outside the scope of this specification.

NOTE 2:	The Avatar ID associated URL can be included in the Avatar ID list and downloaded to UE. The associated URL enables the UE to download the Avatar representation associated with the Avatar ID.

# * * * * Next change * * * *

C.3.2.1.2	Application Data Channel Establishment and Avatar Animation

After the user selects the avatar ID and runs the avatar communication application, the originating UE shall generate an SDP offer to establish an application data channel as per clause 9.3.2.1.3.2.

The originating UE may initiate the avatar animation negotiation with the DC AS and terminating UE via the established application data channel as per 3GPP TS 23.228 [3].

Based on the avatar animation negotiation result:

-	if the rendering mode is negotiated to originating UE centric, the originating UE may get the avatar representation identified with the selected avatar ID via the application data channel if needed, and performs avatar animation with the selected avatar representation. In addition, the UE shall send the animated video stream over RTP to the peer UE;

-	if the rendering mode is negotiated to terminating UE centric,

1)	the originating UE shall request the terminating UE to perform avatar animation, which should include the avatar ID selected by the user of originating UE and sends the avatar metadata to terminating UE through the established application data channel and audio/video through RTP channel; and

2)	the terminating UE shall download the avatar representation using the orginating UE ID, the avatar ID and associated URL received from orginating UE as specified in bullet 1) and the terminating UE ID and performs the avatar animation based on the received avatar metadata; and

-	if the rendering mode is negotiated to network centric, the originating UE:

1)	may initiate a new P2A2P or P2P application data channel, which are used for avatar rendering-related data transmission between the two UEs and the network, and

2)	 sends the avatar metadata to through the newly established application data channel and/or audio/video.

If the terminating UE initiates the avatar communication, the procedure for originating UE applies to the terminating UE and the procedure for terminating UE applies to the originating UE.

# * * * * End of changes * * * *