3GPP TSG-CT WG1 Meeting #157	C1-256795

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0844 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing MAC addresses of Initiating UE for Ethernet traffic via 5G ProSe layer-3 multi-hop UE-to-UE relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In CT1#156, C1-255525 (CR#0810) added a new section “8c.2.8” to the spec for provisioning of the MAC addresses of the source and target 5G ProSe multi-hop layer-3 end UEs, and also clarified that the 5G ProSe multi-hop layer-3 UE-to-UE relay maintains the mapping between PC5 links and Ethernet MAC addresses. However, the CR failed to also cover the fact that the Relay needs to indicate its own MAC address, so that the receiving relay can map this info with the source and/or target end UE MAC address, when it is maintaining the association. - Based on the above, the inclusion criteria for a few Information Elements in ProSe direct link establishment accept, ProSe direct link modification request/accept and ProSe direct link security mode complete need to be modified |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The MAC address of the initiating 5G ProSe multi-hop layer-3 end UE ahs been added to one bullet in section 8c.2.8, for the case where the link is being modified - The inclusion criteria for source and target 5G ProSe layer-3 end UE MAC addresses have been modified by adding the “multi-hop” possibility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8c.2.8; 10.3.2.7; 10.3.6.7; 10.3.7.6; 10.3.14.7, 10.3.6.1,10.3.6.x(new),10.3.7.1,10.3.7.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 8c.2.8	Support for Ethernet traffic via 5G ProSe multi-hop layer-3 UE-to-UE relay

When a 5G ProSe direct link is being established or modified between the source 5G ProSe multi-hop layer-3 end UE and the 5G ProSe multi-hop layer-3 UE-to-UE relay UE (called source 5G ProSe direct link),  5G ProSe direct link is being established or modified between any two 5G ProSe multi-hop layer-3 UE-to-UE relay UE (called UE-to-UE relay 5G ProSe direct link), and 5G ProSe direct link is being established or modified between the 5G ProSe multi-hop layer-3 UE-to-UE relay UE and the target 5G ProSe multi-hop layer-3 end UE (called target 5G ProSe direct link), for Ethernet traffic:

a)	if the source 5G ProSe direct link is being established, MAC address of the source 5G ProSe multi-hop layer-3 end UE is indicated in the PROSE DIRECT LINK SECURITY MODE COMPLETE message of the source 5G ProSe direct link and MAC address of the target 5G ProSe multi-hop layer-3 end UE shall be indicated in the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message of the source 5G ProSe direct link;

b)	if the source 5G ProSe direct link is being modified:

1)	MAC address of the target 5G ProSe multi-hop layer-3 end UE shall be indicated in the PROSE DIRECT LINK MODIFICATION ACCEPT message of the source 5G ProSe direct link; or

2)	MAC address of the source 5G ProSe multi-hop layer-3 end UE shall be indicated in the PROSE DIRECT LINK MODIFICATION REQUEST message of the source 5G ProSe direct link if the MAC address of the source 5G ProSe multi-hop layer-3 end UE changed;

c)	if the UE-to-UE relay 5G ProSe direct link is being established, MAC address of the source 5G ProSe multi-hop layer-3 end UE is indicated in the PROSE DIRECT LINK SECURITY MODE COMPLETE message of the source 5G ProSe direct link and MAC address of the target 5G ProSe multi-hop layer-3 end UE is indicated in the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message of the source 5G ProSe direct link;

d)	if the UE-to-UE relay 5G ProSe direct link is being modified:

1)	MAC address of the target 5G ProSe multi-hop layer-3 end UE and the MAC address of the initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE shall be indicated in the PROSE DIRECT LINK MODIFICATION ACCEPT message of the UE-to-UE relay 5G ProSe direct link if the MAC address of the target 5G ProSe multi-hop layer-3 end UE changed; and

2)	MAC address of the source 5G ProSe multi-hop layer-3 end UE and the MAC address of the initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE shall be indicated in the PROSE DIRECT LINK MODIFICATION REQUEST message of the UE-to-UE relay 5G ProSe direct link if the MAC address of the source 5G ProSe multi-hop layer-3 end UE changed;

e)	if the target 5G ProSe direct link is being established, MAC address of the source 5G ProSe multi-hop layer-3 end UE is indicated in PROSE DIRECT LINK SECURITY ESTABLISHMENT ACCEPT message of the target 5G ProSe direct link and MAC address of the target 5G ProSe multi-hop layer-3 end UE is indicated in the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message of the target 5G ProSe direct link; and

f)	if the target 5G ProSe direct link is being modified:

1)	MAC address of the source 5G ProSe multi-hop layer-3 end UE shall be indicated in the PROSE DIRECT LINK MODIFICATION REQUEST message of the target 5G ProSe direct link; or

2)	MAC address of the target 5G ProSe multi-hop layer-3 end UE may be indicated in the PROSE DIRECT LINK MODIFICATION ACCEPT message of the target 5G ProSe direct link if the MAC address of the target 5G ProSe multi-hop layer-3 end UE changed.

The 5G ProSe multi-hop layer-3 UE-to-UE relay UE maintains associations between the 5G ProSe direct links and Ethernet MAC addresses provided by the 5G ProSe multi-hop layer-3 end UEs of the 5G ProSe direct links, shall ensure that each 5G ProSe multi-hop layer-3 end UE uses a different MAC address, and shall use the association for forwarding of the user data packets.

The 5G ProSe multi-hop layer-3 end UE maintains associations between the 5G ProSe direct link and Ethernet MAC address provided by the other 5G ProSe multi-hop layer-3 end UE and shall use the Ethernet MAC address provided by the other 5G ProSe layer-3 end UE as destination address of the user data packets.

Next Change

#### 10.3.2.7	Target 5G ProSe layer-3 end UE MAC address

The UE shall include this IE when the MAC address of the target 5G ProSe layer-3 end UE, or target 5G ProSe multi-hop layer-3 end UE needs to be indicated.

NextChange

#### 10.3.6.1	Message definition

This message is sent by the UE to another peer UE to initiate the direct link modification procedure. See table 10.3.6.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION REQUEST

Significance:	dual

Direction:	UE to peer UE

Table 10.3.6.1.1: PROSE DIRECT LINK MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Link modification operation code | Link modification operation code 11.3.19 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 5-65537 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| 66 | Source end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 67 | Target end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 2D | Target end UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4 |
| 33 | Source 5G ProSe layer-3 end UE MAC address | MAC address 11.3.45 | O | TLV | 8 |
| 9- | Relay reselection indication | Relay reselection indication 11.3.46 | O | TV | 1 |
| 2F | List of candidates U2U relay UE info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 30 | Source end UE IP address | UE IP address 11.3.49 | O | TLV | 7-19 |
| 31 | List of target end UE IP addresses | List of UE IP addresses 11.3.50 | O | TLV | 7-257 |
| 32 | List of candidates U2U relay UE layer-2 ID | List of candidates U2U relay UE layer-2 IDs 11.3.48 | O | TLV | 5-257 |
| 50 | New MSBs of KNRP ID | MSBs of KNRP ID 2 11.3.52 | O | TLV | 4 |
| 70 | List of multi-hop remote UE parameters | List of multi-hop remote UE parameters 11.3.58 | O | TLV-E | 22-65538 |
| 10 | Child UEs multi-hop user info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 11 | End-to-end QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 12 | Multi-hop path info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| xx | Initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE MAC address | MAC address 11.3.45 | O | TLV | 8 |

Next Change

#### 10.3.6.7	Source 5G ProSe layer-3 end UE MAC address

The UE shall include this IE when the MAC address of the source 5G ProSe layer-3 end UE, or source 5G ProSe multi-hop layer-3 end UE needs to be indicated.

Next Change

#### 10.3.6.x	Initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE MAC address

The UE shall include this IE to indicate the MAC address of the initiating 5G ProSe layer-3 UE-to-UE relay UE when the link between any two 5G ProSe multi-hop layer-3 UE-to-UE relay is being modified.

Next Change

#### 10.3.7.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link modification request is accepted. See table 10.3.7.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACCEPT

Significance:	dual

Direction:	UE to peer UE

Table 10.3.7.1.1: PROSE DIRECT LINK MODIFICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACCEPT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
| 79 | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| 66 | Source end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 67 | Target end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 2E | Target 5G ProSe layer-3 end UE MAC address | MAC address 11.3.45 | O | TLV | 8 |
| 9- | Relay reselection indication | Relay reselection indication 11.3.46 | O | TV | 1 |
| 34 | New 5G ProSe UE-to-UE relay UE info | User info ID 2 11.3.39 | O | TLV | 8 |
| 30 | Source end UE IP address | UE IP address 11.3.49 | O | TLV | 7-19 |
| 35 | Target end UE IP address | UE IP address 11.3.49 | O | TLV | 7-19 |
| 53 | Target end UE IP address for new relay | UE IP address 11.3.49 | O | TLV | 7-19 |
| 40 | New LSBs of KNRP ID | LSBs of KNRP ID 2 11.3.53 | O | TLV | 4 |
| 10 | End-to-end QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 12 | Multi-hop path info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| yy | Initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE MAC address | MAC address 11.3.45 | O | TLV | 8 |

#### 10.3.7.x	Initiating 5G ProSe multi-hop layer-3 UE-to-UE relay UE MAC address

The UE shall include this IE to indicate the MAC address of the initiating 5G ProSe layer-3 UE-to-UE relay UE when the link between any two 5G ProSe multi-hop layer-3 UE-to-UE relay is being modified.

Next Change

#### 10.3.7.6	Target 5G ProSe layer-3 end UE MAC address

The UE shall include this IE when the MAC address of the target 5G ProSe layer-3 end UE, or target 5G ProSe multi-hop layer-3 end UE needs to be indicated.

NextChange

#### 10.3.14.7	Source 5G ProSe layer-3 end UE MAC address

The UE shall include this IE when the MAC address of the source 5G ProSe layer-3 end UE, or source 5G ProSe multi-hop layer-3 end UE needs to be indicated.