3GPP TSG-CT WG1 Meeting #157	C1-256813

Sophia Antipolis, France, 13-17 October 2025

Source:	Lenovo

Title:	Pseudo-CR on Aimlec_AIMLEClientParticipation API

Spec:	3GPP TS 24.560V1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Aimlec_AIMLEClientParticipation API is missing in TS 24.560.

There are some errors in clauses related for client participation service such as:

-	reordering the data types in clause 6.1.6.1

-	correction of description of aimlOperation  in clause 6.1.6.2.4

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24560V1.1.0.

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Aimlec_AIMLEClientParticipation API.

Table 6.1.6.1-1: Aimlec_AIMLEClientParticipation API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlecParticipationReq | 6.1.6.2.2 | Represents the participation request for the AIML operations. |  |
| AimlecParticipationResp | 6.1.6.2.3 | Represents the participation response for the AIML operations. |  |
| ClienSetPart | 6.1.6.3.3 | Represents the participation request for the AIMLE client set. |  |
| SchedAimlOperation | 6.1.6.2.4 | Represents the scheduled AIML participation type. |  |

Table 6.1.6.1-2 specifies data types re-used by the Aimlec_AIMLEClientParticipation API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_AIMLEClientParticipation API.

Table 6.1.6.1-2: Aimlec_AIMLEClientParticipation API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlOperation | 6.3.6.3.5 | Represents the type of the AIML operation. |  |
| DatasetRequirement | 3GPP TS 29.482 [7] | Represents the dataset requirements. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [5] | Represents an offered scheduled communication time. |  |
| ServiceRequirement | 3GPP TS 29.482 [7] | Represents the service requirements. |  |

* * * Next Change * * * *

##### 6.1.6.2.4	Type: SchedAimlOperation

Table 6.1.6.2.4-1: Definition of type SchedAimlOperation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlOperation | AimlOperation | M | 1 | Identifies the type of the AIML operation. |  |
| aimlOperSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for the AIML operation. |  |

* * * Next Change * * * *

## A.2	Aimlec_AIMLEClientParticipation API

openapi: 3.0.0

info:

title: Aimlec_AIMLEClientParticipation

version: 1.0.0-alpha.1

description: |

API for Client Participation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V1.2.0; Artificial Intelligence Machine Learning (AIML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-cp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/participation:

post:

summary: >

Used by AIMLE server to manage the participation of AIMLE clients in AIML operations.

operationId: ClientParticipation

tags:

- AIMLE client for participation of the AIML operations request

requestBody:

description: Contains information of the AIMLE client participates the AIML operations.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationReq'

responses:

'200':

description: Contains the outcome of the successful AIMLE client participation.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimlecParticipationReq:

description: Represents the participation request for the AIML operations.

type: object

required:

- requesterId

- clientSetId

- clientSetPart

- mlModelId

- schedAimlOperations

- dataSetReq

- serviceReq

properties:

requesterId:

description: Represents the requester identity.

type: string

clientSetId:

description: Represents the AIMLE client set identity.

type: string

clientSetPart:

description: Identifies the participation request for the AIMLE cient set.

$ref: '#/components/schemas/ClientSetPart'

mlModelId:

description: Identifies the identity of the ML model for AIML operation.

type: string

schedAimlOperations:

type: array

items:

$ref: '#/components/schemas/SchedAimlOperation'

minItems: 1

dataSetReq:

description: Identifies the dataset requirements.

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/DatasetRequirement'

serviceReq:

description: Identifies the service requirements.

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ServiceRequirement'

AimlecParticipationResp:

description: Represents the participation response for the AIML operations.

type: object

required:

- clientStatus

properties:

clientStatus:

description: true indicates the willingness and false indicates the denial.

type: boolean

SchedAimlOperation:

description: Represents the scheduled AIML participation type.

type: object

required:

- aimlOperation

properties:

aimlOperation:

description: Identifies the type of the AIML operation.

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlOperSched:

description: Identifies the schedule of the AIML operation.

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

# Enumerations

ClientSetPart:

anyOf:

- type: string

enum:

- JOIN

- DEPART

- type: string

description: >

This string provides the participation request for the AIMLE client set.

description: |

Identifies the type of request for  participating the AIMLE client set.

Possible values are:

- JOIN: Request to join the AIMLE client set.

- DEPART: Request to depart from the AIMLE client set.

* * * End of Changes * * * *