3GPP TSG-CT WG1 Meeting #157	C1-256822

Sophia Antipolis, France, 13-17 October 2025	(was C1-256389)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0113 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's note under clause A.4.4.3.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains the following editor’s note under clause A.4.4.3.2.6, quote: Editor's note [WID: XRM_Ph2_App, CR#: 0095]: The definition of multimodalSealddFlowId is FFS. Last meeting (CT1#156), the need of having a multimodalSealddFlowId attribute for the response message to an SEALDD multi-modal transmission connection establishment request was questioned. This resulted in the editor note above being added to the specification. Having a dedicated multi‑modal flow identifier is useful when messages have to reference, correlate, or manage the entire group of SEALDD flows as a single logical object. However, it is unnecessary if each response always contains exactly one grouping of sealdd flows and there is no reference to that grouping from elsewhere (any other message). Analysis of stage-2 requirements from TS 23.433, the a dedicated multi‑modal flow identifier is firstly optional. Furthermore, it is only indicated by the server to the client (direction server->client) and never from the client to the server. Also, for the the response message to an SEALDD multi-modal transmission connection establishment request only one grouping of sealdd flows is possible per message. However, the server can optionally reference a grouping of seal flows by using multi‑modal flow identifier in the SEALDD client policy configuration request (clause 9.10.3.7 of TS 23.433) and the Transmission quality measurement subscription request (clause 9.7.3.6 of TS 23.433). Hence, there is optinally reference to multi-modal grouping of sealdd flows from other messages, so if the multi‑modal flow identifier is provided in those messages, it could be beneficial to manage an entire group of sealdd flows as a single logical object using an identifier at the client side (UE). It is therefore proposed to support a dedicated multi‑modal flow identifier as per stage-2 description. Anyhow, note that an implementation can decide whether to use that provided multi‑modal flow identifier or use any other implementation method internally at the client side (UE) to reference a group of sealdd flows (e.g., memory pointers). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification is updated as per described in the reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification after freezing the release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.24.4, A.4.4.3.2.4, A.4.4.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.2.24.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.4.4.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req"; and

b)	a "XREstablishmentRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-res"; and

b)	shall attempt to create the SDD multi-modal transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include a "XREstablishmentResponse" object in the CoAP POST 2.01 (Created) response message:

i)	shall include a "result" attribute set to "success";

ii)	shall include a "sealddMultimodalFlow" attribute set to the information of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal service traffic;

A)	shall include a "sealddMultimodalFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal traffic; and

B)	 shall include a "sealddFlowsInfos" attribute set to the list of SDDM flows used by the SDDM-C and SDDM-S;

shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

may include a "portNumber" attribute specifying the identity of the port number of the traffic;

may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

iii)	may include a "headerExtType" attribute specifying the header extension information;

iv)	may include a "headerExtId" attribute specifying the header extension identity;

v)	may include a "packetizationIndication" attribute specifying the packet indication information;

vi)	may include a "payloadType" attribute specifying the payload type information; and

vii)	may include a "payloadFormat" attribute specifying the payload format information; and

2)	otherwise, shall include a "XR EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response as specified in clause A.4.4.2.2.3.1.

The SDDM-S shall send the CoAP POST response towards the SDDM-C.

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	To align with stage 2 on changing XR to multi-modal is FFS.

* * * Next Change * * * *

##### A.4.4.3.2.4	Type: XREstablishmentResponse

Table A.4.4.3.2.4.1: Definition of type XREstablishmentResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| sealddMultimodalFlow | SealddMultiModalFlowInfo | M | 1 | Information of the multi-modal SDDM flow used by the SDDM-C and SDDM-S (NOTE 2). |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE 1). |  |
| protocolDescriptorInfo | ProtocolDescriptorInfo | O | 0..1 | Specifying the information of the protocol of the VAL traffic (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "failure". NOTE 2: This attribute may be included if result is set to "success". |  |  |  |  |  |

* * * Next Change * * * *

##### A.4.4.3.2.6	Type: SealddMultiModalFlowInfo

Table A.4.4.3.2.6.1: Definition of type SealddMultiModalFlowInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddMultimodalFlowId | Uinteger | M | 1 | Identity of the multi-modal SDDM flow used by the SDDM-C and SDDM-S to identify application traffic. |  |
| sealddFlowsInfos | array(SealddFlowsInfo) | M | 1..N | Information of the SDDM flows used by the SDDM-C and the SDDM-S associated with the multi-modal flow. |  |

* * * End of Changes * * * *