3GPP TSG-CT WG1 Meeting #159	C1-260591

# Goa, India, 9th – 13th February 2026	was C1-260075

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6773 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retry after P-CSCF not reachable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The availability of emergency services is critical. Failure to reach emergency services can lead to life threatening situations and it has led to fatal situations when such failures happened in the field. Recently occurred incidents has shown that the UE's ability to retry emergency registration to a second (or third) P-CSCF is critical to ensure emergency service availability. This is even more important in cases when the emergency service in the CS domain is not possible e.g., CS domain is not available due to 2G/3G sunset. Current specs leave the actions up to UE implementation which may lead to unpredictable behavior unfortunately. Thus, there should clear guidelines on what the UE shall do to perform emergency services or at least for UE to inform the end user that there is a failure so that the end user could take other actions to obtain emergency services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change of a "may" to a "shall" in regards to the UE's action to retry a different P-CSCF at registration of emergency services. The use of the verb shall mandates the UE to retry the emergency registration. Adding a Note that clarifies the need for retries. Backward compatibility analysis: This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear guidelines on how to behave if the UE failes to reach a P-CSCF at emergency registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Change should to shall |  |  |  |  |  |  |  |  |

==============First change==============

#### 5.1.6.1	General

A CS and IM CN subsystem capable UE shall follow the conventions and rules specified in 3GPP TS 22.101 [1A] and 3GPP TS 23.167 [4B] to select the domain for the emergency call attempt. If the CS domain is selected, the UE shall attempt an emergency call setup using appropriate access technology specific procedures.

NOTE 1:	For CS systems based on 3GPP TS 24.008 [8], clause B.5 applies.

The UE shall determine, whether it is currently attached to its home operator's network (e.g. HPLMN or subscribed SNPN) or to a different network than its home operator's network (e.g. VPLMN or non-subscribed SNPN) by applying access technology specific procedures described in the access technology specific annexes.

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN or subscribed SNPN) and the UE is currently registered and the IP-CAN does not define emergency bearers, the UE shall attempt an emergency call as described in subclause 5.1.6.8.4.

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN or subscribed SNPN) and the UE is currently registered and the IP-CAN defines emergency bearers and the core network has indicated that it supports emergency bearers, the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN or subscribed SNPN) and the UE is not currently registered, the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

If the IM CN subsystem is selected and the UE is attached to a different network than its home operator's network (e.g. VPLMN or non-subscribed SNPN), the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

If the UE supports the emerg-reg timer defined in table 7.8.1, the UE shall start the emerg-reg timer when the UE decides that an emergency call is to be established via the IM CN subsystem. The UE shall stop the timer when the UE determines that an initial emergency registration, as described in subclause 5.1.6.2, is not required or upon receipt of any final SIP response during the initial emergency registration.

If the UE supports emerg-reg-retry timer defined in table 7.8.1, the UE shall start the timer using the final emerg-reg-retry timer value determined based on the configured value in table 7.8.1 when sending an emergency REGISTER. The UE shall stop the emerg-reg-retry timer when the UE receives any final response from this P-CSCF or when the emerg-reg timer has been stopped or expired.

On expiry of emerg-reg-retry timer or if UE encounters any fatal transport error as described in the RFC 3261 [26], the UE shall:

1)	if the emerg-reg timer has not expired and the initial REGISTER request for the initial emergency registration has not received any 2xx response , consider that the emergency registration attempt for this P-CSCF has failed. The UE shall retry registration on a different P-CSCF and restart the emerg-reg-retry timer. If the UE has no more available P-CSCFs, the UE shall stop the emerg-reg timer by considering the emergency registration has failed and follow the procedures related to emergency registration failure that are defined in 3GPP TS 23.167 [4B] subclause 6.1; and

2)	if the initial REGISTER request for the initial emergency registration has not been sent:

-	if the emerg-reg timer has not expired and the UE has successfully established an IP-CAN bearer for an emergency session, consider that the emergency registration attempt for this P-CSCF has failed. The UE shall retry registration on a different P-CSCF if available and restart the emerg-reg-retry timer. If the UE has no more available P-CSCFs the UE shall stop the emerg-reg timer by considering the emergency registration has failed and follow the procedures related to emergency registration failure that are defined in 3GPP TS 23.167 [4B] subclause 6.1; or

NOTE 2:	To retry registration and the availability of multiple P-CSCFs becomes more important in deployments where a CS domain is not available.

On expiry of emerg-reg timer, the UE shall:

-	if the UE has successfully established an IP-CAN bearer for an emergency session, consider that the attempt to set up the emergency call via the IM CN subsystem has failed and follow the procedures related to emergency registration failure that are defined in 3GPP TS 23.167 [4B] subclause 6.1; or

-	if the UE has not successfully established an IP-CAN bearer for an emergency session, consider that the attempt to set up the emergency call via the IM CN subsystem has failed, abort any ongoing IP-CAN procedures for the emergency registration, and follow the procedures for domain selection as defined in 3GPP TS 23.167 [4B] clause H.5.

stop the emerg-reg-retry timer, if running.

The UE may support being pre-configured for the Emerg-reg timer using one or more of the following methods:

a)	the Timer_Emerg-reg leaf of the EFIMSConfigData file described in 3GPP TS 31.102 [15C];

b)	the Timer_Emerg-reg leaf of the EFIMSConfigData file described in 3GPP TS 31.103 [15B]; and

c)	the Timer_Emerg-reg leaf of 3GPP TS 24.167 [8G].

If the UE is configured with both the Timer_Emerg-reg leaf of 3GPP TS 24.167 [8G] and the Timer_Emerg-reg leaf of the EFIMSConfigData file described in 3GPP TS 31.102 [15C] or 3GPP TS 31.103 [15B], then the Timer_Emerg-reg leaf of the EFIMSConfigData file shall take precedence.

NOTE 3:	Precedence for files configured on both the USIM and ISIM is defined in 3GPP TS 31.103 [15B].

If the IM CN subsystem is selected and the UE has no credentials the UE can make an emergency call without being registered. The UE shall attempt an emergency call as described in subclause 5.1.6.8.2.

An IP-CAN can, dependent on the IP-CAN capabilities, provide local emergency numbers (including information about emergency service categories or information about emergency service URNs) to the UE which has that capability, in order for the UE to recognize these numbers as emergency call.

==============End of change==============