3GPP TSG-CT WG1 Meeting #159	C1-260535

# Goa, India, 9th – 13th February 2026											Revision of C1-260099

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0019 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indicate the integrity failure to the lower layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, ZTE，LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | RAN2 LS on integrity failure (C1-257035) points out "integrity check failure" needs to be distinguished by the AS from other no NAS response cases for different AS handling: RAN2 discussed the handling of no NAS response case for an R2D A-IoT NAS message received by the A-IoT device. For some cases other than integrity check failure, RAN2 understands that there is no NAS response from the device, and for such cases, it was agreed that device’s AS will indicate no NAS response expected to the reader. The intention is to e.g. avoid the reader to schedule the retransmission of the problematic A-IoT NAS messages. For the integrity check failure of an R2D A-IoT NAS message received by a device, for now RAN2 assumes no NAS response or AS response from the device to the reader. However, RAN2 would like to ask SA3 and other WGs to take the above information into account and SA3 to check on the below questions: ... The p-CR C1-257198 proposing to indicate the integrity failure to the lower layer was discussed in CT1#158 meeting and postponed due to waiting for the reply from SA3. Considering SA3 Reply LS on integrity failure (C1-260025) confirms different AS handling for integrity check failure case as following, ... Question 1: whether, from the security perspective, it is allowed for the device to send an AS response to the reader for an A-IoT NAS message whose integrity protection check fails; Answer 1: From the security perspective, SA3 prefers not to send an AS response to the reader for an A-IoT NAS message whose integrity protection check fails. However, if an AS response is very critical for operation, then the AS response for integrity check failure must be indistinguishable from AS response for any other failure cases. ... it is suggested to specify indicating the integrity failure to the lower layer to help the AS to distinguish the integrity failure case from other no NAS response cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify indicating the integrity failure to the lower layer for AIoT device in the case an AIoT NAS message fails the integrity check. Backward compatibility analysis: This proposal is backwards compatible as it is a requirement of the AIoT device and has no impacts on the AIOTF and AIoT messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | "integrity check failure" cannot be distinguished by the AS from other no NAS response cases for different AS handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

4.2.3	Integrity protection of AIoT NAS signalling messages

For the AIoT device and AIOTF, integrity protected signalling is applied for AIoT NAS messages. Integrity protection of all AIoT NAS signalling messages, except the INVENTORY REPORT message, is mandatory and the responsibility of the AIoT NAS.

The algorithm to calculate the integrity protection information is described in annex D.3 in 3GPP TS 33.501 [9], and the integrity protection shall include octets 1 and 6 to n of the security protected AIoT NAS messages. The AIoT NAS signalling message is integrity protected by calculating the message authentication code (MAC) using KCommand_int derived from KAIOTF as specified in 3GPP TS 33.369 [6]. The calculated MAC shall be included in the MAC field (see clause 7.2.3).

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers that the AIoT NAS message was discarded due to integrity check failure.

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIOTF shall discard the AIoT NAS message.

==============End of change==============