3GPP TSG-CT WG1 Meeting #158	C1-257751

Dallas, US, 17-21 November 2025	was C1-xxxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0863 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the length of Warning message payload IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The length of Warning message payload IE is TBD. In clause 9.3.51 of TS 23.041: The Warning Message Content NG-RAN IE contains user information, e.g., the message with warning contents, and will be broadcast over the radio interface. This element is a string of maximum 9600 octets; see 3GPP TS 38.413 [40]. The contents would be (according to SIB6/7/8, as specified in 38.331 clause 6.3.1) Then the minimum length is 6 octets: SIB6, 2 octets (messageIdentifier) + 2 octets (serialNumber) + 2 octets (warningType). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the length of Warning message payload IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The length of Warning message payload IE is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

10.9.1	Message definition

This message is sent by the UE over the PC5 interface for broadcasting messages. See table 10.9.1.1.

Message type:	PROSE PC5 BROADCASTING

Significance:	dual

Direction:	UE to peer UE

Table 10.9.1.1: PROSE PC5 BROADCASTING message content for public warning notification

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe broadcasting message type (NOTE 1) | ProSe broadcasting message type 11.9.1 | M | V | 1 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
|  | Warning message payload | Warning message content NG-RAN (NOTE 2) | M | LV-E | 6-9600 |
| NOTE 1: The type is set to "Public warning notification". NOTE 2: The warning message content contains the contents of SIB6, SIB7, or SIB8 as specified in 3GPP TS 38.331 [13]. |  |  |  |  |  |

* * * End of Changes * * * *