3GPP TSG-CT WG1 Meeting #159	C1-260688

# Goa, India, 9th – 13th February 2026	(was C1-260669)

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0147 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's note under clause A.3.7.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2026-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | TS 24.543 contains an editor’s note under clause A.3.7.5.2, quote: Editor's note [WID: XRM_Ph2_App, CR#: 0121]: The CDDL document is FFS. There is need to define the above CDDL document to resolve the editor’s note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of editor’s note by defining the CDDL document. Backward compatibility analysis: The CR is backwards compatible as the CR adds new elements not existing before. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification when the release is frozen. Not possible to encode the Sdd_PolicyConfiguration API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: CDDL document updated. V2: Type "UInt32" is replaced by "Uinteger". |  |  |  |  |  |  |  |  |

==============First change==============

#### A.3.7.5.2	CDDL document

;;; PolicyConfigurationRequest

;;+ Represents a request for an SDDM policy configuration operation.

PolicyConfigurationRequest = {

requestorId: tstr  ; when the requestor is the SDDM-C, e.g. unique client identifier, when the requestor is the SDDM-S, e.g. FQDN or URI.

valServiceId: tstr

? sealddFlowId: Uinteger

? sealddmultimodalFlow: SealddMultiModalFlowInfo

? valTgtUe: ValTargetUe

.? configurationId: Uinteger

? multimodalFlowsAlignmentPolicy: MultimodalFlowsAlignmentPolicy

? sealddUeToUePolicy: SealddUeToUePolicy

* tstr => any

}

;;; PolicyConfigurationResponse

;;+ Represents a response of a SDDM policy configuration request.

PolicyConfigurationResponse = {

result: ResultOp

configurationId: Uinteger

* tstr => any

}

;;; SealddFlowsInfo

;;+ Represents the information of the SDDM flows.

SealddFlowsInfo = {

sealddFlowId: Uinteger

? userPlaneAddress: tstr

? portNumber: Uinteger

? url: tstr

? transportLayer: tstr

* tstr => any

}

;;; SealddMultiModalFlowInfo

;;+ Represents the information of the multi-modal SDDM flow.

SealddMultiModalFlowInfo = {

sealddMultimodalFlowId: Uinteger

sealddFlowsInfos: SealddFlowsInfo

* tstr => any

}

;;; MultimodalFlowsAlignmentPolicy

;;+ Represents the information of flows transmission requirement set to the identity of the multi-modal service.

MultimodalFlowsAlignmentPolicy = {

multimodalServiceId: Uinteger

flowsTransmissionReq: FlowsTransmissionReq

* tstr => any

}

;;; FlowsTransmissionReq

;;+ Represents the Information of flows transmission requirement including the delay requirement, maximum acceptable duration for traffic flow alignment.

FlowsTransmissionReq = {

delayReq: Uinteger

maxAligntime: Uinteger

* tstr => any

}

;;; SealddUeToUePolicy

;;+ Represents the Information of the proximity thresholds for entering/leaving the UE-to-UE direct communication mode.

SealddUeToUePolicy = {

proximityThresholds: ProxymityThresholds

qosThresholds: QoSThresholds

* tstr => any

}

;;; ProxymityThresholds

;;+ Represents the Information of proximity thresholds for entering/leaving the UE-to-UE direct communication mode.

ProxymityThresholds = {

? minUeToUedist: float

? avgUeToUedist: float

? maxUeToUedist: float

* tstr => any

}

;;; QoSThresholds

;;+ Represents the Information of the QoS thresholds for entering/leaving the UE-to-UE direct communication mode.

QoSThresholds = {

? minLatency: Uinteger    ; milliseconds (ms).

? avgLatency: Uinteger    ; milliseconds (ms).

? maxLatency: Uinteger    ; milliseconds (ms).

? minBitRate: Uinteger    ; bits per second (bps) (e.g. 64‑bit).

? avgBitRate: Uinteger    ; bits per second (bps) (e.g. 64‑bit).

? maxBitRate: Uinteger    ; bits per second (bps) (e.g. 64‑bit).

? minPackLossRate: float  ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? avgPackLossRate: float  ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? maxPackLossRate: float  ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? minPackErrRate: float   ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? avgPackErrRate: float   ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? maxPackErrRate: float   ; fractional rate between 0.0 and 1.0 (e.g. 0.01 = 1%).

? minJitter: Uinteger     ; nanoseconds (ns).

? avgJitter: Uinteger     ; nanoseconds (ns).

? maxJitter: Uinteger     ; nanoseconds (ns).

* tstr => any

}

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: tstr                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: tstr                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

ServerId = {

serverId = tstr

}

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

==============End of change==============