3GPP TSG-CT WG3 Meeting #143	C3-254115

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Nokia

Title:	Spatial Anchor Discovery Response OpenAPI update

Spec:	3GPP TS 29.437 and 1.1.0

Agenda item:	19.42

Document for:	Approval

1. Introduction

This pCR proposes layer updates to Spatial Anchor Discovery Response’s Open API handling.

2. Reason for Change

For SpatialAnchorsDiscResp datatype, the mintem is mentioned as 1 whereas it is possible to have zero spatial anchor discovery response.

3. Conclusions

The minitems = 1 in SpatialAnchorsDiscResp is removed.

4. Proposal

It is proposed to agree the following changes to 3GPP TS <TS number and version>.

* * * First Change * * * *

# A.3	SS_SAnDiscovery API

openapi: 3.0.0

info:

title: SAn Server Spatial Anchors Discovery Service

version: 1.0.0-alpha.3

description: |

API for Spatial Anchors Discovery Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V1.1.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssan_disc/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/spatial-anchors:

get:

summary: Discover Spatial Anchors profiles.

operationId: DiscSpatialAnchorsProfiles

tags:

- Spatial Anchors (Collection)

parameters:

- name: anchor-ids

in: query

description: Contains the identifier(s) of the targeted spatial anchor(s).

content:

application/json:

schema:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

- name: area-int

in: query

description: >

Contains the area of interest within which the targeted spatial anchors are located.

content:

application/json:

schema:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

- name: svc-id

in: query

description: Contains the identifier of the target VAL service.

schema:

type: string

- name: anch-inf

in: query

description: >

Contains the application specific information of the targeted spatial anchors.

content:

application/json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bytes'

- name: supp-feats

in: query

description: >

Contains the list of supported feature(s) among the ones defined in clause 6.1.2.8.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The Spatial Anchors profiles matching the filter criteria are returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialAnchorsDiscResp:

description: >

Represents the Spatial Anchors discovery response.

type: object

properties:

results:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorProfile'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- results

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

* * * End of Changes * * * *