3GPP TSG-CT WG3 Meeting #143	C3-254131

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Lenovo

Title:	Pseudo-CR on AIMLES_FLMemberGroupSupport API

Spec:	3GPP TS 29.482V1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Changes are:

-	YAML format of AIMLES_FLMemberGroupSupport API is added.

-	Table 6.1.3.6.1-1 is put together in alphabetic order.

-	MLModelProfile data type is used from clause 6.2.1.6.2.3, thus the editor’s notes in clauses 6.1.3.6.2.2 and 6.1.3.6.2.3 are deleted.

-	requester identity is being added to FlMbrSuppGrp data type.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482V1.1.0.

* * * First Change * * * *

##### 6.1.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.3.6.1-1 specifies the data types defined for the AIMLES_FLMemberGroupSupport API.

Table 6.1.3.6.1-1: AIMLES_FLMemberGroupSupport API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| FlCandidateType | 6.1.3.6.2.5 | Represents information on candidate FL member. |  |
| FlMbrStatus | 6.1.3.6.3.5 | Represents information on whether the FL member is a candidate or is selected. |  |
| FlMbrSuppGrp | 6.1.3.6.2.2 | Create a new Individual FL Member Support Group for an FL process. |  |
| FlMbrSuppGrpPatch | 6.1.3.6.2.3 | Individual FL Member Support Group to be partially modified. |  |
| FlMbrType | 6.1.3.6.3.4 | Represents information on the type of the FL member. |  |
| MlTaskInfo | 6.1.3.6.2.4 | Represents information on the identity and type of the ML task. |  |
| MlTaskType | 6.1.3.6.3.3 | Represents information on the identity of the ML model. |  |

Table 6.1.3.6.1-2 specifies data types re-used by the AIMLES_FLMemberGroupSupport API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_FLMemberGroupSupport API.

Table 6.1.3.6.1-2: Aimles_FLMemberGroupSupport API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| FlGroupInfo | 3GPP TS 24.560 [16] | Created Individual FL Member Support Group for the FL process. |  |
| FlMemberData | 3GPP TS 24.560 [16] | FL member data e.g. FL member identifier, address. |  |
| MLModelProfile | 6.2.1.6.2.3 | Represents the ML Model Profile. |  |
| SupportedFetaures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

* * * Next Change * * * *

6.1.3.6.2.2	Type: FlMbrSuppGrp

Table 6.1.3.6.2.2-1: Definition of type FlMbrSuppGrp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requesterId | string | M | 1 | Identifies the requester. |  |
| valServId | string | O | 0..1 | Identifies the VAL Service, for which the request applies. (NOTE) |  |
| mlModelId | string | O | 0..1 | Identifies the ML model, for which the request applies. (NOTE) |  |
| adaeAnalyticsId | string | O | 0..1 | Identifies the ADAE analytics, for which the request applies. (NOTE) |  |
| mlTask | MlTaskInfo | O | 0..1 | Identifies the ML task, for which the request applies. |  |
| mlModelProfile | MLModelProfile | O | 0..1 | Identifies the ML model profile, for which the Individual FL Member Support Group is requested. |  |
| flCandidates | FlCandidateType | O | 0..N | Identifies one or more selected candidates for the Individual FL Member Support Group. |  |
| flMbrSuppGrp | FlGroupInfo | M | 1..N | Identifies the created Individual FL Member Support Group for the FL process. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.3.8. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

* * * Next Change * * * *

6.1.3.6.2.3	Type: FlMbrSuppGrpPatch

Table 6.1.3.6.2.3-1: Definition of type FlMbrSuppGrpPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | O | 0..1 | Identifies the VAL Service, for which the request applies. |  |
| mlModelId | string | O | 0..1 | Identifies the ML model, for which the request applies. |  |
| adaeAnalyticsId | string | O | 0..1 | Identifies the ADAE analytics, for which the request applies. |  |
| mlTask | MlTaskInfo | O | 0..1 | Identifies the ML task, for which the request applies. |  |
| mlModelProfile | MlModelProfile | O | 0..1 | Identifies the ML model profile, for which the Individual FL Member Support Group is requested. |  |
| flCandidates | FlCandidateType | O | 0..N | Identifies one or more selected candidates for the Individual FL Member Support Group. |  |

* * * Next Change * * * *

# A.X	AIMLES_FLMemberGroupSupport API

openapi: 3.0.0

info:

title: AIMLE Federated Learning Member Group Support Service

version: 1.0.0-alpha.1

description: |

AIMLE Server provides a service consumer to create, update, and delete an FL Member Support

Group for an FL process.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-fl/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Create a new Individual FL Member Support Group resource.

operationId: CreateFLMemberGroupSupport

tags:

- FL Member Group Support

parameters:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

responses:

'201':

description: >

New Individual FL Member Support Group for the FL process is Created.

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configurationId}:

get:

description: Retrieve an existing Individual FL Member Support Group resource.

operationId: RetrieveFLMemberSupportGroup

tags:

- Individual FL Member Support Group

parameters:

- name: configurationId

in: path

description: String identifying an individual FL member support group resource.

required: true

schema:

type: string

responses:

'200':

description: The requested "Individual FL member support group” is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update completely an existing Individual FL Member Support Group resource.

operationId: UpdateFLMemberSupportGroup

tags:

- Individual FL Member Support Group

parameters:

- name: configurationId

in: path

description: String identifying an individual configuration resource.

required: true

schema:

type: string

requestBody:

description: Configuration information to be completely updated.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

responses:

'200':

description: The configuration is updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

'204':

description: successful case but without any Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify partially an existing Individual FL Member Support Group resource.

operationId: ModifyPartiallyFLMemberSupportGroup

tags:

- Individual FL Member Support Group

parameters:

- name: configurationId

in: path

description: Identifier of an individual configuration resource.

required: true

schema:

type: string

requestBody:

description: Configuration information to be partially modified.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrpPatch'

responses:

'200':

description: The individual FL member support group is partially modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/FlMbrSuppGrp'

'204':

description: successful case but without any Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual FL member support group.

operationId: DeleteFLMemberSupportGroup

tags:

- Individual FL Member Support Group

parameters:

- name: configurationId

in: path

description: String identifying an individual FL member support group resource.

required: true

schema:

type: string

responses:

'204':

description: The individual configuration matching configuration Id is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

FlMbrSuppGrp:

description: Represents a new Individual FL Member Support Group for an FL process.

type: object

properties:

requesterId

description: Identity of the requester

type: string

valServId

description: Identity the VAL Service

type: string

mlModelId

description: Identity the ML Model

type: string

adaeAnalyticsId

description: Identity the ADAE analytics

type: string

mlTask

description: Identity the ML task

$ref: '#/components/schemas/MlTaskInfo'

mlModelProfile

description: Identity the ML Modle Profile

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelProfile'

flCandidates:

type: array

items:

$ref: '#/components/schemas/FlCandidateType'

minItems: 0

flMbrSuppGrp:

type: array

items:

$ref: 'TS24560_Aimlec_FLGroupIndication.yaml#/components/schemas/FlGroupInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [valServId]

- required: [mlModelId]

- required: [adaeAnalyticsId]

required:

- requesterId

FlMbrSuppGrpPatch:

description: Represents individual FL Member Support Group to be  partially modified.

type: object

properties:

valServId

description: Identity of the VAL Service

type: string

mlModelId

description: Identity the ML Model

type: string

adaeAnalyticsId

description: Identity the ADAE analytics

type: string

mlTask

description: Identity the ML task

$ref: '#/components/schemas/MlTaskInfo'

mlModelProfile

description: Identity the ML Modle Profile

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelProfile'

flCandidates:

type: array

items:

$ref: '#/components/schemas/FlCandidateType'

minItems: 0

flMbrSuppGrp:

type: array

items:

$ref: 'TS24560_Aimlec_FLGroupIndication.yaml#/components/schemas/FlGroupInfo'

minItems: 0

MlTaskInfo:

description: Represents information on the identity and type of the ML task.

type: object

properties:

mlTaskId

description: Identity of the ML task

type: string

mlTaskType

description: Identity of the type of the ML task

$ref: '#/components/schemas/MlTaskType'

required:

- mlTaskId

- mlTaskType

FlCandidateType:

description: Represents information on candidate FL member.

type: object

properties:

flMbrId

description: Identity of the FL member

type: string

flMbrType

description: Identity of the type of the ML task

$ref: '#/components/schemas/FlMbrType'

flMbrStatus

description: Identity of the status of the ML task

$ref: '#/components/schemas/FlMbrStatus'

required:

- flMbrId

- flMbrType

- flMbrStatus

# Simple data types

# Enumerations

MlTaskType:

anyOf:

- type: string

enum:

- FL_TRAINING

- FL_INFERENCE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the different type of ML task

Possible values are:

- FL_TRAINING: Identifies an ML model training.

- FL_INFERENCE: Identifies an ML model inference.

FlMbrType:

anyOf:

- type: string

enum:

- FL_SERVER

- FL_CLIENT

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the different type of ML member

Possible values are:

- FL_SERVER: Identifies an FL server.

- FL_CLIENT: Identifies an FL client.

FlMbrStatus:

anyOf:

- type: string

enum:

- FL_CANDIDATE

- FL_SELECTED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the information on the status of ML member

Possible values are:

- FL_CANDIDATE: Identifies an FL member being a candidate.

- FL_SELECTED: Identifies an FL member being selected.

* * * End of Changes * * * *